3GPP TSG-RAN3 Meeting #110-e	R3-206553

Electronic meeting, 2nd - 12th November 2020

# Title:	[Reply] LS on System support for Multi-USIM devices

# Response to:	LS (R3-205924/S2-2006037) on “System support for Multi-USIM devices” from SA2

# Release:	Release 17

# Work Item:	LTE_NR_MUSIM-Core

Source:	Qualcomm (RAN3)

To:	SA2, RAN2

Cc:	SA3

Contact Person:

Name:	Xipeng Zhu

E-mail Address:	xipengz@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 thanks SA2 for the questions. After discussion, RAN3 would like to provide following answers.

Q1: Please confirm the feasibility and overhead of sending a Paging Cause in [Uu] Paging message for EPS and for 5GS. [RAN2, RAN3]

Answer: From RAN3 point of view, support of Paging Cause is feasible and the incurred overhead on the backhaul signaling is negligible.

Q3: Please indicate how the paging cause is expected to be supported in RAN nodes (e.g. per PLMN, per TA, per RAN node, per cell) (For NR and E-UTRA) [RAN2, RAN3]

Answer: For uniform behavior in the paging area, per TA or per PLMN is preferred.

Q5: Please provide feedback if it is feasible (and secure) that the Busy Indication is sent as RRC message instead (no NAS message to the CN) i.e. as a RRC response to paging without requiring an RRC connection [RAN2, RAN3, SA3]

Answer: Busy indication as an RRC message is feasible but will require substantial work by RAN3 (e.g. send busy indication to AMF for CN paging case and request 5GC to suspend U-plane for RAN paging case) and thus is not recommended.

Q6: Please indicate whether it is feasible to define an RRC-based leaving and returning procedure in 5GS/NR. [RAN2, RAN3]

Answer: From RAN3 point of view, it is feasible to define a RRC-based leaving and returning procedure in 5GS/NR, based on existing RAN3 procedures, e.g. RRC INACTIVE TRANSITION REPORT for RRC_INACTIVE and UE CONTEXT RELEASE if RAN wants to release UE to RRC_IDLE.

Q7: Please let us know whether changes to 5GS/E-UTRA (Option 5) to support RRC-based leaving is part of RAN Work Item. [RAN2, RAN3].

Answer: Related RAN3 interfaces and procedures are common to NR and 5GS/E-UTRA. The response to this question should be deferred to RAN2.

2. Actions:

To SA2

ACTION:

RAN3 would like to ask SA2 to take the above answer into account.

To RAN2

ACTION:

RAN3 would like to ask RAN2 to take the above answer into account and decide Q6/Q7 from RAN2 perspective.

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3#111e		January 25 – February 5, 2021		Electronic Meeting

TSG-RAN3#112e		May 17 – 28, 2021				Electronic Meeting