3GPP TSG-RAN WG3 Meeting #110-e	R3-206566

Online, 2nd – 13th November, 2020

Agenda item:		8.1

Source:		Vodafone

Title:		Response to SA2 LS S2-2006037: Paging Repetition in         RAN aspects

Document for:		Discussion and Decision

Work Item Code: 		LTE_NR_MUSIM

# Introduction

SA2 working group in their recent Liaison to RAN3, S2-2006037 on the Multi-SIM topic, have asked several questions to be resolved.

In this contribution we attempt to address Question 8 relating to the following proposals raised in the LS:

Repeating paging in the RAN on consecutive POs. for MUSIM devices.

# Discussion

From TS 36.413 we observe that paging is sent over S1 interface from the MME to the eNodeB and the paging message is broadcasted to the UE, as shown in the following Figure 1

Figure 1 Paging Message sent over S1 Interface to the eNodeB

For a mobile terminating Packet Switched (PS) or Circuit Switched (CS) event, the MME sends a paging message to one or multiple eNodeBs over the S1 interface.

In order to minimise the MME/AMF processing load caused by immediately paging all of the RAN nodes in the TAI list, the Paging Policy Differentiation feature (section 4.9 of TS 23.401) can be used so that the MME/AMF can determine which PS domain MT events are for voice calls and which are not.

This enables the MME/AMF to minimise voice call setup latency by immediately paging for these important events in all eNBs/gNBs in the TAI list, while minimising processor load by implementing an escalating paging area approach for less important MT events.

Observation 1: In order to minimise voice call set up delay, the MME is likely to send S1-AP paging messages to all eNBs/gNBs in the TA LIST for the “first page”. This can be many eNBs/gNBs.

However, in order to conserve UE battery power, the HPLMN normally uses firewalls (or NATs) to ensure that ‘random’ IP packets do not cause the mobile to transition from idle to active state. As a result, paging for services other than IMS (voice/messages) or CS-domain (voice/SMS) is relatively rare except for the time period following a radio link failure.

Observation 2: A significant proportion of all S1/N2 paging messages will relate to voice calls and they will be sent in parallel to all the eNBs/gNBs in the TAI list.

Because the core network is not aware of the paging channel configuration in the RAN nodes (and which may be different in different RAN nodes), and, because the core network and RAN nodes need not be time synchronised, the core network does not buffer the MT event. Instead, the core network node (MME or AMF) promptly sends the paging message to the RAN nodes.

Observation 3: The MME and AMF are not aware of when the UE’s Paging Occasion will occur. Even if the CN were to be synchronised with the RAN’s timing, there are other RAN parameters that influence when the Paging Occasion will occur.

The S1 paging message from the MME, contains a UE ID value which is obtained from a function of the IMSI. Based on this number and the value of N (N=min(NB,T)), the eNB calculates the PO of the UE and it buffers the paging message until the PO.

Since the UE already knows it’s IMSI and the eNB has already derived it from the UE ID in the S1 Paging message, both the UE and eNB are synchronized and they use the same PO subframe. Once, the UE receives the paging message addressed to itself, it initiates RACH process by sending a RACH preamble to the eNB.

Taking an example of a voice call, the MME will page many eNodeBs in the tracking area list in order to locate the UE, as shown in the following Figure 2

Figure 2 Multiple Paging Messages sent to many eNodeBs in a Tracking Area List

As illustrated in the Figure 2, eNodeBs 2, 3,…X do not see that UE1 has responded to the paging message on eNodeB1.

Therefore if eNodeBs 2, 3,….X locally repeat the paging message, significant paging resources are wasted.

e.g. if eNode B2 has 3 sectors and supports 4 frequency bands in each sector, one local repetition cause 12 wasteful paging messages to be sent!

Observation 4: The UE is only camped on one cell, so eNBs where the UE is not camped will not see any response from the UE. Hence any RAN-internal paging repetition leads to significant wastage of paging resources (even on one eNB this can involve many paging messages – e.g. paging occurs in 12 cells for a 3 sector, 4 frequency eNB.)

Because of this, section 8.5 of TS 36.413 (S1-AP) states that:

“For each cell that belongs to any of the TAs indicated in the List of TAIs IE, the eNB shall generate one page on the radio interface.”

Observation 5: The paging procedure in the EPS (and in RRC-Idle in the 5GS) is designed based on paging repetition only by the core network.

Proposal 1: the EPS and 5GS design of the RAN NOT locally repeating paging for UEs in RRC Idle should be kept.

# Conclusions

From our technical analysis we have made the following observations:

Observation 1: In order to minimise voice call set up delay, the MME is likely to send S1-AP paging messages to all eNBs/gNBs in the TA LIST for the “first page”. This can be many eNBs/gNBs.

Observation 2: A significant proportion of all S1/N2 paging messages will relate to voice calls and they will be sent in parallel to all the eNBs/gNBs in the TAI list.

Observation 3: The MME and AMF are not aware of when the UE’s Paging Occasion will occur. Even if the CN were to be synchronised with the RAN’s timing, there are other RAN parameters that influence when the Paging Occasion will occur.

Observation 4: The UE is only camped on one cell, so eNBs where the UE is not camped will not see any response from the UE. Hence any RAN-internal paging repetition leads to significant wastage of paging resources (even on one eNB this can involve many paging messages – e.g. paging occurs in 12 cells for a 3 sector, 4 frequency eNB.)

Observation 5: The paging procedure in the EPS (and in RRC-Idle in the 5GS) is designed based on paging repetition only by the core network.

And based on our observations we propose the following:

Proposal 1: the EPS and 5GS design of the RAN NOT locally repeating paging for UEs in RRC Idle should be kept.

In conclusion, in response to question raised by SA2 in their LS:

- Local Paging Repetition in the RAN for UEs in RRC Idle should not be implemented.