3GPP TSG-RAN3 Meeting # 110-e	   R3-206579

# 2 - 12 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.455 | CR | 0116 | rev |  | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting of SFTD measurements in LPPa |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DoCoMo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  | Date: |  |  | 2020-23-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the SFN Time Difference (SFTD) measurement results to be added in OTDOA Information Exchange procedure for LTE-NR cells in LPPa. The SFTD results are reported to the E-SMLC when a UE is in LTE PCell and measures multiple NR cells. For each NR cell detected, a SFTD result is available. The SFTD information is reported without triggering any L3 functions, i.e., the E-SMLC receives what the eNB already has available from the UE’s configuration. The E-SMLC may signal the SFTD result information to the LMF via proprietary signalling (in case the two location servers are not the same). See TS 38.305 section 5.1. The LMF will use the SFTD information for the OTDOA assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding SFTD measurements result with SFN offset and frame boundary for LTE-NR cells values in the OTDOA Information Exchange procedure. Impact Analysis: This CR has isolated impact with the previous version of the specification because this is additional information that may be used by receiving node This CR has no impact from functional point of view. It is signalling of existing information in the network. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insufficient information at location server for the OTDOA assistance data when the UE reports SFTD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.7, 9.2.7, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.1.1.7	OTDOA INFORMATION REQUEST

This message is sent by E-SMLC to request OTDOA information.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| OTDOA Information Type |  | 1 .. <maxnoOTDOAtypes> |  |  | EACH | reject |
| >OTDOA Information Item | M |  | ENUMERATED (pci, cellid, tac, earfcn, prsBandwidth, prsConfigIndex, cpLength, noDlFrames, noAntennaPorts, sFNInitTime, …, e-UTRANAccessPointPosition, prsmutingconfiguration, prsid, tpid, tpType, crsCPlength, MBSFNsubframeConfiguration, nPRSConfiguration, offsetNBChanneltoEARFCN, operationModeInfo, NPRS-ID, dlBandwidth, multipleprsConfigurationsperCell, prsOccasionGroup, prsFrequencyHoppingConfiguration, repetitionNumberofSIB1-NB, nPRSsequenceInfo, NPRS Type 2, tddConfig, sFTD) |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

NEXT CHANGE

### 9.2.7	OTDOA Cell Information

This IE contains OTDOA information of a cell/TP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| OTDOA Cell Information |  | 1 .. <maxnoOTDOAtypes> |  |  |
| >CHOICE OTDOA Cell Information Item | M |  |  |  |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell ID |
| >>Cell ID | M |  | ECGI 9.2.6 |  |
| >>TAC | M |  | OCTET STRING(2) | Tracking Area Code |
| >>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5]. For an inband mode NB-IoT Cell, this IE indicates the E-UTRAN EARFCN. |
| >>PRS Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | Transmission bandwidth of PRS |
| >>PRS Configuration Index | M |  | INTEGER (0..4095, ...) | PRS Configuration Index, ref TS 36.211 [6] |
| >>CP Length | M |  | ENUMERATED (Normal, Extended,...) | Cyclic prefix length of the PRS |
| >>Number of DL Frames | M |  | ENUMERATED (sf1, sf2, sf4, sf6,…) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [6] |
| >>Number of Antenna Ports | M |  | ENUMERATED(n1-or-n2, n4,…) | Number of used antenna ports, where n1-or-n2 corresponds to 1 or 2 ports, n4 corresponds to 4 ports |
| >>SFN Initialisation Time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |
| >>E-UTRAN Access Point Position | M |  | 9.2.8 | The configured estimated geographical position of the antenna of the cell/TP. |
| >>PRS Muting Configuration | M |  | 9.2.9 | The configuration of positioning reference signals muting pattern. |
| >>PRS-ID | M |  | INTEGER (0..4095,…) | PRS ID, ref TS 36.211 [6]. |
| >>TP-ID | M |  | INTEGER (0..4095,…) | Identity of the transmission point. This IE together with the PCI and/or PRS-ID may be used to identify the transmission point in case the same physical cell ID is shared by multiple transmission points. |
| >>TP Type | M |  | ENUMERATED (prs-only-tp, …) | A TP which transmits PRS only. |
| >>Number of DL Frames-Extended | M |  | INTEGER (1..160,…) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [6]. |
| >>CRS CP Length | M |  | ENUMERATED (Normal, Extended,...) | Cyclic prefix length of the CRS. |
| >>MBSFN subframe Configuration | M |  | 9.2.14 | The MBSFN subframe configuration. |
| >>NPRS configuration | M |  | 9.2.16 | The NPRS configuration with the mapping to resource elements as specified for the Type 1 NPRS in TS 36.211 sub-clause 10.2.6A.2 [6]. Only applicable for inband mode NB-IoT operation. |
| >>Offset of NB-IoT Channel Number to DL EARFCN | M |  | Offset of NB-IoT Channel Number to EARFCN 9.2.18 | Corresponds to MDL in TS 36.104 [5] |
| >>operationModeInfo | M |  | ENUMERATED (inband, guardband, standalone,...) |  |
| >>NPRS-ID | M |  | INTEGER (0..4095,…) | NPRS ID, ref TS 36.211 [6]. |
| >>DL Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | DL transmission bandwidth expressed in units of resource blocks NRB, ref TS 36.104 [5]. |
| >>PRS Occasion Group | M |  | ENUMERATED (og2, og4, og8, og16, og32, og64, og128, ...) | PRS occasion group in a PRS period, ref TS 36.211 [6]. |
| >>PRS Frequency Hopping Configuration | M |  | 9.2.19 | PRS frequency hopping configuration. |
| >>Repetition Number of SIB1-NB | M |  | ENUMERATED (r4, r8, r16, ...) | Repetition Number of SIB1-NB, refer to TS36.213 [12]. Value r4 corresponds to 4 repetitions, r8 to 8 repetitions, and r16 to 16 repetitions. |
| >>NPRSsequenceInfo | M |  | INTEGER (0..174,…) | The index of the PRB containing the NPRS as defined in the table nprsSequenceInfo to E-UTRA PRB index relation, refer to TS 36.355 [13]. Only included in case of inband mode NB-IoT operation. |
| >>NPRS Type 2 | M |  | 9.2.16 | The NPRS configuration with the mapping to resource elements as specified for the Type 2 NPRS in TS 36.211 sub-clause 10.2.6A.2 [6]. |
| >>TDD Configuration | M |  | 9.2.25 | TDD specific physical channel configuration. |
| >>Result SFTD |  | 1..<maxNRMeas> |  |  |
| >>>NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID of the measured NR cell. |
| >>>SFN-Offset | M |  | INTEGER (0..1023) | The observed SFN and frame timing difference (SFTD) between an E-UTRA PCell and an NR PSCell |
| >>>MeanFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Average value of the frame boundary offset observed between PCell and the measured cell. |
| >>>MinFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Minimum value of the frame boundary offset observed between PCell and the measured cell. |
| >>>MaxFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Maximum value of the frame boundary offset observed between PCell and the measured cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 32. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-ResultsPerSSB-Index-List,

id-ResultsPerSSB-Index-Item,

id-NR-CGI,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxResultsPerSSBIndex,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxNRMeas

UNCHANGED TEXT OMITTED

NEXT CHANGE

#### -- O

OffsetNBChanneltoEARFCN ::= ENUMERATED {

minusTen,

minusNine,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFour,

minusThree,

minusTwo,

minusOne,

minusZeroDotFive,

zero,

one,

two,

three,

four,

five,

six,

seven,

eight,

nine,

...

}

OperationModeInfo ::= ENUMERATED {

inband,

guardband,

standalone,

...

}

OTDOACells ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

oTDOACellInfo					OTDOACell-Information,

iE-Extensions					ProtocolExtensionContainer { {OTDOACells-ExtIEs} } OPTIONAL,

...

}

OTDOACells-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

OTDOACell-Information ::= SEQUENCE (SIZE (1..maxnoOTDOAtypes)) OF OTDOACell-Information-Item

OTDOACell-Information-Item ::= CHOICE {

pCI							PCI,

cellId						ECGI,

tAC							TAC,

eARFCN						EARFCN,

pRS-Bandwidth				PRS-Bandwidth,

pRS-ConfigurationIndex		PRS-Configuration-Index,

cPLength					CPLength,

numberOfDlFrames			NumberOfDlFrames,

numberOfAntennaPorts		NumberOfAntennaPorts,

sFNInitialisationTime		SFNInitialisationTime,

e-UTRANAccessPointPosition	E-UTRANAccessPointPosition,

...,

pRSMutingConfiguration		PRSMutingConfiguration,

prsid						PRS-ID,

tpid						TP-ID,

tpType						TP-Type,

numberOfDlFrames-Extended	NumberOfDlFrames-Extended,

crsCPlength					CPLength,

mBSFNsubframeConfiguration	MBSFNsubframeConfiguration,

nPRSConfiguration			NPRSConfiguration,

offsetNBChanneltoEARFCN		OffsetNBChanneltoEARFCN,

operationModeInfo			OperationModeInfo,

nPRS-ID						INTEGER (0..4095, ...),

dL-Bandwidth				DL-Bandwidth,

pRSOccasionGroup			PRSOccasionGroup,

pRSFreqHoppingConfig		PRSFrequencyHoppingConfiguration,

repetitionNumberofSIB1-NB	RepetitionNumberofSIB1-NB,

nPRSSequenceInfo			NPRSSequenceInfo,

nPRSType2					NPRSConfiguration,

tddConfiguration			TDDConfiguration,

sFTD						ResultSFTD

}

OTDOA-Information-Item ::= ENUMERATED {

pci,

cellid,

tac,

earfcn,

prsBandwidth,

prsConfigIndex,

cpLength,

noDlFrames,

noAntennaPorts,

sFNInitTime,

...,

e-UTRANAccessPointPosition,

prsmutingconfiguration,

prsid,

tpid,

tpType,

crsCPlength,

mBSFNsubframeConfiguration,

nPRSConfiguration,

offsetNBChannelNumbertoEARFCN,

operationModeInfo,

nPRS-ID,

dlBandwidth,

multipleprsConfigurationsperCell,

prsOccasionGroup,

prsFrequencyHoppingConfiguration,

repetitionNumberofSIB1-NB,

nPRSSequenceInfo,

nPRSType2,

tddConfig,

sFTD

}

Outcome ::= ENUMERATED {

failed,

...

}

NEXT CHANGE

#### -- R

RepetitionNumberofSIB1-NB ::= ENUMERATED {

r4,

r8,

r16,

...

}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...),

bandwidth				INTEGER (1..100, ...),

...

}

ResultRSRP ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-Item

ResultRSRP-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRP		ValueRSRP,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRP-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-Item

ResultRSRQ-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRQ		ValueRSRQ,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRQ-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSFTD ::= SEQUENCE (SIZE (1.. maxNRmeas)) OF ResultSFTD-Item

ResultSFTD-Item ::= SEQUENCE {

nRPCI						INTEGER (0..1007),

sFN-Offset					INTEGER (0..1023),

meanFrameBoundary-Offset	INTEGER (-30720..30719),

minFrameBoundary-Offset		INTEGER (-30720..30719),

maxFrameBoundary-Offset		INTEGER (-30720..30719),

iE-Extensions		ProtocolExtensionContainer { { ResultSFTD-Item-ExtIEs} } OPTIONAL,

...

}

ResultSFTD-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

NEXT CHANGE

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

LPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM LPPA-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-uTDOAInformationExchange										ProcedureCode ::= 7

id-uTDOAInformationUpdate										ProcedureCode ::= 8

id-assistanceInformationControl									ProcedureCode ::= 9

id-assistanceInformationFeedback								ProcedureCode ::= 10

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellineNB								INTEGER ::= 256

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxGERANMeas								INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxCellineNB-ext							INTEGER ::= 3840

maxMBSFN-Allocations						INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNRmeas									INTEGER ::= 32

END OF CHANGES