3GPP TSG-RAN WG3 Meeting #110-e	R3-206637

E-meeting, 2 – 12 Nov 2020

Title: 	Discussion on the size limitation of DTLS over SCTP

Source: 	Huawei, China Telecom

Agenda item:	9.3.7.1

Document Type:	discussion

- Introduction

The DTLS (Datagram Transport Layer Security) is defined by Internet Engineering Task Force (IETF) to provide communications privacy over Stream Control Transmission Protocol (SCTP) in RFC6083 [1]. As required in 3GPP TS 33.501 [2], the DTLS over SCTP shall be supported for N2, Xn, F1 and E1interfaces. For example, the following is depicted for Xn interface in TS 33.501 [2].

In addition to IPsec, for the Xn-C interface, DTLS shall be supported as specified in RFC 6083 [58] to provide integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the provisions given in clause 6.2 of TS 33.210 [3].

In this paper, we will discuss the impacts of the size DTLS limitation on network interfaces and the potential solutions.

- Discussion

  - Problem Description

The DTLS is a necessary security feature required in 3GPP TS 33.501. The following message size limitation is described in RFC6083 [1] as follows.

‘DTLS limits the DTLS user message size to the current Path MTU minus the header sizes. For the purposes of running over SCTP, the DTLS path MTU MUST be considered to be 2^14.’

- Observation 1: DTLS over SCTP limits the maximum size of the messages to be 2^14 bit (about 16K).

However, in Release 16, the following inter-node RRC messages e.g. HandoverPreparationInformation, CG-ConfigInfo, UERadioAccessCapabilityInformation etc signaled over the network interfaces may exceed the 16K size. Typically,

- UECapabilityInformation can be up to 9k * 16 Bytes;

- RRCReconfiguration can be up to 9k * 5 bytes.

In RAN2, the RRC segmentation mechanism is introduced over Uu interface considering the maximum PDCP SDU size in NR is 9k. The UL segmentation is applicable to UECapabilityInformation, and the DL segmentation is only applicable to RRCReconfiguration and RRCResume messages. In RAN3, this means that the full size of inter-node RRC cannot be signalled over the network NG, Xn, F1 and E1 interfaces.

For example, during the handover procedure,

- If the full size of UE radio capability information is not signalled, the target node has to fetch the UE radio capability from the UE, which results in the handover latency and bad UE experience.

- If the HandoverPreparationInformation containing the RRCReconfiguration is not fully signalled,

  - the target node has to perform full configuration, which results in large signalling overhead;

  - the lossless handover during handover can not be performed.

- Observation 2. In case of DTLS over SCTP, the full size of inter-node RRC messages may not be signalled over network interfaces.

  - Potential Solutions

The following potential options are provided as follows.

    - Option 1: Indication of incomplete inter-node transfer

This option is to deliver a reduced-size inter-node RRC message from the source node to the target node but with an incomplete indication. Typically, when the full set of inter-node message cannot be delivered, the source node can indicate to the target node that the inter-node message delivered is not incomplete. The reason is that, the behaviour is not clear for the target node in case of incomplete inter-node RRC transfer since the target can not identify whether this is due to the size limitation or the source node real configuration. For example,

- If partial optional information is signalled, the target node may wrongly think this is current real UE configuration.

- If mandatory information is not signalled, the target node has to reject the procedure.

The incomplete indication can be included in the inter-node message, or the network interfaces. However the issue listed above still remains for this option.

Consider whether to introduce an incomplete indication in case incomplete inter-node message is transmitted.

    - Option 2: introduction of segmentation procedure

In this option, a message segmentation procedure is introduced to transmit the full set of inter-node RRC messages over the network interfaces, similar to the segment transfer over the Uu interface.

If the inter-node RRC message size exceeds the limits, the source node segments the network message and sends each segment to the target node in a numbered sequence. The target node assembles the segments into a whole message. The Figure 1 gives an example for XnAP Handover request message.

Figure 1. Message segmentation procedure

Introduce the message segmentation procedure over NG, Xn, F1 and E1 interfaces.

    - Option 3: LS to IETF

In this option, a LS can be sent to IETF to inform this issue, and request to e.g. increase the maximum size. But this solution is time-consuming and may not solve the issue timely.

Consider whether to send LS to IETF on the size restriction for DTLS over SCTP.

- Based the above analysis, option 1 can not fully solve this problem, while option 3 is time-consuming but with unclear outcome. Hence option 2 is suggested. The Xn CR for option 2 is provided in the annex. The rest CRs can be produced online if there is consensus.

- Conclusion

Based on the above discussion, we make the following observations and recommend RAN3 to discuss and adopt the following proposals:

- Consider whether to introduce an incomplete indication in case incomplete inter-node message is transmitted.

Introduce the message segmentation procedure over NG, Xn, F1 and E1 interfaces.

Consider whether to send LS to IETF on the size restriction for DTLS over SCTP.

- Reference

- IETF RFC 6083: "Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

- 3GPP TS 33.501, Security architecture and procedures for 5G system.

- Annex

3GPP TSG-RAN WG3 Meeting #111-e	<TDoc#>

# E-meeting, – Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev |  | Current version: | V 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discussion on the size limitation of DTLS over SCTP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | DTLS over SCTP limits the maximum size of the messages to be 2^14 bit (about 16 K). However, in Release 16, the following inter-node RRC messages e.g. HandoverPreparationInformation, CG-ConfigInfo UERadioAccessCapabilityInformation etc. which will be transferred over the network interface may exceed the 16 K size. In case of DTLS over SCTP, the full set of inter-node RRC messages cannot be signalled over network interfaces. Therefore, it is necessary to introduce the segmentation feature to the message that contains the inter-node RRC messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Dedicated Message Segment Transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interface messages that contains inter-node RRC messages could not be deliveried if the size of those excceds the limit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.4.a, 9.1.3.b, 9.2.3.c, 9.2.3.d, 9.2.3.e |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

<<<<<<<<<<<<<<<<<<<<<<<<<<< Unchanged part is omitted >>>>>>>>>>>>>>>>>>>>>>>>>>>

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Dedicated Message Segment Transfer | DEDICATED MESSAGE SEGMENT |

NEXT CHANGE

### 8.4.a	Dedicated Message Segment Transfer

#### 8.4.a.1	General

The purpose of the Dedicated Message Segment Transfer procedure is to transfer segments of a message between NG-RAN nodes. The procedure is initiated by an NG-RAN node whenever a message exceeds a limit.

#### 8.4.a.2	Successful Operation

The NG-RAN node1 initiates the procedure by sending the DEDICATED MESSAGE SEGMENT message to the candidate NG-RAN node2.

The NG-RAN node2 shall consider the Segment Number IE in the DEDICATED MESSAGE SEGMENT message to assemble the segments in the correct order.

If the Message Segment Type IE is set to “lastSegment”, the NG-RAN node2 shall consider to assemble the message from the received segments.

#### 8.4.a.3	Unsuccessful Operation

Not applicable.

NEXT CHANGE

#### 9.1.3.b	DEDICATED MESSAGE SEGMENT

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer segments of a message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Segment Number | M | 1..<maxnoofSegments> | 9.2.3c |  |  |  |
| Message Segment Container | M |  | 9.2.3.d |  | YES | reject |
| Message Segment Type | M |  | 9.2.3.e |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSegments | Maximum no. of segments. Value is 16. |

NEXT CHANGE

#### 9.2.3.c	Segment Number

This IE identifies the sequence number of a segment of a message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Segment Number | M |  | INTEGER (0..n, ...) |  |

#### 9.2.3.d	Message Segment Container

This IE includes a segment of the encoded message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Segment Container | M |  | OCTET STRING | Include a segment of a message. |

#### 9.2.3.e	Message Segment Type

This IE indicates whether the included message segment is the last segment of the message or not.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Segment Type | M |  | ENUMERATED (notLastSegment, lastSegment, …) |  |