3GPP TSG-RAN WG3 #110-e     					                                                    R3-206655

- 2 - 12 Nov 2020

Online

Agenda item:	10.2.1.1

Source: 	ZTE

Title: 	TP to 38.300 on PCI Selection

Document for:	Approval

Introduction

This contribution provides a TP for BLCR to 38.300 based on the PCI Selection.

Text Proposal for TR 38.300

15.5.y	Support for PCI Optimisation

15.5.y.1	Centralized PCI Assignment

For centralized PCI assignment, in non-split gNB architecture, the OAM assigns a single PCI for each NR cell in the gNB, and the gNB selects this value as the PCI of the NR cell.

It is FFS for centralized PCI assignment in split architecture.

For distributed PCI assignment, in split architecture case, gNB-CU detects and indicates PCI conflict to OAM via gNB-DU, the OAM assigns a new PCI for the cell having PCI conflict and configures it to the gNB-DU.

15.5.y.2	Distributed PCI Assignment

For distributed PCI assignment, in non-split gNB architecture, the OAM assigns a list of PCIs for each NR cell in the gNB, and the gNB selects a PCI value from the list of PCIs.

For distributed PCI assignment, in split architecture case, PCI conflict detection and reassignment are performed by the gNB-CU. Whether a list of available PCIs signaled from DU to CU for each NR cell is needed is FFS.A list of available PCIs needs to be transmitted from gNB-DU to gNB-CU via OAM.