3GPP TSG-RAN WG3 #110-e     					                                                    R3-206658

- 2 - 12 Nov 2020

Online

Agenda item:	10.2.1.5

Source: 	ZTE

Title: 	TP for BL CR 38.463 on Load Balancing Enhancements

Document for:	Approval

Introduction

This contribution provides a TP for BLCR to 38.463. i.e., add slice level DRB usage as load metric over E1.

Text Proposal for 38.463

-------------------------------------------------Start of the first change-------------------------------------------------------------------

9.2.1.19	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |  |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |  |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |  |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |  |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | ignore |  |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Capacity Ind Period, Third Bit = Slice DRB Periodic. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

-------------------------------------------------End of the first change-------------------------------------------------------------------

-----------------------------------------------Start of the second change-----------------------------------------------------------------

9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | Ignore |  |
| Transaction ID | M |  | 9.3.1.53 |  | YES | Reject |  |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | Reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | Reject |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  |  |  |
| HW Capacity Indicator | O |  | 9.3.1.73 |  |  |  |
| >>>>Slice DRB Usage | O |  | 9.3.1.yy |  |  |  |
| >> Shared S-NSSAI List |  | 0..1 |  |  |  |  |
| >>>Shared S-NSSAI Item |  | 1 .. < maxnoofSharedSliceItems> |  |  |  |  |
| >>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>>>TNL Available Capacity Indicator | O |  | 9.3.x.x |  |  |  |
| >>>>HW Capacity Indicator [FFS] | O |  | 9.3.x.x |  |  |  |
| >>>>Slice DRB Usage | O |  | 9.3.1.yy |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofSharedSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 512. [FFS] |

-------------------------------------------------End of the second change----------------------------------------------------------------

-----------------------------------------------Start of the third change-----------------------------------------------------------------

9.3.1.yy	Slice DRB Usage

The Slice DRB Usage IE indicates the usage of the DRBs within a slice.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of DRB | O |  | BIT STRING (SIZE(24)) |  |
| DRB Composite Available Capacity | O |  |  | This IE indicates the overall available resource level of DRB. |

-----------------------------------------------End of the third change-----------------------------------------------------------------