3GPP TSG-RAN WG3 #110-e     					                                                    R3-206678

- 2 - 12 Nov 2020

Online

Agenda item:	10.2.4

Source: 	ZTE

Title: 	Further Discussion on Inter-system Load Balancing in NR

Document for:	Discussion and Approval

Introduction

In RAN3#109-e, the inter-system load balancing in NR has been discussed and some agreements were reached.

Introduce Inter System Load Balancing mechanisms on the basis of the solution available in E-UTRAN

Introduce Inter System Load Balancing by means of mechanisms that resemble or reuse the SON Configuration Transfer IE for the purpose of configuring load balancing metrics and reporting load balancing measurements

Use S1: eNB CONFIGURATION TRANSFER, S1: MME CONFIGURATION TRANSFER, NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER for the transfer of inter system load balancing via means of mechanisms that resemble or reuse the SON Configuration Transfer IEs. It is FFS whether further details on the signalling part need to be introduced.

Adopt signalling of the Composite Available Capacity (Cell Capacity Class value and Capacity Value) for inter system MLB. Adoption of further MLB metrics is FFS.

Event Based Reporting and Periodic Reporting (only in case specific conditions are met), are agreed to be supported for inter system MLB. The mechanism should avoid excessive signaling.

In this contribution, we try to provide the detailed signalling of inter-system MLB between NR and LTE. And we will discuss the other MLB metrics for the inter-system MLB.

Discussion

In last meeting, it was agreed that the SON Configuration Transfer IE in UL or DL RAN Configuration Transfer message over NG should be reused or resembled. However, by checking the current IE table of UL RAN Configuration Transfer message and comparing the detailed description of SON Configuration Transfer IE and Inter-system SON Configuration Transfer IE from [1], it seems that the Inter-system SON Configuration Transfer is more straightforward and appropriate to be extended to achieve the function of  Inter-system load balancing.

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

9.2.7.1	UPLINK RAN CONFIGURATION TRANSFER

This message is sent by the NG-RAN node in order to transfer RAN configuration information.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.3.3.6 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | OCTET STRING | Contains the EN-DC SON Configuration Transfer IE as defined in TS 36.413 [16]. | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | 9.3.3.33 |  | YES | ignore |

      - 9.3.3.33  Inter-system SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, transmitted between an NG-RAN node and an eNB and additionally includes the node identifier of the destination of this configuration information and the node identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Transfer Type | M |  |  |  |
| >from E-UTRAN to NG-RAN |  |  |  |  |
| >>Source eNB-ID |  | 1 |  |  |
| >>>Global eNB ID | M |  | 9.3.1.165 |  |
| >>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| >>Target NG-RAN node ID |  | 1 |  |  |
| >>>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >from NG-RAN to E-UTRAN |  |  |  |  |
| >>Source NG-RAN Node ID |  | 1 |  |  |
| >>>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >>Target eNB-ID |  | 1 |  |  |
| >>>Global eNB ID | M |  | 9.3.1.165 |  |
| >>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| Inter-system SON Information | M |  | 9.3.3.34 |  |

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

Proposal 1:  To be specific, the Inter-system SON Configuration can be reused for inter-system load balancing.

It was agreed that both event based reporting and periodic reporting could be supported for inter-system load balancing. In addition, as the Resource Status Reporting Initiation procedure has been introduced in NR, the similar principle can be used in inter-system load balancing, i.e. the eNB/gNB may request the reporting of load measurements over the S1/NG interface, and the gNB/eNB shall response over the NG/S1 interface. Therefore, the IE Inter-system SON Information can be extended to the IE Inter-system SON Information Request and the IE Inter-system SON Information Reply IE.

Proposal 2: It is proposed to extend the IE Inter-system SON Information to the IE Inter-system SON Information Request and the IE Inter-system SON Information Reply.

Considering the other load metrics for exchange between LTE and NR, after checking the IE tables of Resource Status Update message over X2 and Xn, it can be found that the TNL related load and Radio Resource Status have similar definitions in LTE and NR. However, the value format and granularity of these IEs are not same.

For TNL related load, the corresponding IE in LTE, i.e. S1 TNL Load Indicator, is to indicate the status of S1 TNL load by using low load, medium load, high load and overloaded. While the corresponding IE in NR, i.e. TNL Capacity Indicator, is to indicate the offered and available capacity of Transport Network by using the integer value. In addition, for Radio Resource Status, the corresponding IE in LTE is only in cell level, while the corresponding IE in NR could be in both cell level and beam level.

Therefore, it is not possible for the eNB and gNB to exchange the load metrics directly. As the objective of the inter-system load balancing is to make the source gNB/eNB be aware of the load status of the target eNB/gNB. For the reporting direction from NR to LTE, the load metrics in NR, the Number of active UEs, RRC Connections and TNL Capacity Indicator, could be considered as more straightaway for the eNB to obtain the load status of target gNB.

For the reporting direction form LTE to NR, in addition to the Composite Available Capacity Group. whether the other load metrics of LTE should be exchanged from eNB to gNB is FFS.

Proposal 3:  In addition to the Composite Available Capacity Group, the Number of active UEs, RRC Connections, and TNL Capacity Indicator could be considered to exchange from gNB to eNB.

Conclusion

Proposal 1:  To be specific, the Inter-system SON Configuration can be reused for inter-system load balancing.

Proposal 2: It is proposed to extend the IE Inter-system SON Information to the IE Inter-system SON Information Request and the IE Inter-system SON Information Reply.

Proposal 3:  In addition to the Composite Available Capacity Group, the Number of active UEs, RRC Connections, and TNL Capacity Indicator could be considered to exchange from gNB to eNB.

The corresponding TPs are given in [3] and [4] respectively.

References

- 3GPP TS 38.413 v16.3.0

- R3-206680 TP for BL CR 38.300 on Inter-system Load Balancing  ZTE

- R3-206681 TP for BL CR 38.413 on Inter-system Load Balancing  ZTE