3GPP TSG-RAN3 Meeting #110-e	R3-206736

# E-meeting, 2nd November - 12th November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0539 | rev | 1 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the semantics description of the PDCP SN Status Information IE in the DRB Modified List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The semantics description of the PDCP SN Status Information IE in the tabular is not correct. The PDCP SN Status Information in the DRB Modified List is transferred from the source gNB-CU-UP to the source gNB-CU-CP, in response to the PDCP SN Status Request in the DRB To Modify List. Current spec is miswritten as if the procedure occurs at the target gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The semantics description has been changed. to the target gNB-CU-UP from the source gNB-CU-UP. Impact assessment towards the previous version of the specification (same release): This CR only has no fuctional impact as the semantics description is the only change. The change is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The semantics description remains wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Resubmission to RAN3#110-e |  |  |  |  |  |  |  |  |

Start of the change

9.3.3.19	PDU Session Resource Modified List

This IE contains modified PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |  |  |
| >NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |  |  |
| >Security Result | O |  | 9.3.1.52 |  |  |  |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 |  |  |  |
| >DRB Setup List |  | 0.. 1 |  |  |  |  |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 |  |  |  |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  |  |  |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |  |  |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |  |  |
| >DRB Failed List |  | 0.. 1 |  |  |  |  |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>Cause | M |  | 9.3.1.2 |  |  |  |
| >DRB Modified List |  | 0.. 1 |  |  |  |  |
| >>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters. |  |  |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides PDCP SN Status from the source gNB-CU-UP. |  |  |
| >>>Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  |  |  |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |  |  |
| >>> Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | Indicates the QoS flow(s) for which the gNB-CU-UP has not yet received SDAP end markers after the gNB-CU-CP reconfigured those QoS flow(s) to another DRB. | Yes | ignore |
| >DRB Failed To Modify List |  | 0.. 1 |  |  |  |  |
| >>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>Cause | M |  | 9.3.1.2 |  |  |  |

End of the change