3GPP TSG-RAN3 #110-e	R3-206773

2nd November – 12th November 2020

Online

Agenda Item:	9.3.7.1

Source:	Ericsson, Verizon Wireless, Deutsche Telekom

Title:	MeasGapConfig IE and the Configuration Query IE.

Document for:	Discussion, Decision

# Introduction

In this contribution we would like to examine the relation between the MeasGapConfig IE and the Configuration Query IE. In the following we will further elaborate on this issue and bring forward our proposals

# Discussion

Currently in TS 38.473 the following is stated:

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the CellGroupConfig IE in the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

We believe that the same thinking applies to the MeasGapConfig IE.

Let’s explore the case that the last MeasGapConfig IE that was reported to the gNB-CU was generated using delta configuration. As a result, the gNB-CU does not have the latest complete MeasGapConfig IE. 
This is a similar situation to a CellGroupConfig generated via delta configurations.  In this case too, it is possible that the gNB-CU does not have the latest full version of the CellGroupConfig and needs to request it via signalling the GNB-DU Configuration Query IE.

If a handover is decided, the gNB-CU would need to have a complete MeasGapConfig IE (just like it needs a complete CellGroupConfig) when preparing for the handover. In order to enable the gNB-CU to request a full MeasGapConfig IE, the MeasGapConfig IE can be signalled back from gNB-DU to gNB-CU at reception of the GNB-DU Configuration Query IE by the gNB-DU.

Based on the above we propose that the MeasGapConfig IE should be returned also when the Configuration Query IE is received by the gNB-DU.

Proposal 1: 	the MeasGapConfig IE should be returned also when the Configuration Query IE is received by the gNB-DU.

# Conclusion

### In this contribution the relation between the MeasGapConfig IE and the Configuration Query IE has been discussed and the following proposal was made:

Proposal 1: 	the MeasGapConfig IE should be returned also when the Configuration Query IE is received by the gNB-DU.

Proposal 2: 	RAN3 is kindly asked to agree with the CR in R3-206774 and R3-206775.

# References