3GPP TSG-RAN WG3 #110-e							                                                R3-206793

2-12 November 2020

Online

Agenda item:	10.2.4

Source:	CMCC

Title:	TP to 36.300 for inter-system load balancing

Document for:	Discussion

- Introduction

This contribution provides load reporting description for inter-system load balancing in TS 36.300.

# 2	Text proposal for TS 36.300

--------------------------------------------------Start of the change---------------------------------------------------

### 22.4.1	Support for Mobility Load Balancing

#### 22.4.1.1	General

The objective of load balancing is to distribute cell load evenly among cells or to transfer part of the traffic from congested cells. This is done by the means of self-optimisation of mobility parameters or handover actions.

Self-optimisation of the intra-LTE and inter-RAT mobility parameters to the current load in the cell and in the adjacent cells can improve the system capacity compared to static/non-optimised cell reselection/handover parameters. Such optimisation can also minimize human intervention in the network management and optimisation tasks.

Support for mobility load balancing consists of one or more of following functions:

-	Load reporting;

-	Load balancing action based on handovers;

-	Adapting handover and/or reselection configuration.

Triggering of each of these functions is optional and depends on implementation. Functional architecture is presented in Figure 22.4.1.1-1.

Figure 22.4.1.1-1: Functional architecture of SON load balancing

#### 22.4.1.2	Load reporting

The load reporting function is executed by exchanging cell specific load information between neighbour eNBs over the X2 interface (intra-LTE scenario) or S1 (inter-RAT scenario and EN-DC scenario).

##### 22.4.1.2.1	Load reporting for intra-LTE scenario

The load information consists of:

-	radio resource usage (UL/DL GBR PRB usage, UL/DL non-GBR PRB usage, UL/DL total PRB usage);

-	HW load indicator (UL/DL HW load: low, mid, high, overload);

-	TNL load indicator (UL/DL TNL load: low, mid, high, overload);

-	(Optionally) Cell Capacity Class value (UL/DL relative capacity indicator: the same scale shall apply to E-UTRAN, UTRAN and GERAN cells when mapping cell capacities on this value);

-	Capacity value (UL/DL available capacity for load balancing as percentage of total cell capacity).

NOTE 1:	Capacity value is expressed in available E-UTRAN resources.

NOTE 2:	A cell is expected to accept traffic corresponding to the indicated available capacity.

##### 22.4.1.2.2	Load reporting for inter-RAT scenario

The load information consists of:

-	Cell Capacity Class value (UL/DL relative capacity indicator: the same scale shall apply to E-UTRAN, UTRAN, GERAN and eHRPD cells when mapping cell capacities on this value);

-	Capacity value (UL/DL available capacity for load balancing as percentage of total cell capacity).

NOTE 1:	Capacity value is expressed in available E-UTRAN resources.

NOTE 2:	A cell is expected to accept traffic corresponding to the indicated available capacity.

Event-triggered inter-RAT load reports are sent when the reporting node detects crossing of cell load thresholds.

Load information shall be provided in a procedure separated from existing active mode mobility procedures, which shall be used infrequently and with lower priority with respect to the UE dedicated signalling.

##### 22.4.1.2.3	Load reporting for EN-DC scenario

The load reporting function is executed by the way that an en-gNB provides its load information toward an eNB over the X2 interface.

For an NR cell, the following load related information should be supported which consists of:

-	Radio resource usage (per-SSB-area PRB usage: DL/UL/SUL GBR PRB usage, DL/UL/SUL non-GBR PRB usage, DL/UL/SUL total PRB usage, and DL/UL/SUL scheduling PDCCH CCE usage);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (UL/DL available capacity);

To achieve load reporting function, EN-DC Resource Status Reporting Initiation & EN-DC Resource Status Reporting procedures are used.

##### 22.4.1.2.4	Load reporting for E-UTRAN NR inter-system scenario

The load information consists of:

-	radio resource usage (UL/DL GBR PRB usage, UL/DL non-GBR PRB usage, UL/DL total PRB usage);

-	HW load indicator (UL/DL HW load: low, mid, high, overload);

-	TNL load indicator (UL/DL TNL load: low, mid, high, overload);

-	Cell Capacity Class value (UL/DL relative capacity indicator: the same scale shall apply to E-UTRAN and NR cells when mapping cell capacities on this value);

-	Capacity value (UL/DL available capacity for load balancing as percentage of total cell capacity);

-	Number of Active UEs (UL/DL number of active UEs per QCI).

To achieve load reporting function, the procedures are FFS.

Both event-triggered and periodic inter-system load reporting are supported. Event-triggered inter-system load reports are sent when the reporting node detects crossing of cell load thresholds.

#### 22.4.1.3	Load balancing action based on handovers

The source cell may initiate handover due to load (see clauses 10.1.2 and 10.2.2). The target cell performs admission control for the load balancing handovers. A handover preparation related to a mobility load balancing action shall be distinguishable from other handovers, so that the target cell is able to apply appropriate admission control.

#### 22.4.1.4	Adapting handover and/or reselection configuration

This function enables requesting of a change of handover and/or reselection parameters at target cell. The source cell that initialized the load balancing estimates if it is needed to change mobility configuration in the source and/or target cell. If the amendment is needed, the source cell initializes mobility negotiation procedure toward the target cell.

The source cell informs the target cell about the new mobility setting and provides cause for the change (e.g. load balancing related request). The proposed change is expressed by the means of the difference (delta) between the current and the new values of the handover trigger. The handover trigger is the cell specific offset that corresponds to the threshold at which a cell initialises the handover preparation procedure. Cell reselection configuration may be amended to reflect changes in the HO setting. The target cell responds to the information from the source cell. The allowed delta range for HO trigger parameter may be carried in the failure response message. The source cell should consider the responses before executing the planned change of its mobility setting.

All automatic changes on the HO and/or reselection parameters must be within the range allowed by OAM.

--------------------------------------------------End of the change-----------------------------------------------------