3GPP TSG-RAN WG3 #110-e							                                                R3-206795

2-12 November 2020

Online

Agenda item:	15.2

Source:	CMCC

Title:	TP to 38.890 for NR QoE management

Document for:	Discussion

- Introduction

This contribution provides potential solutions and procedures for NR QoE management in TR 38.890.

# 2	Text proposal for TR 38.890

--------------------------------------------------Start of the change---------------------------------------------------

6	Potential NR QoE solutions and procedures

Editor note: Description of potential NR QoE solutions and procedures, including but not limited to reuse Trace or MDT Functionality/Framework.

## 6.X RAN-initiated procedure for NR QoE measurement

The following figure gives an illustrative call flow for RAN-initiated NR QoE measurement,

Figure 1 RAN-initiated NR QoE measurement procedures

- NG-RAN decides to initiate QoE measurement by UE, and transmit RAN-initiated QoE Configuration including service type to UE AS.

- UE AS obtains the service type that triggers the QoE measurement, and forwards the QoE Configuration to UE NAS.

- UE NAS performs QoE measurement according to event-based and/or time-based criteria, generates QoE report once the situation meets the criteria, and sends QoE report to UE AS.

- UE AS associates radio related measurements with QoE report, after UE AS receives the QoE report from UE NAS.

- UE AS sends QoE report, together with associated radio related measurements to RAN.

- RAN is able to identify the QoE degradation caused by RAN, based on associated radio related measurements and/or QoE report.

--------------------------------------------------End of the change-----------------------------------------------------