3GPP TSG-RAN3 #110-e	R3-206808

2nd November – 12th November 2020

Online

Agenda Item:	09.3.7.1

Source:	Ericsson, Verizon Wireless, Deutsche Telekom

Title:	Transparent signalling of MeasGapConfig IE at gNB-CU

Document for:	Discussion, Decision

# Introduction

In this contribution we would like to explore the consequences of including the MeasGapConfig IE in the UE CONTEXT SETUP RESPONSE message, UE CONTEXT MODIFICATION RESPONSE message and the UE CONTEXT MODIFICATION REQUIRED message. In the following we will further elaborate on this issue and bring forward our proposals

# Discussion

Currently in TS 38.473 the following is stated:

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

We believe that the same thinking applies to the MeasGapConfig IE. This would serve as a clarification in TS 38.473 and also it should be included for consistency with the statement on CellGroupConfig IE.

Based on the above we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume. The MeasGapConfig IE shall transparently be signalled to the UE.

Proposal 1: 	we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume. The MeasGapConfig IE shall transparently be signalled to the UE.

By similar reasoning the same applies also for the UE CONTEXT MODIFICATION RESPONSE message and the UE CONTEXT MODIFICATION REQUIRED message. So, the following proposals are made.

Proposal 2: 	we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration. The MeasGapConfig IE shall transparently be signalled to the UE.

Proposal 3: 	we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall perform RRC Reconfiguration. The MeasGapConfig IE shall transparently be signalled to the UE.

# Conclusion

### In this contribution the consequences of including the MeasGapConfig IE in the UE CONTEXT SETUP RESPONSE message and the UE CONTEXT MODIFICATION RESPONSE message has been discussed and the following proposal was made:

Proposal 1: 	we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume. The MeasGapConfig IE shall transparently be signalled to the UE.

Proposal 2: 	we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration. The MeasGapConfig IE shall transparently be signalled to the UE.

Proposal 3: 	we propose that it is clarified in TS 38.473 that if the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall perform RRC Reconfiguration. The MeasGapConfig IE shall transparently be signalled to the UE.

Proposal 4: 	RAN3 is kindly asked to agree with the CRs in R3-206809 and R3-206810.

# References