3GPP TSG-RAN WG3 Meeting #110e	R3-206816

# Online Meeting, 2nd – 12th November 2020

Agenda Item:	9.3.7.1

Source:	Ericsson, China Telecom, China Unicom

Title: 	Handling of unknown RRC establishment cause

Document for:	Discussion and Approval

# 1	Discussion

According to the RRC specification TS38.331 a gNB is not expected to reject an RRCSetupRequest due to unknown cause value being used by the UE.

RRCSetupRequest message -- ASN1START -- TAG-RRCSETUPREQUEST-START RRCSetupRequest ::= SEQUENCE { rrcSetupRequest RRCSetupRequest-IEs } RRCSetupRequest-IEs ::= SEQUENCE { ue-Identity InitialUE-Identity, establishmentCause EstablishmentCause, spare BIT STRING (SIZE (1)) } InitialUE-Identity ::= CHOICE { ng-5G-S-TMSI-Part1 BIT STRING (SIZE (39)), randomValue BIT STRING (SIZE (39)) } EstablishmentCause ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess, spare6, spare5, spare4, spare3, spare2, spare1} -- TAG-RRCSETUPREQUEST-STOP -- ASN1STOP RRCSetupRequest-IEs field descriptions. establishmentCause Provides the establishment cause for the RRCSetupRequest in accordance with the information received from upper layers. gNB is not expected to reject an RRCSetupRequest due to unknown cause value being used by the UE.. ue-Identity UE identity included to facilitate contention resolution by lower layers..

NG-RAN needs to map the RRC RRC Establishment case in the NGAP message, for example I the Initial UE Message. However it does not provide the mapping which can be used to indicate that the RRC Establishment case is unkown.

What to do when the UE provides an unknown establishment cause over RRC is unspecifed

The actions that the NG-RAN node may taken, depends on the implemention could be as in below:

- Use a radon cause as RRC Establishmen cause in the Initial UE message.

- Ignore RRC establishment request by the UE

- Reject RRC establishment request by the UE

There is a cause code in NGAP “notAvilable”, we propose to specify so that the Unknow RRC Est cause can be mapped to it.

#### 9.3.1.111 RRC Establishment Cause This IE indicates the reason for RRC Connection Establishment as received from the UE in the EstablishmentCause defined in TS 38.331 [18] and TS 36.331 [21], or the reason for RRC Connection Resume as received from the UE in the ResumeCause defined in TS 38.331 [18] and TS 36.331 [21], or the reason for RRC Connection Establishment as received from the UE in the EstablishmentCause-NB defined in TS 36.331 [21]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RRC Establishment Cause; M; ENUMERATED (emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess, …, notAvailable, mo-ExceptionData); The notAvailable value is used in case the UE is re-establishing an RRC connection but there is fallback to RRC connection establishment as described in [18], or the ResumceCause received from the UE does not map to any other value of the RRC Establishment Cause IE, or the EstablishmentCause received from the UE is unknown..

Proposal 1: RAN3 to agree to specify to use the “notAvailable” code point  in NGAP for the Unknown RRC EstablishmentCause.

The updates to the text as in the submitted CR [1] and [2].

# 2	Proposals

Proposal 1: RAN3 to agree to specify to use the “notAvailable” code point  in NGAP for the Unknown RRC EstablishmentCause.

# 3	Reference

[1]	R3-206814	Handling of unknown RRC establishment cause, Ericsson

[2]	R3-206815	Handling of unknown RRC establishment cause, Ericsson