3GPP TSG-RAN WG3 #110 electronic	R3-206835

Online, 2-12 November, 2020

Agenda Item:	22.2.4

Source:	CATT

Title:	Consideration on MBS context management over F1 and E1

Document for:	Discussion, Decision

1	Introduction

In last RAN3 meeting, some agreements of MBS session resource over NG has been made:

We define MBS session resource in analogy with PDU session resource, e.g. including radio part, CP part, NG-UP part, MBS context in RAN

MBS session resource establishment is requested by 5GC (similarly to the PDU session establishment for unicast)

RAN may request MBS session resource UP establishment, e.g. in handover (FFS). The signaling procedure (e.g. nested in handover signaling or new procedure, whether a single procedure is used or not, …) is FFS.

In this contribution, we provide some analysis and proposals on MBS context management over F1 and E1 interface.

2	Discussion

## 2.1	Control Plane over F1 interface：

Both broadcast and multicast sevice are applicable for supporting MBS reception of Idle or Inactive UE. According to the email discussion of RAN2 on MBS Idle mode support [2], most companies assume that LTE SC-PTM technique can be used as the baseline of MBS reception for Idle or Inactive UE.

Observation 1: The LTE SC-PTM technique can be used as the baseline of NR MBS reception for Idle or Inactive UE.

In legacy SC-PTM, there is a MCCH and multiple MTCH logical channels, in which the MTCH logical channel is established and maintained per MBMS session, while for NR MBS, if the LTE SC-PTM technique can be reused, there should also be a separate PTM logical channel for MBS reception of Idle or Inactive UE, e.g, namely NR MRB. Accordingly, an independent radio configuration procedure for the PTM channel should be considered

Observation 2: An independent radio configuration procedure for the PTM channel can be considered in NR MBS.

Based on LTE SC-PTM mechanism, the signaling flow over RAN interface is shown as follows:

The MCE in above figure acts as the decision maker of SC-PTM or MBSFN mode, and it transparently signals the MBMS session related information from CN towards eNB by a non UE associated procedure over M2. And then, the radio configuration of MTCH logical channel for the MBMS session is independently configured to Idle UE by eNB itself. Similarly for NR MBS, the radio configuration of PTM logical channel for certain MBS session should also be independently configured to Idle or Inactive UE by gNB itself.

Observation 3: The radio parameters of NR PTM logical channel for a MBS session can be independently configured to Idle or Inactive UE by gNB itself.

Furthermore, for RAN split architecture, from the perspective of functional allocation, gNB-CU hosts RRC, SDAP and PDCP protocols of the gNB, and gNB-DU hosts RLC, MAC and PHY layers of the gNB.

According to RAN2's email discussion on the MBS L2 architecture for PTM channel [3], most companies assume that the function of mapping from QoS flows to MBS RBs in SDAP is needed for NR MBS. Additionally, the following PDCP functions are at least supported for NR MBS:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	duplicate discarding.

Therefore, it can be assumed that SDAP and PDCP entities are required for NR MBS

Observation 4: It can be assumed that SDAP and PDCP entities are required for NR MBS

In this way, gNB-CU needs to configure SDAP and PDCP layers for each NR PTM logical channel, while the configuration of other layers such as RLC, MAC, PHY should be done by gNB-DU.

Proposal 1: gNB-CU is responsible for configuring SDAP and PDCP layers of NR PTM logical channel, while gNB-DU is responsible for configuring other layers such as RLC, MAC, PHY.

Specifically, to inform gNB-DU of the configuration information for these NR PTM logical channels, a new non-UE associated procedure needs to be introduced into F1 interface; This procedure is responsible for assigning PTM bearer identity and transmitting MBS session related information, such as Qos profile, to gNB-DU .

Proposal 2: A new non-UE associated procedure needs to be introduced into F1 interface to independently transfer the configuration information for NR PTM logical channel and MBS session related information, such as Qos profile, etc.

In addition, for MBS reception of connected UE, gNB is allowed to transmit MBS data via PTP or split mode (PTP+PTM), and which mode is used should be under the discussion of RAN2. However, at least in terms of protocol stack architecture, either PTP or split mode (PTP+PTM) needs to support the entire UP protocol stack, i.e. SDAP+PDCP+RLC+MAC, as well as at least needs to configure the radio parameters of the PTP logical channel for MBS reception of connected UE.

Similarly，for RAN split architecture, gNB-CU needs to configure SDAP and PDCP layers for each NR PTP logical channel, while the configuration of other layers such as RLC, MAC, PHY should be done by gNB-DU

Proposal 3: gNB-CU is responsible for configuring SDAP and PDCP layers of NR PTP logical channel, while gNB-DU is responsible for configuring other layers such as RLC, MAC, PHY.

Specifically, to inform gNB-DU of the configuration information for these NR PTP logical channels, a UE Associated Procedure is required on the F1 interface; This procedure is responsible for assigning PTP bearer identities and transmitting MBS session-related information, such as QOS parameter information, etc.

According to the last RAN3 discussion, most companies believe that "the UE context on F1 should contain MBS context information." But it needs to be clarified that the MBS context information here is a parameter configuration related to PTP logical channel.

Proposal 4: The UE context on F1 should contain MBS context information，which is related to PTP logical channel.

2.2	User Plane over F1 interface：

For 5G network architecture with CU and DU split, CP and UP separation, how to transfer MBS service data over F1 interface needs further discussion in RAN3. Based on the classification of MBS transport mode over N3 interface, it is necessary to distinguish the two different transport modes over F1 interface.

For 5GC Shared MBS traffic delivery method, there are two cases to be discussed. One case is that PTP mode over Uu interface is applied, for this case, an individual MBS transport tunnel should be established for each MBS session of UE between CU and DU. Other case is that PTM mode over Uu interface is applied, for this case, a F1 shared MBS transport mode should be established within the DU, it means that a single copy of those MBS data packets is deliverd to a DU, which then delivery them to one or more cells.

In addition, for split mode (PTP+PTM), the “split” point between PTM and PTP should be PDCP, and PDCP feedback/retransmission could be supported for at least the services which need a relatively high QoS demand. If PDCP feedback is supported, it seems more suitable to deliver the retransmitted packets over the PTP leg. And if the gNB is a split one, this packet should be delivered in a separate F1-U tunnel, or otherwise the gNB-DU cannot deduce the UE toward which the packet should be sent. The case on transmitting the forwarded data in the target gNB is similar. Therefore, separate F1-U tunnel should be used for PTP delivery other than the “main” F1-U tunnel used for PTM delivery.

As depicted in the following figure, individual and shared MBS transport mode can exist over F1 interface simultaneously:

Proposal 5: In case PTP mode over Uu interface is applied, a F1 individual MBS transport tunnel should be established for each MBS session of UE between CU and DU.

Proposal 6: In case PTM mode over Uu interface is applied, a F1 shared MBS transport mode should be established for each MBS session between CU and DU.

Proposal 7: Individual and shared MBS transport mode can exist over F1 interface simultaneously.

2.3	Impact over E1 interface：

For separation architecture of user and control plane, it is necessary to consider how to design the signaling procedure over E1 interface. Similarly, from the perspective of functional allocation, gNB-CU-UP hosts SDAP and PDCP Protocols of the UP plane. Therefore, according to the above discussion on the MBS signaling procedure over F1 interface, the configuration information of SDAP and PDCP layers for each NR PTM logical channel should be transferred between gNB-CU-CP and gNB-CU-UP. Additionally, the configuration information of SDAP and PDCP layers for each NR PTP logical channel should be transferred between gNB-CU-CP and gNB-CU-UP.

Proposal 8: The configuration information of SDAP and PDCP layers for each NR PTM logical channel should be transferred between gNB-CU-CP and gNB-CU-UP

Proposal 9: The configuration information of SDAP and PDCP layers for each NR PTP logical channel should be transferred between gNB-CU-CP and gNB-CU-UP

Specifically, to inform gNB-CU-UP of the configuration information for these NR PTM logical channels, a new non-UE associated MBS bearer management procedure needs to be introduced into E1 interface; This procedure is responsible for assigning PTM bearer identity and transmitting MBS session related information, such as Qos profile, and radio parameters of SDAP and PDCP layer for the MBS session to gNB-CU-UP .

Proposal 10: A new non-UE associated MBS bearer management procedure needs to be introduced into E1 interface to independently transfer the configuration information for NR PTM logical channel and MBS session related information, such as Qos profile, etc.

Specifically, to inform gNB-CU-UP of the configuration information for these NR PTP logical channels, a UE associated bearer management Procedure is required on the E1 interface; This procedure is responsible for assigning PTP bearer identities and transmitting MBS session-related information, such as QOS parameter information, and parameter configurations for SDAP and PDCP

Proposal 11: Current BEARER CONTEXT SETUP REQUEST procedure can be reused to transfer MBS context information，which is related to PTP logical channel, to gNB-CU-UP.

3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: The LTE SC-PTM technique can be used as the baseline of NR MBS reception for Idle or Inactive UE.

Observation 2: An independent radio configuration procedure for the PTM channel can be considered in NR MBS.

Observation 3: The radio parameters of NR PTM logical channel for a MBS session can be independently configured to Idle or Inactive UE by gNB itself.

Observation 4: It can be assumed that SDAP and PDCP entities are required for NR MBS

Proposal 1: gNB-CU is responsible for configuring SDAP and PDCP layers of NR PTM logical channel, while gNB-DU is responsible for configuring other layers such as RLC, MAC, PHY.

Proposal 2: A new non-UE associated procedure needs to be introduced into F1 interface to independently transfer the configuration information for NR PTM logical channel and MBS session related information, such as Qos profile, etc.

Proposal 3: gNB-CU is responsible for configuring SDAP and PDCP layers of NR PTP logical channel, while gNB-DU is responsible for configuring other layers such as RLC, MAC, PHY.

Proposal 4: The UE context on F1 should contain MBS context information，which is related to PTP logical channel.

Proposal 5: In case PTP mode over Uu interface is applied, a F1 individual MBS transport tunnel should be established for each MBS session of UE between CU and DU.

Proposal 6: In case PTM mode over Uu interface is applied, a F1 shared MBS transport mode should be established for each MBS session between CU and DU.

Proposal 7: Individual and shared MBS transport mode can exist over F1 interface simultaneously.

Proposal 8: The configuration information of SDAP and PDCP layers for each NR PTM logical channel should be transferred between gNB-CU-CP and gNB-CU-UP

Proposal 9: The configuration information of SDAP and PDCP layers for each NR PTP logical channel should be transferred between gNB-CU-CP and gNB-CU-UP

Proposal 10: A new non-UE associated MBS bearer management procedure needs to be introduced into E1 interface to independently transfer the configuration information for NR PTM logical channel and MBS session related information, such as Qos profile, etc.

Proposal 11: Current BEARER CONTEXT SETUP REQUEST procedure can be reused to transfer MBS context information，which is related to PTP logical channel, to gNB-CU-UP.

4	Reference

- RP-201038    “New Work Item on NR Multicast and Broadcast Services”

- R2-200xxxx	Summary of Email Discussion [Post111-e][906][MBS] Idle mode support, CATT

- R2-200xxxx	Email discussion [Post111-e][904][MBS] L2 Architecture, Huawei

5	Text Proposals for stage 3

5.1	TP for TS 38.463

--------------------------------Start of the First Change-----------------------------

8	E1AP procedures

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| MBS Bearer Context Setup |  | MBS BEARER CONTEXT SETUP REQUEST |  | MBS BEARER CONTEXT SETUP RESPONSE |  | MBS BEARER CONTEXT SETUP FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |

------------------------------------------Next Change---------------------------------------

8.3.X	MBS Bearer Context Setup

8.3.X.1	General

The purpose of the MBS Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a MBS bearer context in the gNB-CU-UP.

8.3.X.2	Successful Operation

TBD

8.3.X.3	Unsuccessful Operation

TBD

8.3.X.4	Abnormal Conditions

TBD

------------------------------------------Next Change---------------------------------------

9.2.Y.1	MBS BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a MBS bearer context.

Direction: Gnb-CU-CP  Gnb-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.Y.2	MBS BEARER CONTEXT SETUP RESPONSE

This message is sent by the Gnb-CU-UP to confirm the setup of the requested MBS bearer context.

Direction: Gnb-CU-UP  Gnb-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.Y.3	MBS BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the MBS bearer context was unsuccessful.

Direction: Gnb-CU-UP  Gnb-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |

- -------------------------------------End of the Changes----------------------------------

5.2	TP for TS 38.473

--------------------------------Start of the First Change-----------------------------

8	F1AP procedures

8.1	List of F1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  |  |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  |  |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  |  |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| MBS Context Setup |  | MBS CONTEXT SETUP REQUEST |  | MBS CONTEXT SETUP RESPONSE |  | MBS CONTEXT SETUP FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |

------------------------------------------Next Change---------------------------------------

8.3.X	MBS Context Setup

8.3.X.1	General

The purpose of the MBS Context Setup procedure is to establish the MBS Context including, MRB configuration.

8.3.X.2	Successful Operation

TBD

8.3.X.3	Unsuccessful Operation

TBD

8.3.X.4	Abnormal Conditions

TBD

------------------------------------------Next Change---------------------------------------

9.2.Y.1	MBS CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a MBS context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.Y.2	MBS CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a MBS context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.Y.3	MBS CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the MBS context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |

- -------------------------------------End of the Changes----------------------------------