3GPP TSG-RAN3 Meeting #110-e	R3-206839

Electronic meeting, 2nd - 12th November 2020

3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007880

October 12 – 23, 2020, Electronic

# Title:	LS on Use of Survival Time for Deterministic Applications in 5GS

# Release:	Rel-17

# Work Item:	FS_IIoT

Source:	3GPP SA WG2

To:	3GPP RAN WG2, RAN WG3

Cc:	3GPP SA WG1

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	Devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

3GPP SA WG2 has studied a key issue for introducing “Use of Survival Time for Deterministic Applications in 5GS” and selected the solutions as the basis for normative work. One of the basic principles of the agreed conclusions is that SMF determines TSCAI Survival Time and sends it to the NG-RAN. The Survival Time in TSCAI may be expressed, assuming cyclic traffic of the deterministic application:

- as a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI in Rel-16; or

- as a maximum number of consecutive data burst transmission failures, where a data burst corresponds to a single application message.

i) and ii) are intended to be equivalent in determining the application’s tolerance to loss. Note that, when survival time is provided, there is a single data burst per period. SA WG2 kindly requests 3GPP RAN WG2 to provide their preference of Survival Time definition i) or ii) as defined above.

These definitions correspond to what is specified in TS 22.104, annex C.3.

SA2 also discussed whether receiving Survival time over NGAP is sufficient assistance information for NG-RAN to address the performance targets laid out by SA1 in Table 5.2-1 in TS 22.104.

Note that according to Table 5.2-1 in TS 22.104 different services may share the same survival time but very different communication service availability targets.

2. Actions:

To 3GPP RAN WG2, 3GPP RAN WG3

ACTION:

RAN2, RAN3: SA WG2 kindly requests 3GPP RAN WG2 & 3GPP RAN WG3 to take into account SA2’s conclusion of sending Survival Time in TSCAI from SMF to NG-RAN. It is up to RAN WG2 and RAN WG3 to discuss how NG-RAN uses Survival Time.

SA WG2 also kindly requests 3GPP RAN WG2 and RAN WG3 to provide their feedback on the preference of Survival Time definition of i) or ii) as defined above and to inform SA2 whether receiving survival time is sufficient for NG-RAN to address the performance targets (same Survival Time but different communication service availability for different services) laid out by SA1 in Table 5.2-1 in TS 22.104.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #142E   		16 - 20 November 2020