3GPP TSG-RAN WG3 #110-e	R3-206885

Online, November 2nd – 12th, 2020

Agenda Item:	10.2.3

Source:	CMCC (moderator)

Title:	Support of inter-system inter-RAT energy saving

Document for:	Discussion and Decision

Introduction

-  Topics to discuss:

- Activation ID

- Inter-system SON Information activation and cell state IEs

- S1 and NG signaling details

- Stage-2

- May also discuss other topics based on contributions

- You may start the discussion based on TPs; Alternatively, you may have a first phase for high level agreements and then proceed to the TPs in the second phase

(CMCC - moderator)

Summary of offline disc R3-206885

Since the views from the papers proposed in RAN3#110-e meeting are relatively consistent, we hope to reach the agreement on stage2 TPs, stage3 detail IEs and even stage 3 TPs if possible before Thursday.

For the Chairman’s Notes

Propose to agree the following proposals:

Inter-system SON Information Request/Rely IEs are carried at the top-level Inter-system SON Information IE and Cell State Indication IE is carried in the sub-level IE Inter-system SON Information Report for NG and S1 signalling.

An Activation ID should be included in cell activation request and reply messages.

A list of cells that the eNB wants to activate should be added in the cell activation request IE.

Activated cell list should be added in cell activation reply IE as a response to the cell activation request.

Stage 2：

Agree the TP for TS38.300 in R3-207008 revision of R3-206799.

Agree the TP for TS36.300 in R3-207007 revision of R3-206797.

Stage 3：

Agree the TP for TS 38.413 in R3-206098

Agree the TP for TS 36.413 in R3-206698

Open Issues:

No consensus on minimum activation time and a new cell status,  need further discussion.

Discussion

S1 and NG Signaling Details

There are two options to carry the detailed IEs in the Inter-system SON Information IE.

Option 1: Inter-system SON Information Request/Rely IEs are carried in the top-level Inter-system SON Information IE and Cell State Indication IE is carried in the sub-level IE Inter-system SON Information Report.

Option 2: All detailed IEs are carried in the sub-level IE Inter-system SON Information Report.

In reference paper, companies in [1] [3] [4] and [5] prefer to choose the Option 1. As comparison, the company in [2] proposes the Option 2. It seems that most of companies prefer Option 1. So the rapporteur proposes to agree:

Proposal 1: Agree to use option 1 for NG and S1 signalling.

Question 1: Companies are invited to give your views on the proposal 1.

| Company | Comment |
| --- | --- |
| Huawei | Agree to proposal 1. |
| CMCC | Agree to proposal 1. |
| China Telecom | Agree to proposal 1 |
| Qualcomm | Agree |
| ZTE | Agree to proposal 1. |
| Ericsson | We can accept the majority view and go for proposal 1 |
| Lenovo, Motorola Mobility | Agree to proposal 1. |
| Samsung | Proposal 1 is ok for us as well. |

Stage2 Details

After the in depth discussion in RAN3 #109e meeting, the essential principles of Inter-System Inter RAT energy saving have been addressed. R3-205665 and R3-205666 are agreed as the basis for the stage 2 TPs. In this meeting, the two TPs are updated in [6] and [7] by further refining the text.

Question: Do you agree with the text proposal for TS 36.300 and TS 38.300 provided in R3-206797 and R3-206799?

| Company | Comment |
| --- | --- |
| Huawei | We are fine with the two stage 2 TPs. |
| CMCC | We agree the two stage 2 TPs. |
| China Telecom | Fine with two stage 2 TPs |
| Qualcomm | Fine with both the stage 2 TPs. One thing we seek clarification is whether we need to clarify what happens in case the cell activation/deactivation requests are sent in the other direction i.e. an NG-RAN requests an eNB to switch on or off for ES purpose (please note that the proposed stage 3 signaling doesn’t prohibit this signaling although this might be a use case of little practical interest). A clarification in stage 2 or stage 3 text that this signaling in opposite direction is not supported or left to RAN implementation (can be ignored) can be useful. |
| ZTE | Fine with two stage 2 TPs |
| Ericsson | We think Stage 2 needs to be modified. For the TP to 36.300, it is not correct to state that E-UTRAN provides the coverage layer. This might be true today, but it will not be the case in e.g. 2 years from now. The text in TS36.300 and 38.300 should be future proof. Therefore for R3-206797 we foresee the following changes: The basic coverage may be provided by - E-UTRAN, UTRAN or GERAN cells, in the case of E-UTRA cells; - E-UTRA cells, in the case of EN-DC cells. - E-UTRA cells, in the case of NR cells. […] ##### 22.4.4.2.3 NR cell case For Inter-RAT Inter-system energy saving, in case the eNB providesing basic coverage, it may request a NR cell re-activation based on its own cell load information or neighbour cell load information and receive the cell re-activation reply. The switch-on decision may also be taken by O&M. The eNB can be notified of the status of the concerned NR cell. The cell activation, cell activation reply and cell status notification information are transferred over S1 interface and NG interface. For R3-206797, we see the following changes: The eNB NG-RAN node providing basic coverage may request a NG-RAN node’s NR cell re-activation based on its own cell load information or neighbour cell load information, the switch-on decision may also be taken by O&M. The eNB NG-RAN node providing basic coverage requests a NR NG-RAN node’s cell re-activation towards the NG-RAN node and receivesed the NG-RAN node’s NR cell re-activation reply from the NG-RAN node over the S1 interface and NG interface. With these changes we can cosign the TPs. |
| Lenovo, Motorola Mobility | Fine with two stage 2 TPs. |
| Samsung | Fine with two stage 2 TPs. |
| CMCC | For Qualcomm’s clarification on Stage2 details, the scenario is eNB cell provides basic coverage while NR cell serves capacity booster, and the stage 2 describes this scenario and solutions. The other direction is not precluded, but for the moment, the scenario for the other direction is not clear, so for the moment clarification is not needed We are OK with Ericsson’s most of the modification on Stage2 TPs, except the change from “eNB” to “NG-RAN node”. In the modification, it seems describing intra-system EE, not inter-system inter-RAT EE. But we are addressing inter-system. So we think this change is not needed. Other parts we are fine. Revised TPs will be provided. |

Stage 3 IE details

Activation ID

Based on the reference papers, an activation ID should be included in the request and response messages aiming that the eNB can track and combine messages that belongs to a particular cell.

Proposal 2: An Activation ID should be included in cell activation request and reply messages.

Question: Do you agree with introducing an activation ID in cell activation request and reply messages?

| Company | Comment |
| --- | --- |
| Huawei | Agree. This is needed for multiple parallel activation transactions towards a single gNB. |
| CMCC | Agree. |
| China Telecom | Agree |
| Qualcomm | Agree. Activation ID is useful to track the response messages in case of multiple activation requests |
| ZTE | Agree |
| Ericsson | Agree |
| Lenovo, Motorola Mobility | Agree |

Cells to Activate List

Based on the reference papers, a list of cells that the eNB wants to activate can be added in the cell activation request IE.

Proposal 3: A list of cells that the eNB wants to activate should be added in the cell activation request IE.

Question: Do you agree with introducing cells to activate list in the cell activation request IE?

| Company | Comment |
| --- | --- |
| Huawei | Agree |
| CMCC | Agree. |
| China Telecom | agree |
| Qualcomm | Agree |
| ZTE | Agree |
| Ericsson | Agree |
| Lenovo, Motorola Mobility | Agree |
| Samsung | Agree |

Activated Cells List

Based on the reference paper, activated cell list is a list of cells which have already been activated by the gNB can be added in cell activation response IE.

Proposal 4: Activated cell list should be added in cell activation reply IE as a response to the cell activation request.

Question: Do you agree with introducing activated cell list in cell activation reply IE?

| Company | Comment |
| --- | --- |
| Huawei | Agree. |
| CMCC | Agree. |
| China Telecom | Agree |
| Qualcomm | Agree |
| ZTE | Agreed |
| Ericsson | Agree |
| Lenovo, Motorola Mobility | Agree |
| Samsung | Agree |

Minimum Activation Time

The minimum activation time that the requesting node wants the cells to be activated can be contained in the cell activation request to avoid ping-pong effect.

Proposal 5: The minimum activation time that the requesting node wants the cells to be activated should be contained in the cell activation request.

Question: Do you agree with introducing minimum activation time for inter-system inter-RAT energy saving?

| Company | Comment |
| --- | --- |
| Huawei | OK to have. But no strong view. |
| CMCC | Agree. |
| China Telecom | Agree |
| Qualcomm | OK, will make it uniform with inter-RAT ES between U-TRAN and E-UTRAN |
| ZTE | Not see the necessary. The behavior after receiving this IE needs more discussion. We agree that the booster cell can only return dormant mode after the minimum timer expired, but it is not needed to specify the behavior as “prevent idle mode UEs from camping on the cell and may prevent incoming handovers to the same cell”. |
| Ericsson | We do not see the need for this. |
| Lenovo, Motorola Mobility | Agree. |
| Samsung | Agree |

Cell Status

- In [8], it is stated the essence of energy saving solution is to switch off some radio resources of a concern cell. Given that the NR cell capacity is significantly reduced in case the energy saving features enabled, 4G network can be configured with some RRM policies to not handover or redirection UEs to 5G SA network. Therefore, based on the above requirements, a new cell state “low power consumption” was proposed to be introduced in Rel-17 inter-RAT inter-system energy saving.

Question: Do you agree to introduce a new cell status “low power consumption” in Rel-17?

| Company | Comment |
| --- | --- |
| CMCC | In general, we are supportive with the proposal. In our view, the new cell status is not dedicated for inter-system inter-RAT EE, it can be also applied in intra-system EE. On the other hand, the new cell state “low power consumption” should be clearly defined to make it interoperable. |
| China Telecom | Agree. As stated in [8], the switching off operation is not a good choice for energy saving in practical network. “low power consumption” is meant to indicate 4G network not to handover or redirection any UEs to 5G cell. in this meeting, we need to discuss whether to introduce the new cell state “low power consumption” in Rel-17. if this state can be agreed, intra-system scenario can be treated in TEI-16 in next meeting. |
| Qualcomm | Might be useful. Can discuss this later |
| ZTE | Agree with CMCC, it can be also applied in intra-system energy saving. |
| Ericsson | We do not think this is needed, nor this is interoperable. It is not possible to understand what is the behavior of the node receiving the new IE: will the node reduce its capacity? And how? Would the node reduce its coverage? And of how much? To achieve a fully interoperable solution the complexity of the standardization effort increases considerably. Given that the mechanisms described are implementation specific we would like to leave these aspects out of 3GPP |
| Lenovo, Motorola Mobility | Same view with CMCC. In general, we are supportive with the proposal. But “low power consumption” state should be clearly defined |
| Huawei | This is a new concept and it is the first time to discuss. We are OK to further study the feasibility and to clarify the questions from all companies above. |
| Samsung | Same view with Huawei. |

Conclusion, Recommendations [if needed]

If needed

References

- R3-206796 Discussion on inter-system inter-RAT energy saving, CMCC.

R3-206047 TP for SON BLCR for 38.423: Inter-System Inter-RAT Energy Saving, Samsung.

R3-206098 (TP for SON BL CR for TS 38.413): Inter-system inter-RAT energy saving, Huawei.

R3-206184 Inter-system Inter-RAT Energy Saving, Qualcomm Inc.

R3-206699 (TP for SON BL CR for TS 38.413) Introduce Inter-System Inter-RAT Energy Saving, ZTE.

R3-206797, TP to 36.300 for inter-system inter-RAT energy saving, CMCC.

- R3-206799, TP to 38.300 for inter-system inter-RAT energy saving, CMCC.

- R3-206536,New Cell Status for Energy Saving Operation, China Telecom