3GPP TSG-RAN WG3 Meeting #110-e	R3-206886

E-meeting, 2 – 12 November, 2020

Agenda item:	10.2.4

Source:	Ericsson (moderator)

Title:		CB: # 1010_SONMDT_InterSystemLoad - Summary of email discussion

Document for:	Approval

# 1	Introduction

This paper provides summary of discussions at RAN#110-e on:

CB: # 1010_SONMDT_InterSystemLoad

-  Topics to discuss:

- Load measurements reporting granularity (e.g. cell level, beam level and per slice)

- Inter-system SON Information load balancing IEs

- Details of event-based load balancing

- Details of periodic load balancing

- Number of active UEs, RRC Connections, and TNL Capacity Indicator

- NG, S1 and stage-2 TPs

- May also discuss other topics based on contributions

- Propose to have the discussion in two phases; if there are agreements in the first phase, can proceed to discuss TPs in the second phase

(E/// - moderator)

# 2	For the Chairman’s Notes

Proposals after first round of discussions:

Conclusion 1: It is proposed to agree to introduce a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via modification of the Inter-System SON Information IE

Conclusion 2: It is proposed to agree to introduce a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via reuse of the Inter-System SON Configuration Transfer IE

Conclusion 3: It is proposed to agree to periodic inter system load reporting with periodicity not lower than 1000ms

FFS whether to send an LS to CT

Conclusion 4: It is proposed to agree to introduce threshold based load reporting.

Conclusion 5: Given the majority of companies in favour of explicitly configured thresholds (e.g. Upper and Lower Thresholds based on CAC values and reporting criteria such as “reporting above threshold”, “reporting below threshold”, “reporting between thresholds”…), it is proposed to agree to explicit threshold configuration

Conclusion 6: It is FFS whether encoding of CAC will be according to the sender’s rules or whether it is according to the CAC defined in LTE, e.g. 36.413

Conclusion 7: Given the majority view, there is no support of PRB utilisation for inter system load balancing in the current release

Conclusion 8: Given the majority view, there is no support of per slice load information for inter system load balancing in the current release

Conclusion 9: Given the majority view, there is no support of Number of active UEs for inter system load balancing in the current release

Conclusion 10: It is FFS whether to support signalling of the Number of RRC connections for inter system load balancing in the current release

Conclusion 10: It is FFS whether to support signalling of the TNL Available Capacity for inter system load balancing in the current release

# 3	Discussion

## 3.1	Inter System Load Reporting Procedure

A number of companies agree to introduce a mechanism based on Inter System Resource Status Request/Response/Update that relies on modification of the Inter-System SON Information IE in TS38.413 and based on the introduction of a new IE over the S1 named Inter-System SON Information Transfer.

The messages to be used for transferring procedure messages for inter system load reporting are the S1: eNB CONFIGURATION TRANSFER, S1: MME CONFIGURATION TRANSFER, NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER.

Companies are invited to provide their view on introducing a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via modification of the Inter-System SON Information IE

| Company | Comment |
| --- | --- |
| Ericsson | Agree to the proposal |
| Huawei | Agree |
| Qualcomm | Agree |
| CMCC | Agree |
| Nokia | All right |
| ZTE | Agree |
| Samsung | Agree |

Summary of discussion after first round:

Conclusion 1: It is proposed to agree to introduce a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via modification of the Inter-System SON Information IE

Companies are invited to provide their view on introducing a new mechanism for Inter System Status Request/Response/Update over S1: eNB CONFIGURATION TRANSFER, S1: MME CONFIGURATION TRANSFER, via introduction of a new Inter-System SON Information Transfer IE

| Company | Comment |
| --- | --- |
| Ericsson | Agree to the proposal |
| Huawei | We can refer to the one in NGAP |
| Qualcomm | We think we don’t need to introduce a new IE in S1AP and instead can refer to the IE in NGAP. S1: eNB CONFIGURATION TRANSFER and S1: MME CONFIGURATION TRANSFER has Inter-system SON Configuration Transfer which is an OCTET STRING and refers to the IE in NGAP. The IE in NGAP can be sent in both directions i.e. from source eNB to target NG-RAN and vice versa. Therefore, we don’t need to introduce a new IE. |
| CMCC | Agree |
| Nokia | Indeed, reference to one specification is probably necessary to avoid re-encoding of the information in the CN. |
| ZTE | Similar view with Huawei and Nokia. |
| Samsung | Similar view with Huawei and Nokia. |

Summary of discussion after first round:

Conclusion 2: It is proposed to agree to introduce a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via reuse of the Inter-System SON Configuration Transfer IE

## 3.2	Inter System Event Based Load Reporting

Proposals have been submitted in the direction of reporting inter system load information periodically and on an event based manner.

While event based reporting has been proposed on the basis of thresholds configured in different possible ways, periodic reporting has been proposed with periodicity that is much reduced when compared to intra system load reporting (periods of (1000ms, 2000ms, 5000ms,10000ms, …)).

Are companies in favour of periodic inter system load reporting with periodicity not lower than 1000ms?

| Company | Comment |
| --- | --- |
| Ericsson | Agree to the proposal. With a sufficiently long periodicity, periodic reporting could be feasible and help having a continuous monitoring of load conditions. |
| Huawei | Agree |
| Qualcomm | Agree |
| CMCC | Agree to the proposal. |
| Nokia | Technically, this is all right. But shouldn’t CT WGs be consulted on the signalling load impact? |
| ZTE | Agree |
| Samsung | Agree |

Summary of discussion after first round:

Conclusion 3: It is proposed to agree to periodic inter system load reporting with periodicity not lower than 1000ms

FFS whether to send an LS to CT

Are companies in favour of threshold based load reporting? If yes, should thresholds be:

- Implicitly configured (e.g. a pre-set Number Of Measurement Reporting Levels) or

- Explicitly configured (e.g. Upper and Lower Thresholds based on CAC values and reporting criteria such as “reporting above threshold”, “reporting below threshold”, “reporting between thresholds”…)

| Company | Comment |
| --- | --- |
| Ericsson | We are in favour of threshold based Load Reporting. We support explicitly configured thresholds because they provide more precise configuration of the events triggers for load reporting |
| Huawei | We prefer to use explicitly configured, and to start by re-using existing solutions in LTE: number of thresholds. Enhancements could be discussed later. |
| Qualcomm | Yes, we do support threshold based load reporting. We don’t think we need to choose between implicitly configured vs. explicitly configured reporting Implicitly configured (e.g. Number of Measurement Report Levels) is supported for inter-RAT load balancing in E-UTRAN and U-TRAN. We can use a similar method here as well. This IE is also be configurable and can have multiple levels say up to 20 linear levels as event triggers Explicitly configured is also useful for finer control on event triggers and can be introduced here. |
| CMCC | Support threshold based load reporting, and both methods are useful. In addition, we support to adopt more load metrics besides CAC, therefore the definition of the threshold needs FFS. |
| Nokia | Probably explicit thresholds are more useful, as they allow to obtain reports at the moment that the initiating node exactly needs them. |
| ZTE | Fine with the threshold based load reporting, and the explicit threshold is more straightforward. |
| Samsung | We prefer explicit threshold. |

Summary of discussion after first round:

Conclusion 4: It is proposed to agree to introduce threshold based load reporting.

Conclusion 5: Given the majority of companies in favour of explicitly configured thresholds (e.g. Upper and Lower Thresholds based on CAC values and reporting criteria such as “reporting above threshold”, “reporting below threshold”, “reporting between thresholds”…), it is proposed to agree to explicit threshold configuration

## 3.3	Inter System Load Reporting Metrics

The majority of companies are in favour of reporting the Composite Available Capacity for inter system Load Reporting.

It is therefore proposed to report from E-UTRAN to NG-RAN the Composite Available Capacity Group IE as defined in TS 36.423 and from NG-RAN to E-UTRAN the Composite Available Capacity Group IE as defined in TS 38.423. If companies have a different understanding they can state their comments below:

| Company | Comment |
| --- | --- |
| Huawei | We prefer to only use the CAC defined in 36.423. The most critical part when designing this solution is to reduce the signalling impact. Although SSB reporting is supported on Xn, this is a completely different scenario. Reporting a single values seems like a simpler way to go and would also make the event triggered reporting work better. If we report CAC values per SSB, what is the benefit of setting events? It will hardly reduce the signalling? |
| CMCC | We agree to the proposal. In our opinion, the real bottleneck of the network is the number of concurrent transmitted messages, not the payload within each message, especially when the payload does not increase that much in size. In addition, what we need to consider at this stage is whether the introduction of a potential load metric is beneficial to help perform subsequent MLB operations. The event setting issue can be FFS then. |
| Nokia | We confirm support for the CAC. Regarding the definition, at this moment, we are more neutral regarding the encoding method: it should either be that encoding is related to the sender’s system (NR CAC from gNB, LTE CAC from eNB), or only LTE CAC from both nodes. |
| ZTE | Similar concern with Nokia, whether we should limit the same metric between the NG-RAN and E-UTRAN to be the same granularity? |
| Ericsson | We agree with the proposal |

Summary of discussion after first round:

Conclusion 6: It is FFS whether encoding of CAC will be according to the sender’s rules or whether it is according to the CAC defined in LTE, e.g. 36.413

Companies propose reporting of other metrics for inter system Load Balancing, which are listed below on a metric by metric basis.

Do companies support the reporting of PRB usage between E-UTRAN and NG RAN

| Company | Comment |
| --- | --- |
| Ericsson | Not feasible. It cannot be assumed that E-UTRAN nodes would support an understanding of the RB structure of NG-RAN nodes. By signalling PRB usage we force E-UTRAN nodes to be updated whenever new RB structures/numerologies are introduced in NG-RAN. The important information to acquire is the available capacity, which is used to determine if load based mobility can be triggered. PRB utilisation does not reflect available capacity, but only represents what PRBs are used, independently of whether they are used for overprovisioning or for minimum resource allocation. |
| Huawei | Similar view as E///. Further, we would need to consider whether this should trigger event based reporting and how. |
| Qualcomm | Similar view as E/// and Huawei. In general, if we introduce more load metrics, signalling for supporting event based load reporting would become more complex |
| CMCC | We support the reporting of PRB usage between E-UTRAN and NG-RAN. We do not think the update to E-UTRAN node brings too much extra effort, since every release may bring impact to E-UTRAN node, and the update to E-UTRAN is necessary. In addition, as an observation of our contribution, CAC cannot solve the interoperability issue. Compared to CAC, PRB usage is clear in definition, and brings benefit for the source node to choose potential target node. For the event based load reporting, we need to further discuss when and how to trigger event based reporting, after introducing more metrics for load reporting. |
| Nokia | How would it work in terms of triggering? Would there be a separate threshold for PRBs, or once the CAC threshold is reached, PRB usage would be appended? We are sceptical in both cases: if there are separate thresholds, the system will get overly complicated; otherwise, if PRB usage is just appended to the CAC report, it would be useless (provided more-or-less randomly). |
| Samsung | Same view with E///. |

Summary of discussion after first round:

Conclusion 7: Given the majority view, there is no support of PRB utilisation for inter system load balancing in the current release

Do companies support the reporting of per slice load information?

| Company | Comment |
| --- | --- |
| Ericsson | Not feasible. There is no concept of slicing in E-UTRAN, hence there would be no information to report from E-UTRAN to NG-RAN. Likewise, there is no way an eNB can use per slice load information because the eNB does not serve any slice. Hence, in a load based handover, the eNB would not know to which network slice the UE’s bearers will be mapped and for that the eNB will not be able to make use of any per slice load information received from the NG RAN. The eNB does not even know which slice a terget NG-RAN node supports. |
| Huawei | Similar view as E///. |
| Qualcomm | Same as above |
| CMCC | No strong preference. |
| Nokia | Indeed, due to limitations of LTE, this may be hardly usable. |
| ZTE | Maybe not needed |
| Samsung | Same view with E///. |

Summary of discussion after first round:

Conclusion 8: Given the majority view, there is no support of per slice load information for inter system load balancing in the current release

Do companies support the reporting of Number of active UEs between E-UTRAN and NG RAN

| Company | Comment |
| --- | --- |
| Ericsson | Not needed. In order to be understood and used, this metric requires full knowledge of the resource structure of the sending node, as well as knowledge of the resource utilisation. It is difficult to ensure such understanding between E-UTRAN and NG-RAN |
| Huawei | Agree with E///. Further, we would need to consider whether this should trigger event based reporting and how. |
| Qualcomm | Same as above |
| CMCC | Support to report. Number of active UEs is a useful complementary to PRB usage. For the event based load reporting, we need to further discuss when and how to trigger event based reporting, after introducing more metrics for load reporting. |
| Nokia | Same as in case of PRB usage: either useless (if appended randomly to CAC reports) or make the reporting system very complicated if separate thresholds are to be set. |
| ZTE | Considering one scenario that one eNB (SA) has two potential target gNBs (SA) for offloading, if the Number of active UEs can be transmitted from NG-RAN to E-UTRAN by inter-system load balancing, the eNB can have a better view to select the target gNB for offloading, i,e, the gNB with less number of active UEs. |
| Samsung | We don’t see the obvious benefit and we prefer to not include it. |

Summary of discussion after first round:

Conclusion 9: Given the majority view, there is no support of Number of active UEs for inter system load balancing in the current release

Do companies support the reporting of Number of RRC connections between E-UTRAN and NG RAN

| Company | Comment |
| --- | --- |
| Ericsson | It may be feasible to provide an understanding of CP load. However for the sake of simplicity and reduction of information signalled across systems we do not think it is essential. |
| Huawei | Similar as E// but we think also this one is difficult to use in this scenario (similar as above). Further, we would need to consider whether this should trigger event based reporting and how. |
| Qualcomm | Same as above |
| CMCC | Support to report RRC connections. It reflects the CP load. In our opinion, the real bottleneck of the network is the number of concurrent transmitted messages, not the payload within each message, especially when the payload does not increase that much in size. |
| Nokia | Same as in case of PRB usage: either useless (if appended randomly to CAC reports) or make the reporting system very complicated if separate thresholds are to be set. |
| ZTE | Same view as the case of Number of active UEs. |
| Samsung | We don’t see the obvious benefit and we prefer to not include it. |

Summary of discussion after first round:

Conclusion 10: It is FFS whether to support signalling of the Number of RRC connections for inter system load balancing in the current release

Do companies support the reporting of TNL Available Capacity between E-UTRAN and NG RAN

| Company | Comment |
| --- | --- |
| Ericsson | It may be feasible to provide an understanding of TNL load. However, this would require to update the concept of TNL Available Capacity in E-UTRAN and NG-RAN because these systems have different ways to define this parameter. In light of this we would promote a simpler solution and not include it. |
| Huawei | TNL is not so useful by itself. It should be used in combination with e.g. PRB. Hence we think it is better to not have it. Further, we would need to consider whether this should trigger event based reporting and how. |
| Qualcomm | Same as above |
| CMCC | TNL load can be considered. |
| Nokia | Same as in case of PRB usage: either useless (if appended randomly to CAC reports) or make the reporting system very complicated if separate thresholds are to be set. |
| ZTE | TNL load is a common metric for both LTE and NR, so this metric could be considered if we can define this parameter in a common way. |
| Samsung | We don’t see the obvious benefit and we prefer to not include it. |

Summary of discussion after first round:

Conclusion 11: It is FFS whether to support signalling of the TNL Available Capacity for inter system load balancing in the current release

# 4	Conclusion, Recommendations [if needed]

If needed

# 5	References

[1] R3-206185, Inter-System Mobility Load Balancing (Qualcomm Incorporated)

[2] R3-206195, (TP for SON BLCR for 38.413) Inter-System Load Balancing (Huawei)

[3] R3-206934, (TP for SON for TS 38.300, TS 38.413, TS 36.300, TS 36.413): Inter-System Load Balancing BL CR (Ericsson)

[4] R3-206678, Further Discussion on Inter-system Load Balancing in NR (ZTE)

[5] R3-206786, Further discussion on inter-system load balancing (CMCC)