3GPP TSG-RAN WG#110-e	R3-206947

# 2 - 12 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0129 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the configuration of subframe #0 and #5 for MCH in MBMS dedicated cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast-Core |  |  |  |  |  | Date: |  |  | 2020-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In dedicated MBMS cell, it should be possible to configure all subframes including subframe #0 and #5 as MBSFN subframe(s). However, (P)MCH cannot be scheduled in subframe #0 and #5. In particular with the introduction of 3ms MBMS slot length (i.e., 0.37kHz subcarrier spacing) in Rel-16, it is necessary that subframe #0 and #5 can be used for (P)MCH to improve network resource utilization as shown in figure below. Therefore, it is important that MBMS area configuration can schedule the (P)MCH in subframe#0 and #5. This is supported in RRC signalling after approval of CR#4259r2 (R2-2004429), so M2 needs to align. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new extension is added to the MBSFN Subframe Configuration IE, which enables subframes #0 and #5 to be scheduled in an MBSFN area. Impact analysis: This CR has isolated impact towards the previous version of the specification (same release). The impact can be considered isolated because the change only affects the scheduling functionality (MBSFN Subframe Configuration). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network cannot use subframe #0 and #5 to schedule (P)MCH, thereby limiting the resource utilization. Misalignment with RAN2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.17, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Resubmission to RAN3#109-e r2: Resubmission to RAN3#110-e r3: Corrections (ASN, RRC IE name, impact statement) |  |  |  |  |  |  |  |  |

#### 9.2.1.17	MBSFN Subframe Configuration

This IE indicates the MBSFN Subframe Configuration, as defined in TS 36.331 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Radio Frame Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32) | Semantics along the IE definition in TS 36.331 [11]. |
| Radio Frame Allocation Offset | M |  | INTEGER (0..7) | Semantics along the IE definition in TS 36.331 [11]. |
| CHOICE Subframe Allocation | M |  |  |  |
| >One Frame |  |  |  |  |
| >>One Frame Item | M |  | BIT STRING (SIZE(6)) | Semantics along the IE definition in TS 36.331 [11]. |
| >Four Frames |  |  |  |  |
| >>Four Frame Item | M |  | BIT STRING (SIZE(24)) | Semantics along the IE definition in TS 36.331 [11]. |
| CHOICE Subframe Allocation Extended | O |  |  |  |
| >One Frame Extension |  |  |  |  |
| >>One Frame Extension Item | M |  | BIT STRING (SIZE(2)) | Semantics along the definition of the MBSFN-SubframeConfig-v1430 IE as specified in TS 36.331 [11]. |
| >Four Frames Extension |  |  |  |  |
| >>Four Frame Extension Item | M |  | BIT STRING (SIZE(8)) | Semantics along the definition of the MBSFN-SubframeConfig-v1430 IE as specified in TS 36.331 [11]. |
| CHOICE Subframe Allocation Further Extension | O |  |  |  |
| >One Frame Further Extension |  |  |  |  |
| >>One Frame Further Extension Item | M |  | BIT STRING (SIZE(2)) | Semantics along the definition of the MBSFN-SubframeConfig-v1610 IE as specified in TS 36.331 [11]. |
| >Four Frames Further Extension |  |  |  |  |
| >>Four Frame Further Extension Item | M |  | BIT STRING (SIZE(8)) | Semantics along the definition of the MBSFN-SubframeConfig-v1610 IE as specified in TS 36.331 [11]. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

M2AP-Ies {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Ies (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MCH-Scheduling-PeriodExtended,

id-MCH-Scheduling-PeriodExtended2,

id-Modification-PeriodExtended,

id-Modulation-Coding-Scheme2,

id-Repetition-PeriodExtended,

id-Subcarrier-SpacingMBMS,

id-SubframeAllocationExtended,

id-SubframeAllocationFurtherExtension,

maxnoofMBSFNareas,

maxnoofPMCHsperMBSFNarea,

maxnoofCells,

maxnoofMBMSServiceAreasPerCell,

maxnoofSessionsPerPMCH,

maxnooferrors,

maxnoofCellsforMBMS

**  skip unchanged asn  **

MBSFN-SynchronisationArea-ID ::= INTEGER (0..65535)

MBSFN-Subframe-Configuration ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset		INTEGER (0..7),

subframeAllocation				CHOICE {

oneFrame						BIT STRING (SIZE (6)  ),

fourFrames						BIT STRING (SIZE (24) ) },

iE-Extensions					ProtocolExtensionContainer { { MBSFN-Subframe-Configuration-ExtIEs} } OPTIONAL,

...

}

MBSFN-Subframe-Configuration-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

{ID id-SubframeAllocationExtended		CRITICALITY reject	EXTENSION SubframeAllocationExtended	PRESENCE optional} |

{ID id-SubframeAllocationFurtherExtension	CRITICALITY reject	EXTENSION SubframeAllocationFurtherExtension		PRESENCE optional},

...

}

MCCH-Update-Time ::= INTEGER (0..255)

**  skip unchanged asn  **

#### -- S

SC-PTM-Information ::= SEQUENCE {

mbmsCellList					MBMS-Cell-List,

mbms-E-RAB-QoS-Parameters		MBMS-E-RAB-QoS-Parameters,

iE-Extensions					ProtocolExtensionContainer { {SC-PTM-Information-ExtIEs} } OPTIONAL,

...

}

SC-PTM-Information-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

SFN ::= INTEGER (0..1023)

Subcarrier-SpacingMBMS ::= ENUMERATED {khz-7dot5, khz-1dot25, ...}

SubframeAllocationExtended	::=	CHOICE {

oneFrameExtension				BIT STRING (SIZE(2)),

fourFrameExtension				BIT STRING (SIZE(8)),

choice-extension				ProtocolIE-Single-Container { { SubframeAllocationExtended-ExtIEs} },

...

}

SubframeAllocationExtended-ExtIEs M2AP-PROTOCOL-IES ::= {	...

}

SubframeAllocationFurtherExtension	::=	CHOICE {

oneFrameFurtherExtension				BIT STRING (SIZE(2)),

fourFrameFurtherExtension				BIT STRING (SIZE(8)),

choice-extension				ProtocolIE-Single-Container { { SubframeAllocationFurtherExtension-ExtIEs} },

...

}

SubframeAllocationFurtherExtension-ExtIEs M2AP-PROTOCOL-IES ::= {	...

}

#### -- T

NEXT CHANGE

### 9.3.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

**  skip unchanged asn  **

id-MCH-Scheduling-PeriodExtended2											ProtocolIE-ID ::= 48

id-Subcarrier-SpacingMBMS													ProtocolIE-ID ::= 49

id-SubframeAllocationExtended												ProtocolIE-ID ::= 50

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item 								ProtocolIE-ID ::= 51

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea									ProtocolIE-ID ::= 52

id-SubframeAllocationFurtherExtension										ProtocolIE-ID ::= XX