3GPP TSG-RAN WG3 #110-e	                                                                     R3-207041

2-12 November 2020

Online

Title:	Response to restricting the rate per UE per network slice

Response to:        R3-206840/S2-2007946

Release:	Release 17

Work Item:

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:	DAPENG LI

E-mail Address:	li.dapeng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 thanks SA2 share the progress on FS_eNS_Ph2.

RAN3 acknowledge the Key issue#3 and corresponding solutions (i.e. solution#22,37,43) and start initial analysis. From RAN3 point of view, Solution#22, Solution#37 and Solution#43 are all feasible from signalling perspective.

For Solution #37, RAN3 does not see the use of signaling the S-MBR to NG-RAN for UE-AMBR calculation. However sending the S-MBR is needed for the limitation of GBR unless the CN ensures that 5GC limits the GFBR of all QoS flows to S-MBR.

For solution#22, RAN3 acknowledges impact on admission control due to S-MBR apply not only for Non-GBR traffic but also for GBR traffic.

For solution#43, the solution may introduce additional notification signaling overhead over interfaces.

2. Actions:

RAN3 kindly asks SA2 and RAN2 to take above into account and feedback if needed.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #111-e		          25 Jan – 05 Feb 2021		  Online

TSG RAN WG3 Meeting #112-e		           17 May – 28 May 2020		  Online