3GPP TSG-RAN WG3 Meeting #110e	R3-207071

E-meeting, 2 – 12 Nov 2020

Title:	LS to SA2 on Immediate Suspension

Response to:	-

Release:	Rel-16

Work Item:	LTE_eMTC5-Core, NB_IOTenh3-Core

Source:	TSG RAN WG3

To:	SA2

Cc:	-

Contact Person:

Name:	Zijiang Ma

E-mail Address:	ma.zijiang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R3-207088, R3-206984

1. Overall Description:

In Rel-16, immediate Suspension has been supported in NGAP. That means if the Suspend Request Indication IE is included in the UE CONTEXT RESUME REQUEST message, the AMF shall, if supported, consider that the NG-RAN node is requesting immediate transition to RRC IDLE with Suspend as specified in TS 23.502. If the Suspend Response Indication IE is included in the UE CONTEXT RESUME RESPONSE message, the NG-RAN node shall suspend the UE context, the UE-associated logical NG-connection and the related PDU session contexts and transitions the UE to RRC_IDLE. Thus, there is not additional UE Context Suspend procedure during immediate Suspension case.

To support immediate Suspension procedure, RAN3 has agreed to introduce Information on Recommended Cells and RAN Nodes for Paging IE and Paging Assistance Data for CE Capable UE IE in NGAP: UE CONTEXT RESUME REQUEST message.

2. Actions:

To SA2 group.

ACTION: 	RAN3 respectfully asks SA2 to take the above information into account and update their related specification.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN3#111-e		February     2021

3GPP RAN3#112-e		May             2020