3GPP TSG-RAN WG3 #110-e	R3-207072

2-12 November 2020

Online

Agenda item:	15.2

Source: 	ZTE,Samsung

Title: 	TP for TR38.890 on addition management-based signalling for NR QoE

Document for:	Discussion and Approval

Introduction

This contribution provides the signaling of management-based solution for NR QoE in TR 38.890.

Text proposal for TR 38.890

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

6	Potential NR QoE solutions and procedures

Editor note: Description of potential NR QoE solutions and procedures, including but not limited to reuse Trace or MDT Functionality/Framework.

6.Z 	Management-based procedures(FFS)

6.z.x Management-based activation procedures

Note :The section concerns activation of measurement collection job.

The procedure is used for activating the QoE measurement configured and triggered by OAM shown in figure 6.z-1.

The OAM sends the QoE measurement configuration to NG-RAN node. NG-RAN finds a target individual UE(FFS) or qualified UEs that meet the criteria (e.g. application layer capability, service type, etc.). NG-RAN node sends the QoE measurement configuration to the AS layer of the UE or each UE. UE AS layer sends the QoE measurement configuration to UE application layer. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if meets, start QoE measurement and reporting.

UE application layer sends the QoE report to the UE AS layer. UE AS layer sends the QoE report to NG-RAN node. Then the NG-RAN node transmits the QoE report to the final destination configured (e.g. QoE server).

Management based QoE configuration should not override signalling based QoE configuration (FFS).

Figure 6.z-1

6.z.y Management-based deactivation procedures

Note: The section concerns deactivation of measurement collection job. Specific UE is not targeted (FFS).

The procedure is used for deactivating the QoE measurement triggered by OAM shown in figure 6.z-2. FFS the procedure used for deactivation the QoE measurement due to RAN overload.

OAM sends the deactivation indication to NG-RAN node to indicate which QoE measurement should be deactivated, then NG-RAN node sends it to UE AS layer, and UE AS layer sends it to the application layer in UE. The detail of deactive behavior for NG-RAN ,UE AS and UE application layer FFS.

The application layer in UE checks whether there is an ongoing session pertaining to the QMC (according to the SA4 requirements). The measurement can be stopped only if there is no such an ongoing session. (FFS)

Figure 6.z-2

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>