3GPP TSG-RAN3 Meeting #110-e	R3-207121

Electronic meeting, 2nd - 12th November 2020

Title:	[Draft] Reply LS on AS rekeying handling

Response to:	Reply LS (S2-2005909) on AS rekeying handling from SA2

Release:	Rel-15, Rel-16

Work Item:	5GS_Ph1, NR_NewRAT-Core

Source:	Huawei [to be RAN3]

To:	SA2

Cc:	SA3

Contact Person:

Name:	Yan Wang

Tel. Number:

E-mail Address:	wangyan7@Huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

RAN3 thanks SA2 and SA3 for reply LSs on AS rekey handing (S2-2005909, S3-201484).

About the following feedback:

SA3 S3-201484:

It is acceptable to SA3, for the network to give up the AS re-keying procedure and only initiate the emergency fallback procedure when the two procedures collide.

SA2 S2-2005909:

SA2 discussed the behaviour of NG-RAN node if it receives one NGAP UE CONTEXT MODIFICATION REQUEST message that includes both the Security Key IE and the Emergency Fallback Indicator IE.

From SA2 point of view, the AS re-keying procedure and the Emergency Fallback procedure are two different procedures and may collide with a very low probability. The Emergency Fallback shall have the highest priority and also require a strict service delay, so to guarantee the success of Emergency Fallback, the Emergency Fallback shall take precedence over AS rekeying if they collide.

Consequently, SA2 agrees with the feedback of SA3 to give up the AS re-keying procedure and only initiate the emergency feedback procedure when the two procedures collide.

When the AS re-keying procedure and the Emergency Fallback procedure collides, the AMF gives up the AS re-keying procedure and only initiates the emergency fallback procedure. If AMF includes both the Security Key IE and the Emergency Fallback Indicator IE within one NGAP UE CONTEXT MODIFICATION REQUEST message, NG-RAN takes it as abnormal case.

During RAN3 discussion, about how to handling the abnormal case in which both the Security Key IE and the Emergency Fallback Indicator IE are included in one NGAP UE CONTEXT MODIFICATION REQUEST message, there are two alternatives:

- Alternative 1: the NG-RAN node shall regard it as abnormal and fail the UE Context Modification procedure, a new cause may be introduced in the UE CONTEXT MODIFICATION FAILURE message.

- Alternative 2: the NG-RAN node shall regard it as abnormal and only perform emergency fallback, the NG-RAN node will feedback to AMF that as-rekey is failed in the UE CONTEXT MODIFICATION RESPONSE message.

RAN3 would like to ask SA2 on the preference of these two alternatives.

2. Actions:

To SA2 group:

ACTION: RAN3 respectfully asks SA2 to feedback the preference of the two alternatives on the abnormal case handling.

3. Date of Next TSG SA WG2 Meetings:

RAN3#111	25th January – 5th February 2021	Online