# 3GPP TSG-RAN WG3 #110-e						R3-207140

# 2 – 12 November 2020

# Title:	Reply LS on Latency of NR Positioning Protocols

# Response to:	Reply LS on Latency of NR Positioning Protocols (R3-207042/ R2-2010873)

# Release:	Release 17

# Work Item:	FS_NR_pos_enh

Source:	Huawei to be RAN3

To:	RAN2, RAN1

Cc:	SA2

Contact Person:

Name:	Philippe Reininger

Tel. Number:

E-mail Address:	Philippe dot Reininger at Huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 would like to thank RAN2 for the Reply LS on Latency of NR Positioning Protocols.

RAN3 noticed that RAN2 provided information on delays related to node processing and RAN3 interfaces ….

Considering the information provided and the “latencies include processing delays of the various involved nodes (UE, gNB, AMF, LMF) and signalling delays between nodes”, RAN3 would like to clarify that a suitable implementation associated with a performant backhaul type (for the ideal backhaul, the example of latency in TR 36.932 shows that the latency is less than 2.5 us), in an adequate network deployment can allow the processing time and interface delay between the RAN and Core network to be further reduced and reach the zero delay.

RAN3 also informs the propagation time in gNB should also consider the delay between the gNB-CU and the gNB-DU.

RAN3 kindly as then RAN2 and RAN1 to consider these corrections:

Table 1: 	Latency Components

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 0-3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 0-3 | AMF Processing |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB | 0-0.5 |  |
| TgNB-DU-gNB-CU | 0-10 |  |
| TgNB-AMF | 0--10 |  |
| TAMF-LMF | 1-10 |  |
| TAMF-GMLC | 3-10 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | Wait for RAN1 | PHY DL-PRS measurement time; best possible case |
| TUL-Meas | Wait for RAN1 | PHY UL-PRS measurement time; assume the same value as for DL-PRS. |

2. Actions:

To RAN WG2, WG1.

ACTION: 	RAN3 kindly asks RAN1 and RAN2 to consider the corrected table above.

3. Date of Next RAN3 Meetings:

RAN3#111-e	February 2021	Electronic meeting