3GPP TSG-RAN WG3 Meeting #110-e	R3-207157

E-meeting, 2 – 12 Nov 2020

Title: 	(TP for SON BL CR for TS 38.401): PCI selection in split architecture

Source: 	Huawei, Ericsson, NEC, ZTE, CMCC

Agenda item:	10.2.1.1

Document for:	Other

1. Introduction

This contribution contains the TP for SON BL CR for TS 38.401 for PCI selection function.

Annex – TP for SON BL CR for TS 38.401

7.x	PCI Optimisation Function

The PCI Optimization Function in non-split gNB case is specified in TS 38.300 [2].

For centralized PCI assignment in split gNB architecture, the gNB-CU detects PCI conflict of NR cells and reports the NR cells suffering PCI confilict to OAM directly. The OAM is in charge of reassigning a new PCI for the NR cell subject to PCI conflict.

For distributed PCI assignment in split gNB architecture, the OAM assigns a list of PCIs for each NR cell and sends the configured PCI list to the gNB-CU. If the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either during F1 Setup procedure or gNB-CU configuration update procedure.