3GPP TSG-RAN WG3 Meeting #110-e	R3-207187was R3-2-7119

2 – 12 Nov 2020

Title: 	TP on potential RAN3 impacts about the QoE measurement configuration, reporting and releasing under SA, NSA and MR-DC operation

Source: 	Huawei, China Unicom, Ericsson, Samsung, ZTE

Agenda item:	15.2

Document Type:	Discussion

1. Introduction

This TP to 38.890 tries to reflect the following agreements:

For QoE measurement configuration and Reporting

- RAN is not allowed to intervene, i.e. pause, activate or de-activate an ongoing QoE measurement collection, unless instructed otherwise by OAM; FFS whether RAN is allowed to release an ongoing QoE measurement reporting; liaise RAN2 for confirmation

- In case of RAN overload in standalone connectivity, RAN can stop new QoE measurement configurations, release existing QoE measurement configurations and pause QoE measurement reporting. FFS for the details under EN-DC/MR-DC operation.

- The RAN may be configured by the OAM with triggering conditions to control the start/stop of QoE measurements. The criteria could be e.g. event-, threshold- or time-based, details are FFS

- Signaling-based QoE measurement (Management-based QoE measurement: capture potential problems, if any, in TP to be agreed) could be configured in a certain area and towards an individual specific UE;

- FFS whether Multiple QoE measurements for a UE could be supported, this could be left RAN2 to decide

2. Reference

- RP-193256, New SID: Study on NR QoE management and optimizations for diverse services

- R3-206734, Discussions on potential RAN3 impacts about the QoE measurement configuration, reporting and releasing under SA, NSA and MR-DC operation	Huawei, China Unicom

3. Annex- TPs to be captured in TR

6.x.1 QoE measurement triggering and stopping

QoE measurement triggering and stopping can be realized using time-based and/or threshold-based criteria, configured by the OAM. Time-based QoE measurement triggering and stopping in NR is achieved by reusing mechanisms specified in LTE for the start and stop of QoE measurements. Threshold-based QoE measurement triggering and stopping allows to start and stop QoE measurement when given thresholds are passed.

Meanwhile, RAN is not allowed to intervene, i.e. pause, activate or de-activate an ongoing QoE measurement collection unless instructed otherwise by the OAM.

6.x.2 Release of QoE measurement configuration

An NG-RAN node can issue a release of QoE measurement configuration for UEs previously configured for QoE measurement reporting, provided that the session for which the QoE measurements are reported is completed.

Editor’s Note: FFS whether RAN is allowed to release an ongoing QoE measurement reporting; to be confirmed by RAN2.

6.x.3 QoE measurement handling at RAN overload

In case of RAN overload in standalone connectivity, RAN can stop new QoE measurement configurations, release existing QoE measurement configurations and pause QoE measurement reporting. FFS for details under EN-DC/MR-DC operation.