3GPP TSG-RAN3 Meeting #110-e	R3-207194

E-meeting, 2 - 12 November 2020

Title: 	Solutions and Flow charts of network slice service continuity

Source: 	Huawei

Agenda item:	17.2

Document Type:	Discussion and Decision

Annex 1– TP

6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

<Unchanged Text Omitted>

6.2.m Candidate solutions with/without CN involvement

Fig. 6.2.m.1: Slice re-mapping solutions: (a) with CN impact; (b) without CN impact

- This solution is applicable to scenario 2, where there are two possible slice re-mapping solutions depending on whether the CN is involved.

Fig. 6.2.m.1 (a) shows the re-mapping solution where both the RAN and CN parts are involved. In this case, the CN procedure is involved.

Fig. 6.2.m.1 (b) shows the re-mapping solution where the CN pat of the slice is not changed while the RAN part of the slice is remapped. The UL/DL traffics are relayed between the S-gNB and the T-gNB via the Xn tunnel.

Editor’s note: The handling of the UE at the target node needs to be clarified.

Editor’s note: Whether CN involvement is required, needs to be evaluated.

Editor’s note: Slice remapping needs to be defined in line with SA2 definitions.

6.2.Y Slice Re-mapping Message Sequence Charts

Editor’s note: Feasibility of this solution at system level requires further work including checking with SA2.

.

<Unchanged Text Omitted>

6.2.Y.4 Slice Remapping decision in 5GC and target gNB at NG based handover

Fig. 4: Slice re-mapping/fallback determined by the AMF and T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the initial slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing or re-mapped/fallback slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.1, the T-gNB shall include the further re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

Editor’s note: The efficiency of the solution needs to be further evaluated.

6.2.Y.5 Slice Remapping decision in SN for MR-DC case

Fig. 5: Slice re-mapping/fallback determined by the SN

This flow chart applies to the scenario of resource shortage only.

- The MN sends the SN Addition Request message to the SN.

- If the UE’s ongoing slice(s) is rejected by the SN, based on the slice re-mapping policy described in section 6.2.1, the SN makes the slice re-mapping/fallback decision. The SN shall include the slice re-mapping/fallback decision in the SN Addition Request Acknowledge message to the MN.

- The MN may send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.

6.2.Y.6 Slice Remapping decision in MN for MR-DC case

Fig. 6: Slice re-mapping/fallback determined by the MN

This flow chart applies to the scenario of resource shortage only.

- The MN makes the slice re-mapping/fallback decision and include the decision in the SN Addition Request message to the SN.

- The SN confirms the slice re-mapping/fallback decision made by the MN in the SN Addition Request Acknowledge message.

- The MN may send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.