3GPP TSG-RAN WG3 #110-e	R3-207201

2-12 November 2020

Online

Agenda item:	15.2

Source: 	ZTE,Samsung, Ericsson

Title: 	TP for TR38.890 on addition management-based signalling for NR QoE

Document for:	Discussion and Approval

Introduction

This contribution provides the signaling of management-based solution for NR QoE in TR 38.890.

Text proposal for TR 38.890

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

6	Potential NR QoE solutions and procedures

Editor note: Description of potential NR QoE solutions and procedures, including but not limited to reuse Trace or MDT Functionality/Framework.

6.Z 	Management-based procedures (FFS)

6.z.x Management-based activation procedures

Note: The section concerns activation of measurement collection job. It needs to be further checked if specific UEs can be targeted by the management based QoE measurement configuration.

The procedure is used for activating the QoE measurement configured and triggered by OAM shown in figure 6.z-1.

The OAM sends the QoE measurement configuration to NG-RAN node. NG-RAN finds multiple qualified UEs that meet the criteria (e.g. area scope, application layer capability, service type, etc.) or a single specific UE. NG-RAN node sends the QoE measurement configuration to the AS layer of the specific UE or each qualified UE. UE AS layer sends the QoE measurement configuration to UE application layer. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if the criteria are met, start QoE measurement and reporting.

FFS whether multiple QoE measurements could be configured for a UE at the same time.

UE application layer sends the QoE report to the UE AS layer. UE AS layer sends the QoE report to NG-RAN node. Then the NG-RAN node transmits the QoE report to the final destination configured (e.g. the MCE).

Note: the QoE report may also be visible at the NG-RAN node. (to be confirmed)

Management based QoE configuration should not override signalling based QoE configuration.(FFS)

Regarding Management-based configuration towards an individual specific UE the following needs to be considered:

- How to inform the OAM when specific UEs become RRC connected, and about served UEs per gNB. This is to enable the OAM of sending the QoE measurement configuration for a specific UE towards its serving gNB when the targeted UE becomes RRC connected;

- How to identify the connected UE in the gNB in a way visible to the OAM;

- Although it is acknowledged that QoE measurement configuration towards an individual specific UE is already ensured using signaling based configuration (via the CN, it should be noted that OAM can issue per-user policies for which the feedback in terms of user experience is required

Figure 6.z-1 Management-based NR QoE activation procedure

6.z.y Management-based deactivation procedures

Note: The section concerns deactivation of measurement collection job.

The procedure is used for deactivating the QoE measurement triggered by OAM shown in figure 6.z-2. FFS the procedure used for deactivation the QoE measurement due to RAN overload.

OAM sends the deactivation indication to NG-RAN node to indicate which QoE measurement should be deactivated,

The NG-RAN node checks whether there is an ongoing session pertaining to the QMC (according to the SA4 requirements). The QoE measurement configuration can be deactivated only if there is no such an ongoing session. If the NG-RAN node determines that the QoE measurement configuration is to be deactivated, then NG-RAN node sends the deactivation indication  to the UE AS layer, and UE AS layer sends it to the application layer in UE. The details are FFS.

Figure 6.z-2 Management-based NR QoE deactivation procedure

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>