3GPP TSG-RAN WG3 Meeting #110-e	R3-207217

# Online, November 2nd – 12th 2020

Agenda Item:	15.2

Source:	Ericsson

Title:	pCR for TR 38.890: Mobility Support for NR QoE Management

Document for:	Agreement

# 1	Introduction

The intention of this PCT is to capture the following agreements into TR 38.890:

Support mobility for QoE measurements in CONNECTED state, the QoE measurement configuration transfer is supported on the Xn and NG interfaces, inside the Trace Activation IE.

QoE measurement configuration for a UE is exchanged between network nodes to support keeping the QoE measurement configuration in INACTIVE state mobility.

Discuss the potential solutions fulfilling SA4 requirement that a QoE measurement for an ongoing session shall not be interrupted, even if the UE moves across area boundaries during the session (out of the area or intermittently in and out of the area).

Discuss whether, and under which conditions, the target node may decide the subsequent handling of management-based QoE configuration.

Discuss whether inter-RAT and/or inter-system mobility for QoE measurements should be supported.

All of the above applies at least to signaling-based activation; management-based activation to be further checked

# Annex: pCR to TR 38.890

-------------------------------------------Change 1-------------------------------------------

### 6.x NR QoE Support for Mobility

Seamless mobility is a key functionality in NR and its impacts should be measurable at the application layer. To enable measuring the impact of the mobility on the application and users’ QoE, it is required to support QoE measurements in mobility scenarios at least for signalling based QoE.

Editor's note: Management-based activation to be further checked.

In LTE, to support the QoE measurement in mobility scenarios, the QoE configuration is forwarded from the source eNB to the target eNB as part of Trace Activation IE over X2 interface. The same IE is sent over S1 interfaces for mobility scenarios when the X2 interface is not established between the source and target.

In NR, to support mobility for QoE measurements in CONNECTED state, the QoE measurement configuration transfer is supported on the Xn and NG interfaces, inside the Trace Activation IE. To support keeping QoE measurement configuration in INACTIVE state mobility, QoE measurement configuration for a UE can be fetched from the node hosting the UE Context.

In addition, the SA4 requirements for QoE measurements stipulate that the client shall check the QoE configuration only when a session starts. This means that the client shall continue the QoE measurements for an ongoing session even if the UE moves out of the configured area. The SA4 requirements are RAT-independent and shall therefore be applied to the mobility solution for QoE measurement in NR, as well.

NOTE: the solutions enabling the fulfilment of the SA4 QoE requirements are FFS.

NOTE: FFS whether inter-RAT and/or inter-system mobility for QoE measurements should be supported.

NOTE: FFS whether, and under which conditions, the target node may decide the subsequent handling of management based QoE configuration.

- -------------------------------------------End of changes-------------------------------------