3GPP TSG-RAN WG3 #111									      R3-210010

25th January – 4th February 2021 Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0530 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to 38.413_Addition of SON features enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR contains the enhancements to SON features |  |  |  |  |  |  |  |  |
| Summary of change: |  | <to be completed before submitting> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced SON features not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 9.3.1.10, 9.3.1.xx (new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR TS 36.300 CR TS 38.401 CR 0165 TS 38.423 CR 0517 TS 38.473 CR 0710 TS 36.413 CR 1800 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: resubmission with added text proposals agreed |  |  |  |  |  |  |  |  |

### First Change

### 8.8.2	Downlink RAN Configuration Transfer

#### 8.8.2.1	General

The purpose of the Downlink RAN Configuration Transfer procedure is to transfer RAN configuration information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Downlink RAN configuration transfer

The procedure is initiated with an DOWNLINK RAN CONFIGURATION TRANSFER message sent from the AMF to the NG-RAN node.

If the NG-RAN node receives, in the SON Configuration Transfer IE or in the EN-DC SON Configuration Transfer IE, the SON Information IE containing the SON Information Request IE, it may transfer back the requested information either towards the NG-RAN node indicated in the Source RAN Node ID IE of the SON Configuration Transfer IE or towards an eNB indicated in the Source eNB-ID IE of the EN-DC SON Configuration Transfer IE by initiating the Uplink RAN Configuration Transfer procedure.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the Xn TNL Configuration Info IE containing the Xn Extended Transport Layer Addresses IE, it may use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the SON Information IE containing the SON Information Reply IE including the Xn TNL Configuration Info IE as an answer to a former request, it may use it to initiate the Xn TNL establishment.

In case the IP-Sec Transport Layer Address IE is present and the GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE.

In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the Xn GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE.

In case the Xn GTP Transport Layer Addresses IE is empty and the IP-Sec Transport Layer Address IE is present, SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE, within the Xn Extended Transport Layer Addresses IE.

In case the Xn SCTP Transport Layer Addresses IE is present and the IP-Sec Transport Layer Address IE is also present, the concerned SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in this IP-Sec Transport Layer Address IE, within the Xn Extended Transport Layer Addresses IE.

If the NG-RAN node receives the SON Information IE containing the SON Information Report IE it may use it as specified in TS 38.300 [8].

If the NG-RAN node receives the Inter-system SON Information IE containing the Inter-system SON Information Report IE it may use it as specified in TS 38.300 [8].

If the NG-RAN node receives the Inter-system SON Information IE containing the Inter-system SON Information Request IE, it may use it as specified in TS 38.300 [8].

If the NG-RAN node is configured to use one IPsec tunnel for all NG and Xn traffic (IPsec star topology) then the traffic to the peer NG-RAN node shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

### Next Change

#### 9.3.3.34	Inter-system SON Information

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information | M |  |  |  |
| >Inter-system SON Information Report |  |  |  |  |
| >>Inter-system SON Information Report | M |  | 9.3.3.36 |  |
| >Inter-system SON Information Request |  |  |  |  |
| >>Inter-system SON Information Request | M |  | 9.3.3.xx |  |
| >Inter-system SON Information Reply |  |  |  |  |
| >>Inter-system SON Information Reply | M |  | 9.3.3.yy |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

#### 9.3.3.35	SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >Failure Indication Information |  |  |  |  |
| >>Failure Indication | M |  | 9.3.3.37 |  |
| >HO Report Information |  |  |  |  |
| >>HO Report | M |  | 9.3.3.39 |  |
| >Successful HO Report List |  | 0..1 |  |  |
| >> Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  |
| >>> Successful HO Report Container | O |  | OCTET STRING | FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. FFS |

Editor's Note: The ASN.1 changes for the above changes are not yet included

#### 9.3.3.36	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >>Inter-system HO Report | M |  | 9.3.3.40 |  |
| >Failure Indication Information |  |  |  |  |
| >>Inter-system Failure Indication | M |  | 9.3.3.38 |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication | M |  | 9.3.3.aa |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

### Next Change

#### 9.3.3.xx	Inter-system SON Information Request

This IE contains the request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Request | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Request | M |  | 9.3.3.zz |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

### Next Change

#### 9.3.3.yy	Inter-system SON Information Reply

This IE contains the reply information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Response | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Reply | M |  | 9.3.3.bb |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

### Next Change

#### 9.3.3.zz	Cell Activation Request

This IE contains request information for inter-RAT Cell Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activation ID | M |  | INTEGER (0..16384) | Allocated by the eNB. |
| Cells to Activate List |  | 1..<maxnoofCellsinNGRANNode> |  |  |
| >NG-RAN CGI | M |  | 9.3.1.73 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNGRANNode | Maximum no. of cells that can be served by a NG-RAN node. Value is 16384. |

Editor's Note: The ASN.1 changes for the above changes are not yet included

### Next Change

#### 9.3.3.aa	Cell State Indication

This IE contains notification information for inter-RAT Cell Activation and Deactivation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Cell List |  | 1..<maxnoofCellsinNGRANNode> |  |  |
| >NG-RAN CGI | M |  | 9.3.1.73 |  |
| >Notify Flag | M |  | ENUMERATED (Activated, Deactivated, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNGRANNode | Maximum no. of cells that can be served by a NG-RAN node. Value is 16384. |

Editor's Note: The ASN.1 changes for the above changes are not yet included

### Next Change

#### 9.3.3.bb	Cell Activation Reply

This IE contains reply information for inter-RAT Cell Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activated Cells List |  | 1..<maxnoofCellsinNGRANNode> |  |  |
| >NG-RAN CGI | M |  | 9.3.1.73 |  |
| Activation ID | M |  | INTEGER (0..16384) | Allocated by the eNB. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNGRANNode | Maximum no. of cells that can be served by a NG-RAN node. Value is 16384. |

Editor's Note: The ASN.1 changes for the above changes are not yet included

### End of Changes