3GPP TSG-RAN WG3 Meeting #111-e	R3-210011

# E-meeting, 25 Jan – 5 Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0710 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SON features enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR contains the enhancements to SON features |  |  |  |  |  |  |  |  |
| Summary of change: |  | <to be completed before submitting> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced SON features not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR TS 36.300 CR TS 38.401 CR 0165 TS 38.423 CR 0517 TS 38.413 CR 0530 TS 36.413 CR 1800 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: - include agreements from RAN3#110 found in R3-207158 and R3-207208 - editorial + added editorial notes for cases where ASN.1 is not yet available Rev1: Resubmission to RAN3#111 |  |  |  |  |  |  |  |  |

# 8	F1AP procedures

<<unchanged text omitted >>

## 8.11	Self Optimisation Support procedures

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

<<unchanged text omitted >>

# 9	Elements for F1AP Communication

<<unchanged text omitted >>

## 9.2	Message Functional Definition and Content

<<unchanged text omitted >>

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoof SuccessfulHOReports> |  |  |  | ignore |
| >>Successful HO Report Container | M |  | OCTET STRING | FFS on the definition | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is FFS |

Editor's Note: The ASN.1 changes for the above changes are not yet included

<<unchanged text omitted >>

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

<<unchanged text omitted >>

#### 9.3.1.140	NR PRACH Configuration List

This IE indicates the PRACH resources used or reserved in the UL carrier(s) or SUL carrier(s) of the current NR cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR PRACH Configuration Item |  | 0..< maxnoofPrachConfiguration > |  | Length=0 means releasing of all NR PRACH Configuration Items for this UL or SUL. |
| >NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | The SCS of the carrier to which this PRACH Configuration Item relates, i.e. Δf in Section 5.3.2 in TS 38.211 [33]. The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [17]. NOTE: Its value may not be identical to the SCS of PRACH. |
| > PRACH Frequency Start from Carrier | M |  | INTEGER (0.. maxNrofPhysicalResourceBlocks-1, …) | Lowest number of resource blocks which can be used to deliver MSG1 or the preamble part of MSGA, counting from the start number of the corresponding carrier. Identical to in Section 5.1.2.2.2 in TS 38.214 [34] plus msg1-FrequencyStart or msgA-RO-FrequencyStart-r16 in TS 38.331 [8]. |
| > PRACH-FDM | M |  | ENUMERATED (one, two, four, eight, …) | M in Section 6.3.3.2 in TS 38.211 [33]. |
| >PRACH Configuration Index | M |  | INTEGER (0.. 255, …) | See Section 6.3.3.2 in TS 38.211 [33]. |
| >SSB per RACH Occasion | M |  | ENUMERATED (oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen, ...) | Number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. |
| >CHOICE FreqDomainLength | M |  |  | For the case of PRACH resources reserved for BFR or MSG1-based SI Request, L139 is always used. |
| >>L839 |  |  |  |  |
| >>>L839 Info |  | 1 |  |  |
| >>>>Root Sequence Index | M |  | INTEGER (0..837) | See Section 6.3.3.1 in TS 38.211 [33]. |
| >>>>Restricted Set Config | M |  | ENUMERATED (unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, …) | See Section 6.3.3.1 in TS 38.211 [33]. |
| >>L139 |  |  |  |  |
| >>>L139 Info |  | 1 |  |  |
| >>>> PRACH SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | Subcarrier Spacing of PRACH, i.e. ΔfRA in Section 5.3.2 in TS 38.211 [33]. |
| >>>>Root Sequence Index | M |  | INTEGER (0..137) | See Section 6.3.3.1 in TS 38.211 [33]. |
| >Zero Correlation Zone Config | M |  | INTEGER (0..15) | See Section 6.3.3.1 in TS 38.211 [33]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPhysicalResourceBlocks-1 | Maximum no. of Physical Resource Blocks minus 1. Value is 274. |
| maxnoofPrachConfiguration | Maximum no. of PRACH Configuration. Value is 16. |

<<unchanged text omitted >>

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

<<unchanged text omitted >>

### 9.4.5	Information Element Definitions

<<unchanged text omitted >>

L139Info ::= SEQUENCE {

prachSCS						ENUMERATED {scs15, scs30, scs60, scs120, ...},

rootSequenceIndex			INTEGER (0..137)								OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {L139Info-ExtIEs} } 		OPTIONAL,

...

}

L139Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<unchanged text omitted >>

NRPRACHConfigItem ::= SEQUENCE {

nRSCS					NRSCS,

prachFreqStartfromCarrier	INTEGER (0..maxnoofPhysicalResourceBlocks-1, ...),

prachFDM						ENUMERATED {one, two, four, eight, ...},

parchConfigIndex			INTEGER (0..255, ...),

ssb-perRACH-Occasion		ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen, ...},

freqDomainLength			FreqDomainLength,

zeroCorrelZoneConfig		INTEGER (0..15),

iE-Extension		ProtocolExtensionContainer { { NRPRACHConfigItem-ExtIEs} } 		OPTIONAL,

...

}

NRPRACHConfigItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}