3GPP TSG-RAN WG3 Meeting #111-e 	R3-210077

# E-meeting, -, 25.01 - 05.02.2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definitions of SCG failure events for MRO for SN change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 11.01.2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN3 works on MRO for PSCell change. In order to define proper solutions, the definitions of the events that the MRO is to address are needed to be standardised. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new sub-clause is added to describe MRO in MR-DC scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be hard to anchor the solutions, if definitions are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

First change, ommited text not changed

## 10.A	Support for Self-Organised Networks

### 10.A.1	General

Support for self-organised networks is described in subclause 22.4 in TS 36.300 [2] and in subclause 15.5 in TS 38.300 [3].

### 10.A.2	MRO for PSCell change

One of the functions of MRO for PSCell change is to detect SCG failures that occur due to Too late SN change triggering or Too early SN change triggering, or Triggering SN change to wrong SN. These problems are defined as follows:

-	[Too late SN change triggering] an SCG failure occurs after the UE has stayed for a long period of time in a PSCell of the SN; the measurements reported in the SCG failure report from the UE indicate another PSCell in the same or a different SN as the best candidate for SCG connection.

-	[Too early SN change triggering] an SCG failure occurs shortly after a successful PSCell change or an SCG failure occurs during the PSCell change procedure (intra-SN or inter-SN); the measurements reported in the SCG failure report from the UE indicate the same PSCell that the UE used prior to the failure as the best candidate for SCG connection.

-	[Triggering SN change to wrong SN] an SCG failure occurs shortly after a successful PSCell change or an SCG failure occurs during the PSCell change procedure (intra-SN or inter-SN); the measurements reported in the SCG failure report from the UE indicate a PSCell different than the source PSCell or the target PSCell, in the same or a different SN, as the best candidate for SCG connection.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

Remaining text not changed