3GPP TSG RAN WG3#111-e	R3-210111

25th January-5th February, 2021

Online

Title: 	(TP for SON BLCR 38.473) Introducing UE Assistant Identifier in F1AP for Successful HO Report

Source: 	China Telecom

Agenda item:	10.2.1.3

Document for:	Approval

Introduction

In this document we discussed the Sucessful Handover Report related information in F1AP and give our proposal and text proposal associated to TS38.473.

Discussion

In the last RAN3#110e meeting, we discussed the Sucessful Handover Report in the CB: # 1003_SONMDT_SuccessHO, and it has been agreed to reuse ACCESS AND MOBILITY INDICATION message to transmit Successful Handover report over F1 interface.

In this document, we provide the TP on introducing the UE Assistant Identifier IE in F1AP for Successful HO Report, currently the Successful Handover Report is defined as a list, the NG-RAN node can transmit several UEs’ Successful HO Report to its peer node, including the UE Assistant Identifier IE (which in fact is the gNB-DU UE F1AP ID) for each Successful Handover Report in F1AP can help the DU to identify the successful handover report belongs to which UE, identifying this information is beneficial for the gNB-DU to optimize the handover related parameters. Therefore, we propose to include the UE Assistant Identifier IE in F1AP for each Successful HO Report.

Proposal 1: Include the UE Assistant Identifier IE in F1AP for each Successful HO Report.

Text Proposal

Following changes depend on the agreed TP[R3-207208].

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoof SuccessfulHOReports> |  |  |  | ignore |
| >>Successful HO Report Container | M |  | OCTET STRING | FFS on the definition | YES | ignore |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64.FFS |

/////////////////////////////////////////////////////////////////////////change end////////////////////////////////////////////////////////////////////////////////////

Reference

- TS38.473, F1AP, (Release 16)

- TS38.401, Architecture description, (Release 16)

- R3-205510, Summary of Offline Discussion on Successful Handover Report, Samsung, 2020-08.

- R3-206879, Summary of Offline Discussion on Successful Handover Report, Samsung (moderator), 2020.11.

- R3-207208, (TP for SON BLCR 38.473) Introducing the successful HO report in F1AP, China Telecom.