3GPP TSG-RAN WG3 Meeting #111-e	R3-210137

# E-meeting, 25 Jan – 5 Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | DraftCR | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Activation and de-activation of SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN3 has agreed the procedures of deactivation/re-activation of SCG in SN addition procedure and in the SN modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of deactivation/re-activation of SCG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network does not support the deactivation/re-activation of SCG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.xx(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.401 CR 0160 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.xx	Deactivation and re-activation of SCG

In MR-DC with NR SCG, the network can deactivate the SCG in the SN addition procedure and in the SN modification procedure.

Figure 10.xx-1: Support of deactivation/re-activation of SCG in the SN addition procedure

1.	The MN decides to deactivate the NR SCG before adding the NR SCG.

2.	The MN triggers the SN addition procedure and informs the SN to deactivate the SCG.

3.	The SN replied in the SN addition request acknowledge that the SCG is added and deactivated.

4.	The MN informs the UE to deactivate the SCG [FFS, pending to RAN2].

5/6.	The MN receives the assistance information from the SN and the UE on the user traffic activity of the SCG.

7.	The MN decides to re-activate the SCG.

8.	The MN triggers the SN modification procedure and informs the SN to activate the SCG.

9.	The SN replied in the SN modification request acknowledge that the SCG is activated.

10.	The MN informs the UE to activate the SCG [FFS, pending to RAN2].

Figure 10.xx-2: Support of deactivation/re-activation of SCG in the SN modification procedure

0. The SN notifies the MN about user data inactivity.

1.	The MN decides to deactivate the NR SCG.

2.	The MN triggers the SN modification procedure and informs the SN to deactivate the SCG.

3.	The SN replied in the SN modification request acknowledge that the SCG is deactivated.

4.	The MN informs the UE to deactivate the SCG [FFS, pending to RAN2].

5/6.	The MN receives the assistance information from the SN and the UE on the user traffic activity of the SCG.

7.	The MN decides to re-activate the SCG.

8.	The MN triggers the SN modification procedure and informs the SN to re-activate the SCG.

9.	The SN replied in the SN modification request acknowledge that the SCG is activated.

10.	The MN informs the UE to re-activate the SCG [FFS, pending to RAN2].