3GPP TSG-RAN WG3 Meeting #111-e	R3-210141

E-meeting, 25 Jan – 5 Feb 2021

Title:	[Draft] Reply LS on small data transmission

Response to:	LS R2-2010839 on LS to RAN3 on small data transmission from RAN2

Release:	Release 17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Huawei [to be TSG RAN WG3]

To:	TSG RAN WG2

Cc:

Contact Person:

Name:	Yan Wang

E-mail Address:	wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 would like to thank RAN2 for providing information relating the agreements on small data transmission. Based on the RAN2 agreements, RAN3 has discussed the topic would like to feedback the following RAN3 agreements:

RACH based SDT:

- Context fetch and data forwarding with anchor re-location and without anchor re-location will be supported;

- The last serving gNB makes the decision on with or without anchor relocation, by taking the assistance information from the new gNB into account, the detailed assistance information is FFS;

- New gNB processes the RLC PDU for the SDT DRB based on stored RLC configuration.

- In case of RACH based SDT without anchor relocation, the last serving gNB provides the stored RLC configuration to the new gNB, and the UL/DL PDCP PDU(s) will be forwarded via DRB level data forwarding tunnel(s).

CG based SDT:

- The gNB-CU gets the CG configuration from gNB-DU before configuring the UE to INACTIVE state.

- the gNB-DU maintains the UE Context (including CG configuration, RLC configuration) for the inactive mode UEs configured with SDT CG resources.

2. Actions:

To RAN2

ACTION: 	RAN3 respectfully asks RAN2 to take the above into account for small data transmission.

3. Date of Next TSG-RAN WG3 Meetings:

3GPP RAN3#112e		17 May – 28 May			Electronic Meeting