3GPP TSG-RAN WG3 Meeting #111 e                  	R3-210174

E-Meeting, 25th Jan– 4th Feb, 2021

Source:	vivo

Title:	Signaling support for paging cause

Agenda Item:	30

Document for:	Discussion and Decision

# Introduction

One of the objective of support for Multi-SIM devices  [1] is:

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR. [ RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

SA2 has approved a new WID [2] with following objective:

The objective is to specify enhancements to 5GS and EPS functionality and procedures for the following aspects: - A single paging cause with the meaning of “voice” for both EPS and 5GS according to the conclusions in TR 23.761 clause 8.1. - NAS Busy Indication for both EPS and 5GS in RRC_IDLE mode according to the conclusions in TR 23.761 clause 8.1. - Enabling paging reception for EPS according to the conclusions in TR 23.761 clause 8.2. Editor's note: The objective on enabling paging reception for EPS and the corresponding solution needs to be confirmed by RAN plenary. - NAS-level leaving procedure for EPS according to the conclusions in TR 23.761 clause 8.3. - Leaving procedure for 5GS. NOTE 1: The decision to support NAS-based leaving or RRC-based leaving or both will be determined taking into account feedback from RAN2. NOTE 2: The WID objectives will be further updated based on progress of work in other WGs and on open issues identified in the SA2. Work will be done in collaboration with RAN2, RAN3 and SA3 WGs.

At RAN2#112-e meeting, RAN2 has begun discussion on the feasibility of paging cause with following agreements:

Use: "Extending paging signalling is possible but RAN2 haven’t decided on overall feasibility of paging cause, including how it should be supported."

At RAN3#110-e meeting, RAN3 has also begun discussion on the feasibility of paging cause with following agreements:

A1: From RAN3 point of view, it is feasible to include paging cause over network interfaces, assuming that the size of paging cause is limited. The final decision about whether to introduce paging cause can be decided by other groups.

This contribution discusses paging cause signaling over network interfaces and proposes some baseline CRs for the signaling support.

# Discussion

## Paging Cause

Paging cause solution is described in 3GPP TR 23.761. The main procedures are highlighted in below box as follows:

#### 6.1.3.1 Handling of MT service with Paging Cause for UE in CM_Idle in 5GS The solution has impact on the Network Triggered Service Request procedure in TS 23.502 [6] clause 4.2.3.3. The changes relative to the existing procedure are indicated in bold underlined text. Only the impacted steps are shown. Figure 6.1.3.1-1: Network Triggered Service Request (based on TS 23.502 [6] Figure 4.2.3.3-1) ------- #### 6.1.3.2 Handling of MT service with Paging Cause in RRC_Inactive mode Figure 6.1.3.2-1is the call flow of handling of MT service with Paging Cause in RRC_Inactive mode. Figure 6.1.3.2-1 Handling of MT service with Paging Cause in RRC_Inactive mode 1. NG-RAN receives the DL data (control plane data and/or user plane data) in RRC_Inactive mode. If handling of MT service with Paging Cause is supported by NG-RAN, NG-RAN determines the Paging Cause based on the Paging Cause field included in the CN tunnel header of an incoming DL PDU. Alternatively, the NG-RAN determines the Paging Cause based on specific 5QI and ARP of the QoS flows for the downlink data packet and the corresponding PPI in the CN tunnel header. NG-RAN sends the paging message with the Paging Cause. #### 6.1.3.3 Handling of MT service with Paging Cause in EPS Figure 6.1.3.3-1 is handling of MT service with Paging Cause in EPS. Figure 6.1.3.3-1: Handling of MT service with Paging Cause in EPS

## Paging Cause for UE in CM_Idle in 5GS

From the main procedure, for the paging cause to be sent to UE, the AMF may send a Paging message to NG-RAN node(s) via 3GPP access including the Paging Cause provided by the SMF. Thus, from the procedure, the paging needs to be signaled from AMF to NG-RAN node over NG.

- To support paging cause in 5GS, the paging cause needs to be signalled between AMF and NG-RAN node over NG.

In 3GPP TS 38413, the paging message is used to page UE by sending it from the AMF to gNB. By adding a paging cause, the paging message can carry the paging cause. Thus, the PAGING message should be modified to include the paging casue as shown in below example:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.3.1.138 |  | YES | ignore |
| NB-IoT Paging DRX | O |  | 9.3.1.139 | If this IE is present, the Paging DRX IE is ignored. | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| WUS Assistance Information | O |  | 9.3.1.143 |  | YES | ignore |
| Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| Paging Cause | O |  |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

Therefore,

- It is proposed to add a new IE “Paging Cause” in the NG PAGING message, at least for voice support.

## Paging cause in RRC_Inactive mode

As the procedure shows, as for RRC idle in 5GS, the paging cause would be sent by AMF to NG-RAN node which determines the paging cause based on the Paging Cause field included in the CN tunnel header of an incoming DL PDU. The NG-RAN node then performs the RAN paging.

Additionally, as described in 3GPP TS38423, the RAN PAGING message can also be propagated among neighboring NG-RAN nodes by means of Xn RAN PAGING signaling. Thus, in case of RAN paging, if paging cause is supported, the paging cause needs to be added in the RAN PAGING message and signaled over Xn, as the below example shows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 |  | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Paging Cause | O |  |  |  | YES | ignore |

Therefore,

- In case of RAN paging, paging message can support paging cause and the paging cause can be included in RAN PAGING message.

Therefore,

- It is proposed to add a new IE “Paging Cause” in the Xn PAGING message, at least for voice support.

For paging in 5GS (idle or inactive paging), in case of CU/DU split RAN node, the paging message in F1/W1 needs also to contain the paging cause

- In case CU/DU split RAN node, the paging cause is also included in paging message over F1/W1

Therefore,

- It is proposed to add a new IE “Paging Cause” in the F1/W1 PAGING message, at least for voice support.

## Paging Cause for UE in EPS

The procedure of paging cause for UE in EPS shows that:

If the handling of MT service with Paging Cause is supported by Serving GW, Serving GW determines the Paging Cause and includes the Paging Cause in the DDN message sent from SGW to MME. If the Paging Cause is not received, but the Paging Policy Indication is received, the MME determines the Paging Cause.

For mobile terminating signaling and SMS over NAS, the MME determines an appropriate Paging Cause.

MME sends S1 paging message by including the Paging Cause information.

RAN sends the paging message with Paging Cause.

Thus,

- To support paging cause in EPS, the paging cause needs to be signalled between MME and eNB over S1.

In 3GPP TS 36413, the paging message is used to page UE by sending it by the MME to eNB. By adding a paging cause, the paging message can carry the paging cause. Thus, the PAGING message should be modified to include the paging cause as shown in below example:

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.2.3.10 |  | YES | ignore |
| UE Paging Identity | M |  | 9.2.3.13 |  | YES | ignore |
| Paging DRX | O |  | 9.2.1.16 | If the NB-IoT Paging DRX IE is included, the Paging DRX IE is ignored. | YES | ignore |
| CN Domain | M |  | 9.2.3.22 |  | YES | ignore |
| List of TAIs |  | 1 |  |  | YES | ignore |
| >TAI List Item |  | 1 .. <maxnoofTAIs> |  |  | EACH | ignore |
| >>TAI | M |  | 9.2.3.16 |  | - |  |
| CSG Id List |  | 0..1 |  |  | GLOBAL | ignore |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Paging Priority | O |  | 9.2.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.2.1.103 |  | YES | ignore |
| Paging eDRX Information | O |  | 9.2.1.111 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.46 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.2.1.115 |  | YES | ignore |
| NB-IoT UE Identity Index value | O |  | 9.2.3.47 |  | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| CE-Mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Data size | O |  | INTEGER (1..4095, …) | The unit is: bit | YES | ignore |
| WUS Assistance Information | O |  | 9.2.1.158 |  | YES | ignore |
| NB-IoT Paging DRX | O |  | 9.2.1.159x |  | YES | ignore |
| Paging Cause | O |  |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIs | Maximum no. of TAIs. Value is 256. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |

Therefore,

- It is proposed to add a new IE “Paging Cause” in the S1 PAGING message, at least for voice support.

- Adopt the CRs in [3~7] as BL CRs

# Conclusion

This paper discussed signaling support for paging cause. The paper concludes with:

- To support paging cause in 5GS, the paging cause needs to be signalled between AMF and NG-RAN node over NG.

- Addition of paging cause in PAGING message over NG is feasible.

- In case of RAN paging, paging message can support paging cause and the paging cause can be included in RAN PAGING message.

- In case CU/DU split RAN node, the paging cause is also included in paging message over F1/W1

- To support paging cause in EPS, the paging cause needs to be signalled between MME and eNB over S1.

- It is proposed to add a new IE “Paging Cause” in the NG PAGING message, at least for voice support.

- It is proposed to add a new IE “Paging Cause” in the Xn PAGING message, at least for voice support.

- It is proposed to add a new IE “Paging Cause” in the F1/W1 PAGING message, at least for voice support.

- It is proposed to add a new IE “Paging Cause” in the S1 PAGING message, at least for voice support.

- Adopt the draft CRs in [3~7] as BL CRs

# References

[1] RP-202895  Revised WID Support for Multi-SIM devices.

[2] S2-2009247 New WID on system enablers for Multi-USIM devices

[3] R3-210175 BLCR36413 for Multi-USIM devices support, vivo

[4] R3-210176 BLCR37473 for Multi-USIM devices support, vivo

[5] R3-210177 BLCR38413 for Multi-USIM devices support, vivo

[6] R3-210178 BLCR38423 for Multi-USIM devices support, vivo

[7] R3-210179 BLCR38473 for Multi-USIM devices support, vivo