3GPP TSG-RAN WG3 Meeting #111-e	R3-210196

# Online, January 25th – February 5th 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0528 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR38423 for addition of I-RNTI structure indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current non-standardized structure of I-RNTI, there still exists ambiguity issue when new NG-RAN node identifies the old NG-RAN node based on the received I-RNTI, especially for the NG-RAN nodes belonging to different vendors. Some solutions have been proposed, e.g., to introduce short NG-RAN node ID and make a NG-RAN Node aware of all the possible I-RNTI structures (the possible short NG-RAN Node ID lengths) in all the neighboring NG-RAN Nodes. Even this is the case, as the new NG-RAN Node has no exact idea about the real length of NG-RAN Node ID part in I-RNTI, the ambiguity issue for new NG-RAN Node to identify old NG-RAN Node and also the ambiguity issue for old NG-RAN Node to identify UE context still cannot be completely addressed. Therefore, a thorough way to explicitly indicate the I-RNTI structure is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To separate 3bits in I-RNTI for using as I-RNTI structure indication. The value of these 3bits is used for indicating the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ambiguity issue for new NG-RAN Node to identify old NG-RAN Node and also the ambiguity issue for old NG-RAN Node to identify UE context still exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.40, 9.2.3.46 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of first change>>

  - 9.2.3.40	UE Context ID

This IE is used to address a UE Context within an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE Context ID | M |  |  |  |
| >RRC Resume |  |  |  |  |
| >>I-RNTI | M |  | 9.2.3.46 |  |
| >>Allocated C-RNTI | M |  | BIT STRING (SIZE (16)) | Temporary C-RNTI allocated to the UE by the cell where the RRC connection has been requested to be resumed, contained in the MAC RAR as defined in TS 38.321 [35] or in TS 36.321 [36]. |
| >>Access PCI | M |  | NG-RAN Cell PCI 9.2.2.10 | The cell PCI where the RRC connection has been requested to be resumed. |
| >RRC Reestablishment |  |  |  |  |
| >>C-RNTI | M |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRCReestablishmentRequest message (TS 38.331 [10]) or RRCConnectionReestablishmentRequest message (TS 36.331 [14]). |
| >>Failure Cell PCI | M |  | NG-RAN Cell PCI 9.2.2.10 |  |

<<End of first change>>

<<Start of second change>>

  - 9.2.3.46	I-RNTI

The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node. The I-RNTI is partitioned into two parts, the first part identifies the NG-RAN node that allocated the I-RNTI and the second part identifies the UE context stored in this NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE I-RNTI |  |  |  |  |
| >I-RNTI full |  |  |  |  |
| >>I-RNTI full | M |  | BIT STRING (SIZE (40)) | This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 40 bits (refer to I-RNTI-Value IE in TS 38.331 [10] and I-RNTI IE in TS 36.331 [14]). The most significant 3bits indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, e.g., xbits (x=8+2*value of the 3bits). In the remaining bits of I-RNTI, the most significant xbits identify the NG-RAN Node ID part of this NG-RAN Node and the least significant (40-3-x) bits identify the UE Context stored at this NG-RAN Node. |
| >I-RNTI short |  |  |  |  |
| >>I-RNTI short | M |  | BIT STRING (SIZE (24)) | This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 24 bits (refer to ShortI-RNTI-Value IE in TS 38.331 [10] and ShortI-RNTI IE in TS 36.331 [14]). The most significant 3bits indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, e.g., xbits (x=8+2*value of the 3bits). In the remaining bits of I-RNTI, the most significant xbits identify the NG-RAN Node ID part of this NG-RAN Node and the least significant (24-3-x) bits identify the UE Context stored at this NG-RAN Node. |

<<End of second change>>