3GPP TSG-RAN WG3 #111-e																		R3-210202

Online, 25 January – 4 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0712 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.473 on propagation delay compensation enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the gNB-CU and gNB-DU split case, propagation delay compensation (e.g. DLInformationTransfer message encoding) is performed in gNB-CU and the propagation delay information is maintained in gNB-DU, the gNB-DU need to report the propagation delay compensation information to gNB-CU based on the gNB-CU controlling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the Propagation Delay Compensation Information Reporting Control and the Propagation Delay Compensation Information Reporting procedure. - To introduce the definition of the Propagation Delay Compensation Information Report and the Propagation Delay Compensation Information Reporting message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the gNB-CU cannot obtain the propagation delay compensation information, the gNB will not be able to compensate the propagation delay of the reference clock. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.XX, 9.2.YY, 9.3.1.ZZZ, 9.3.1.AAA, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.XX	Propagation Delay Compensation Information Reporting procedures

8.XX.1	Propagation Delay Compensation Information Reporting Control

8.XX.1.1	General

The purpose of the Propagation Delay Compensation Information Reporting Control procedure is to command the gNB-DU to send the requested propagation delay compensation information to the gNB-CU. The procedure uses UE associated signalling.

8.XX.1.2	Successful Operation

Figure 8.XX.1.2-1: Propagation Delay Compensation Information Reporting Control

The gNB-CU initiates the procedure by sending PROPAGATION DELAY COMPENSATION INFORMATION REPORTING CONTROL message to the gNB-DU. Upon reception of the PROPAGATION DELAY COMPENSATION INFORMATION REPORTING CONTROL message, the gNB-DU shall, if supported, perform the requested propagation delay compensation information reporting action.

The Propagation Delay Compensation Request Type IE indicates to the gNB-DU whether:

-	to report on demand;

-	to report periodic, with a frequency as specified by the Propagation Delay Compensation Report Periodicity IE;

-	to stop periodic reporting.

8.XX.1.3	Abnormal Conditions

Not applicable.

8.XX.2	Propagation Delay Compensation Information Report

8.XX.2.1	General

The purpose of the Propagation Delay Compensation Information Report procedure is to report the propagation delay compensation information from the gNB-DU to the gNB-CU. The procedure uses UE associated signalling.

8.XX.2.2	Successful Operation

Figure 8.XX.2-2-1: Propagation Delay Compensation Information Report

The gNB-DU initiates the procedure by sending a PROPAGATION DELAY COMPENSATION INFORMATION REPORT message to the gNB-CU. The PROPAGATION DELAY COMPENSATION INFORMATION REPORT message may be used as a response to the PROPAGATION DELAY COMPENSATION INFORMATION REPORTING CONTROL message.

8.XX.2.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.2.YY	Propagation Delay Compensation Information Reporting messages

9.2.YY.1	PROPAGATION DELAY COMPENSATION INFORMATION REPORTING CONTROL

This message is sent by the gNB-CU and is used to request the gNB-DU to deliver the propagation delay compensation information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Propagation delay compensation Request Type | M |  | 9.3.1.ZZZ |  | YES | ignore |

9.2.YY.2	PROPAGATION DELAY COMPENSATION INFORMATION REPORT

This message is sent by the gNB-DU and is used to report the propagation delay compensation information to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Propagation Delay Compensation Information | M |  | 9.3.1.AAA |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.3.1.ZZZ	Propagation Delay Compensation Information

This IE contains the propagation delay compensation information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Propagation Delay Compensation value | M |  | INTEGER (0..FFS) |  |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.3.1.AAA	Propagation Delay Compensation Request Type

This IE indicates the type of propagation delay compensation information reporting to be handled by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Propagation Delay Compensation Event Type | M |  | ENUMERATED (on demand, periodic, stop, …) |  |
| Propagation Delay Compensation Report Periodicity Value | C-ifPDCEventTypeisPeriodic |  | FFS | Indicates the periodicity of propagation delay compensation information report, Unit in radio frame. |

| C-ifEventYpeisStop | Explanation |
| --- | --- |
| ifPDCEventTypeisPeriodic | This IE shall be present if the Propagation Delay Compensation Event Type IE is set to "periodic". |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.4.3	Elementary Procedure Definitions

FROM F1AP-CommonDataTypes

//SKIP the unrelated part//

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

PropagationDelayCompensationInformationReportingControl,

PropagationDelayCompensationInformationReport,

AccessSuccess,

CellTrafficTrace

FROM F1AP-PDU-Contents

//SKIP the unrelated part//

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-PropagationDelayCompensationRequestType,

id-PropagationDelayCompensationInformation,

id-accessSuccess,

id-cellTrafficTrace

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

//SKIP the unrelated part//

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

propagationDelayCompensationInformationReportingControl|

propagationDelayCompensationInformationReport			,

...

}

//SKIP the unrelated part//

accessSuccess F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessSuccess

PROCEDURE CODE			id-accessSuccess

CRITICALITY				ignore

}

cellTrafficTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-cellTrafficTrace

CRITICALITY				ignore

}

propagationDelayCompensationInformationReportingControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PropagationDelayCompensationInformationReportingControl

PROCEDURE CODE			id-PropagationDelayCompensationInformationReportingControl

CRITICALITY				ignore

}

propagationDelayCompensationInformationReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PropagationDelayCompensationInformationReport

PROCEDURE CODE			id-PropagationDelayCompensationInformationReport

CRITICALITY				ignore

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

FROM F1AP-Containers

//SKIP the unrelated part//

id-TraceCollectionEntityURI,

id-ServingNID,

id-PropagationdelaycompensationRequestType,

id-PropagationDelayCompensationInformation,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofChildIABNodes,

maxnoofServedCellsIAB,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs

FROM F1AP-Constants;

//SKIP the unrelated part//

-- **************************************************************

--

##### -- Positioning Measurement Update

--

-- **************************************************************

PositioningMeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementUpdateIEs} },

...

}

PositioningMeasurementUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PROPAGATION DELAY COMPENSATION INFORMATION REPORTING CONTROL

--

-- **************************************************************

PropagationDelayCompensationInformationReportingControl::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PropagationDelayCompensationInformationReportingControlIEs} },

...

}

PropagationDelayCompensationInformationReportingControlIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-PropagationdelaycompensationRequestType		CRITICALITY ignore	TYPE PropagationdelaycompensationRequest			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PROPAGATION DELAY COMPENSATION INFORMATION REPORT

--

-- **************************************************************

PropagationDelayCompensationInformationReport::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PropagationDelayCompensationInformationReportIEs} },

...

}

PropagationDelayCompensationInformationReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-PropagationDelayCompensationInformation		CRITICALITY ignore	TYPE PropagationDelayCompensationInformation		PRESENCE mandatory	},

...

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

#### -- P

//SKIP the unrelated part//

PacketErrorRate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PropagationDelayCompensationEventType ::= ENUMERATED {

on-demand,

periodic,

stop,

...

}

PropagationDelayCompensationReportPeriodicityValue ::= FFS

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

//SKIP the unrelated part//

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

PropagationDelayCompensationInformation ::= SEQUENCE {

propagationDelayCompensationValue					PropagationDelayCompensationValue,

iE-Extensions											ProtocolExtensionContainer { {PropagationDelayCompensationInformation-ExtIEs} }	OPTIONAL

}

PropagationDelayCompensationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PropagationdelaycompensationRequestType ::= SEQUENCE {

propagationDelayCompensationeventType					PropagationDelayCompensationEventType,

propagationDelayCompensationreportPeriodicityValue		PropagationDelayCompensationReportPeriodicityValue	OPTIONAL,

-- C-ifPDCEventTypeisPeriodic: This IE shall be present if the Propagation Delay Compensation Event Type IE is set to "periodic" in the Propagation Delay Compensation Event Type IE.

iE-Extensions											ProtocolExtensionContainer { {PropagationdelaycompensationRequestType-ExtIEs} }	OPTIONAL

}

PropagationdelaycompensationRequestType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PropagationDelayCompensationValue ::= INTEGER (0..FFS)

//SKIP the unrelated part//

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList						ProtocolIE-ID ::= 390

id-PropagationDelayCompensationRequestType			ProtocolIE-ID ::= 39a

id-PropagationDelayCompensationInformation			ProtocolIE-ID ::= 39b

id-PropagationDelayCompensationInformationReport	ProtocolIE-ID ::= 39c

id-PropagationDelayCompensationInformationReportingControl	ProtocolIE-ID ::= 39d

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>