3GPP TSG-RAN WG3 Meeting #111-e	R3-210272

E-meeting, 25 Jan – 5 Feb 2021

Title:	[DRAFT] Reply LS on 5MBS progress and issues to address

Response to:	LS S2- 2009235 on 5MBS progress and issues to address from SA2

Release:	Release 17

Work Item:	FS_5MBS, NR_MBS-Core

Source:	Huawei [will be RAN3]

To:	SA2, RAN2, SA4

Cc:

Contact person:	Yan Wang

Wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN3 thanks to SA2 on the LS on 5MBS progress and issues to address, about the SA2 identified aspects in the conclusion part that are dependent on RAN WGs feedback and/or confirmation, i.e. serval Editor's notes in TR 23.757, RAN3 would like to provide feedback as follows：

- Editor's note: Whether the UE can stop receiving traffic of a multicast session without indicating leaving in CM-IDLE state or CM-CONNECTED with RRC-INACTIVE state relies on RAN WG feedback.

RAN3 feedback: Subject to RAN2 further discussion after making decision on whether the UE can receive the multicast session in CM-IDLE state or CM-CONNECTED with RRC-INACTIVE state.

- Editor's note: RAN and/or SA3 is assumed to determine the handling of the security for MBS traffic.

RAN3 feedback: Subject to RAN2 and SA3 further discussion in normative phase.

- Editor's note: How the NG-RAN node notify session activation to UEs relies on RAN WG feedback.

RAN3 feedback: For session activation of a multicast session, if the UEs are in CM-IDLE mode, the CN triggered paging will be used. For other cases, how the NG-RAN node notify session activation to UEs will be further discussed in RAN WI.

- Editor’s Note: How 5GC Shared MBS delivery is enabled for the UE will be developed with RAN WGs.

RAN3 feedback: RAN3 already replied, we will support the two methods of 5GC Shared MBS delivery over NG-U. 5GC Shared MBS delivery over Uu interface is up to RAN2.

- Editor's note: It is FFS whether the support for lossless handover with data forwarding from source NG-RAN supporting 5MBS to the target NG-RAN not supporting 5MBS is needed, which needs confirmation by RAN.

RAN3 feedback: RAN3 will further work on that after the MBS session start and the Mobility between MBS supporting nodes.

- Editor’s Note: Whether any assistance information from CN is needed, e.g. for PTP/PTM delivery method decision and switching, needs further confirmation when the relevant conclusion is reached in RAN WGs.

RAN3 feedback: RAN3 have already feedback that RAN3 could not agree for now on assistance information from 5GC to RAN for PTP/PTM delivery method decision and switching but continues discussions.

About the question from SA4, RAN3 would like to feedback as follows:

- SA4 Question: “The existing BM-SC hosts the SYNC (for time synchronization) and RoHC function. The prime reason here is MBSFN operation. SA4 understands that the 5MBS feature does not yet have a requirement for synchronization across adjacent cells, but that the related RAN normative work item does not preclude its introduction in a later release. Does SA2 have any view on the need of SYNC and/or RoHC support in the MBSF-U?”

RAN3 feedback: For SYNC, in this release, there is no SYNC protocol, any SFN operation is transparent to the UE, and any related synchronization is left to network implementation. For RoHC, it is subject to RAN2 feedback.

2	Actions

To SA2, RAN2 and SA4

ACTION: 	RAN3 kindly asks SA2, RAN2 and SA4 take the feedback above into account in their discussion.

3	Dates of next RAN3 meetings

RAN3#112-e	17 May – 28 May			Online