3GPP TSG-RAN WG3 Meeting #111-e	R3-210275

E-meeting, 25 Jan – 5 Feb 2021

Title: 	(TP to MBS TS38.413 BL CR) Support of Shared NG-U Transport

Source: 	Huawei, CMCC

Agenda item:	22.2.2

Document Type:	other

1. Introduction

In R3-210274, it is proposed to introduce two NGAP class1 procedures, including Multicast Distribution Setup procedure, and Multicast Distribution Release procedure. In this contribution, we provide the corresponding Text Proposal to the TS 38.413 BL CR.

2. TP to the MBS TS 38.413 BL CR.

--------------------------------Start of the Change-----------------------------

8	NGAP Procedures

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE REQUEST | MULTICAST DISTRIBUTION RELEASE RESPONSE |  |

--------------------------------Start of the Next Change-----------------------------

8.xx	MBS Session Management Procedures

8.xx.a	Multicast Distribution Setup

8.xx.a.1	General

The purpose of the Multicast Distribution Setup procedure is to assign NG-U resources for one MBS session and the corresponding MBS QoS flows. The procedure uses non-UE-associated signalling.

8.xx.a.2	Successful Operation

Figure 8.xx.a.2-1: Multicast Distribution Setup: successful operation

The AMF initiates the procedure by sending a MULTICAST DISTRIBUTION SETUP REQUEST message to the NG-RAN node including at least the MBS Session ID [FFS]. The AMF responds with a MULTICAST DISTRIBUTION SETUP RESPONSE message.

If the DL Transport Layer Information IE is included in the Multicast Distribution Setup Request Transfer IE in the MULTICAST DISTRIBUTION SETUP REQUEST message, the SMF shall consider that the included information as the downlink termination point for the included associated QoS flows for this MBS session.

If the DL Transport Layer Information IE is not included in the Multicast Distribution Setup Request Transfer IE in the MULTICAST DISTRIBUTION SETUP REQUEST message, the SMF shall interpret that the IP multicast is used for this MBS Session and provide IP Multicast Address Information IE in the Multicast Distribution Setup Response Transfer IE in the MULTICAST DISTRIBUTION SETUP RESPONSE message.

8.xx.a.3	Unsuccessful Operation

Figure 8.xx.a.3-1: Multicast Distribution Setup: unsuccessful operation.

In case the Multicast Distribution cannot be setup successfully, the AMF shall response with the MULTICAST DISTRIBUTION SETUP FAILURE message to the NG-RAN node with an appropriate cause value in the Cause IE.

8.xx.a.4	Abnormal Conditions

Not applicable.

8.xx.b	Multicast Distribution Release

8.xx.b.1	General

The purpose of the Multicast Distribution Release procedure is to enable the release of already established NG-U resources for a given MBS Session. The procedure uses non-UE-associated signalling.

8.xx.b.2	Successful Operation

Figure 8.xx.b.2-1: Multicast Distribution Release initiated by the NG-RAN node

Figure 8.xx.b.2-2: Multicast Distribution Release initiated by the AMF

The NG-RAN node or the AMF initiates the procedure by sending a MULTICAST DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the MULTICAST DISTRIBUTION RELEASE REQUEST message, the receiving node shall send the MULTICAST DISTRIBUTION RELEASE RESPONSE message after successfully remove the corresponding NG-U resource for the MSB Session.

8.xx.b.3	Unsuccessful Operation

Not applicable.

8.xx.b.4	Abnormal Conditions

Not applicable.

--------------------------------Start of the Next Change-----------------------------

9.x.x	MBS Session Management Messages

9.x.x.a	MULTICAST DISTRIBUTION SETUP REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Setup Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Request Transfer IE specified in subclause 9.3.4.a. | YES | reject |

9.x.x.b	MULTICAST DISTRIBUTION SETUP RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Setup Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Response Transfer IE specified in subclause 9.3.4.b. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.x.x.b	MULTICAST DISTRIBUTION SETUP FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U Transport for a MBS Session was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Setup Unsuccessful Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Unsuccessful Transfer IE specified in subclause 9.3.4.c. |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

9.x.x.d	MULTICAST DISTRIBUTION RELEASE REQUEST

This message is sent by the AMF and the NG-RAN node to request the release of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node and NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Release Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Request Transfer IE specified in subclause 9.3.4.d. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.2.6	MULTICAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the NG-RAN node and the AMF to confirm the release of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF and AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Release Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Response Transfer IE specified in subclause 9.3.4.e. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

--------------------------------Start of the Next Change-----------------------------

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

--------------------------------Start of the Next Change-----------------------------

9.3.2.2	UP Transport Layer Information

This IE is used to provide the NG user plane transport layer information associated with a PDU session or an MBS Session for an NG-RAN node – UPF pair. In this release it corresponds to an IP address and a GTP Tunnel Endpoint Identifier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UP Transport Layer Information | M |  |  |  |
| >GTP tunnel |  |  |  |  |
| >>Endpoint IP Address | M |  | Transport Layer Address 9.3.2.4 |  |
| >>GTP-TEID | M |  | 9.3.2.5 |  |

--------------------------------Start of the Next Change-----------------------------

9.3.4.a	Multicast Distribution Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | YES | reject |

9.3.4.b	Multicast Distribution Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| IP Multicast Address Information |  | 0..1 |  |  | YES | reject |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  | - | - |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  | - | - |
| >GTP DL TEID | M |  | GTP-TEID 9.3.2.5 |  | - | - |

9.3.4.c	Multicast Distribution Setup Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.4.d	Multicast Distribution Release Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |

9.3.4.e	Multicast Distribution Release Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

--------------------------------End of the Changes-----------------------------