3GPP TSG-RAN WG3 Meeting #111-e	R3-210278

E-meeting, 25 Jan – 5 Feb 2021

Title: 	(TP to TS 38.470 BL CR and TS 38.460 BL CR) Bearer management over F1 and E1

Source: 	Huawei

Agenda item:	22.2.4

Document Type:	Discussion, Decision

1. Introduction

As discussed in [1], the following proposals can be found:

Proposal 6. Also support IP multicast method for the shared F1-U transport.

Proposal 7: To support Shared NG-U transport, it is needed to introduce a non UE associated Class1 E1AP procedure, e.g. named as Multicast Distribution Setup procedure.

Proposal 8: To support Shared F1-U transport, it is needed to introduce a non UE associated Class1 F1AP procedure, e.g. named as Multicast Distribution Setup procedure.

Proposal 9: To support Shared F1-U transport, it is needed to introduce a non UE associated Class1 E1AP procedure, it could be the same procedure to setup shared NG-U transport, or another new procedure.

In this contribution, we provide corresponding TPs to TS 38.470 and TS38.460 BL CRs.

2. Reference

- R3-210277 (TP to TS 38.401 BL CR) Bearer management over F1 and E1, Huawei

3. TP for TS38.470 BL CR

--------------------------------Start of the Next Change-----------------------------

5.2.x	NR MBS function

Eidtor Notes: TBD

The NR MBS function supports the establishment, modification and releasing the resources for the MBS shared F1-U transmission between the gNB-CU and gNB-DU.

If IP multicast based transmission is used, the gNB-CU allocates the Transport Layer Address used for the IP multicast and the DL TEID used for the shared F1-U Transport, and then the gNB-DU joins the IP multicast group of MBS traffic for the shared F1-U transport.

If IP multicast based transmission is not used, the gNB-DU assigns DL GTP-U Tunnel information used for the shared F1-U Transport, and then the gNB-DU transmit the NR MBS packets to the gNB-DU via the shared F1-U tunnel.

--------------------------------Start of the Next Change-----------------------------

6.1.x	NR MBS procedures

The F1 MBS procedures are listed below:

- [TBD];

- Multicast Distribution Setup procedure

- Multicast Distribution Modification procedure

- Multicast Distribution Release procedure

--------------------------------End of the Changes-----------------------------

4. TP for TS38.460 BL CR

--------------------------------Start of the Change-----------------------------

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DL	Downlink

DRB	Data Radio Bearer

E1AP	E1 Application Protocol

IP	Internet Protocol

MBS                    Multicast and Broadcast Service

SCTP	Stream Control Transmission Protocol

TNL	Transport Network Layer

-------------------------------- Next Change-----------------------------

5	Functions of the E1 interface

5.1	General

The following clauses describe the functions supported over E1.

>>>>>>>>>>skip the unchanged part<<<<<<<<<<

5.1.X	Multicast Distribution Management function

This function is used to establish, modify and release the resources for MBS session related shared F1-U transmission between the gNB-DU and the gNB-CU-UP, and shared NG-U between the gNB-CU-UP and the UPF.

If IP multicast based transmission is used for the shared F1-U Transport, the gNB-CU-UP allocates the Transport Layer Address used for the IP multicast and the DL TEID used for the shared F1-U Transport, provides such information to the gNB-DU via gNB-CU-CP, and then the gNB-DU joins the IP multicast group of MBS traffic for the shared F1-U transport.

If IP multicast based transmission is not used for the shared F1-U Transport, the gNB-DU assigns DL GTP-U Tunnel information used for the shared F1-U Transport, provides such information to the gNB-CU-UP via gNB-CU-CP, and then the gNB-DU-UP transmits the NR MBS packets to the gNB-DU via the shared F1-U tunnel.

If IP multicast based transmission is used for the shared NG-U Transport, the gNB-CU-CP forwards the Transport Layer Address used for the IP multicast and the DL TEID provided from core network used for the shared NG-U Transport to the gNB-CU-UP, and then the gNB-CU-UP joins the IP multicast group of MBS traffic for the shared NG-U transport.

If IP multicast based transmission is not used for the shared NG-U Transport, the gNB-CU-UP assigns DL GTP-U Tunnel information used for the shared NG-U Transport, provides such information to the core network via gNB-CU-CP, and then the core network transmits the NR MBS packets to the gNB-CU-UP via the shared NG-U tunnel.

5.2	TEIDs allocation

The gNB-CU-UP is responsible for the allocation of the F1-U UL GTP TEID for each data radio bearer.

The gNB-CU-UP is responsible for the allocation of the S1-U DL GTP TEID for each E-RAB and the NG-U DL GTP TEID for each PDU Session.

The gNB-CU-UP is responsible for the allocation of the X2-U DL/UL GTP TEID or the Xn-U DL/UL GTP TEID for each data radio bearer.

If IP multicast based transmission is not used for the shared NG-U transport for a MBS session, the gNB-CU-UP is responsible for the allocation of the NG-U DL GTP TEID for the MBS Session.

-------------------------------- Next Change-----------------------------

6	Procedures of the E1 interface

>>>>>>>>>>skip the unchanged part<<<<<<<<<<

6.X	Multicast distribution management procedures

The following list of Multicast distribution management procedures are used to establish, modify or release F1-U Multicast Distribution resources and NG-U Multicast Distribution resources for MBS Session. The procedures providing this functionality are:

-	Multicast Distribution Setup procedure

-	Multicast Distribution Modification procedure

-	Multicast Distribution Release procedure

--------------------------------End of the Changes-----------------------------