3GPP TSG-RAN3 Meeting #111-e	R3-210295

25 January – 4 February 2021 E-Meeting

Title: 	(TP on SN Change failure for 38.423) Addition of SCG Failure Information transfer

Source: 	CATT

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

This contribution is the TP for TS38.423 v16.4.0 with addition of SCG Failure Information.

# 2. Discussion

As discussed in R3-210294, a new XnAP message to send scgFailureInfo from MN to SN is introduced.

# 3. Annex – TP

Start of the first change

8	XnAP procedures

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SCG Failure Information | SCG FAILURE INFORMATION |

8.3.X	SCG Failure Information

8.3.X.1	General

The purpose of the SCG Failure Information procedure is to transfer SCG failure related information from M-NG-RAN to S-NG-RAN.

The procedure uses non UE-associated signalling.

8.3.X.2	Successful Operation

Figure 8.3.X.2-1: SCG Failure Information procedure, successful operation.

The M-NG-RAN-NODE initiates the procedure by sending the SCG FAILURE INFORMATION message to the S-NG-RAN-NODE.

8.3.X.3	Abnormal Conditions

Not applicable.

9.1.2.X	SCG FAILURE INFORMATION

This message is sent by M-NG-RAN-NODE to transfer SCG failure related information to S-NG-RAN-NODE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| SCG Failure Information Container | M |  | OCTET STRING | scgFailureInfo as defined in subclause 11.2.2 in TS 38.331 [10]. | YES | ignore |

End of the last change