3GPP TSG-RAN WG3 #111-e	R3-210310

E-meeting, 25st January – 4th February 2021	Resubmission of R3-206135

Source:	CATT

Title:	(TP on SON for 38.473) TP on PRACH coordination for F1AP

Agenda Item:	10.2.1.7

Document for:	Discussion and decision

# Introduction

This is a TP based on R3-210309 “Discussion on Rel-16 leftover issues for PRACH coordination”.

# TP for TS 38.473

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////

8.2.3	F1 Setup

8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages may be exchanged.

If the F1 SETUP REQUEST message contains the gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU.

If the F1 SETUP REQUEST message contains the gNB-DU Served Cells List IE, the gNB-CU shall take into account as specified in TS 38.401 [4].

For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE and the TAI Slice Support List IE in the F1 SETUP REQUEST message.

The gNB-CU may include the Cells to be Activated List IE in the F1 SETUP RESPONSE message. The Cells to be Activated List IE includes a list of cells that the gNB-CU requests the gNB-DU to activate. The gNB-DU shall activate the cells included in the Cells to be Activated List IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is included in the F1 SETUP RESPONSE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE in the F1 SETUP RESPONSE message, if the available PLMN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s).

For NG-RAN, the gNB-CU may include Available SNPN ID List IE in the F1 SETUP RESPONSE message. If the available SNPN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the SNPN(s) included in the received Available SNPN ID list.

The Latest RRC Version Enhanced IE shall be included in the F1 SETUP REQUEST message and in the F1 SETUP RESPONSE message.

If in F1 SETUP REQUEST message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in F1 SETUP REQUEST message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the Intended TDD DL-UL Configuration IE is present in the F1 SETUP REQUEST message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the F1 SETUP REQUEST message contains the Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the F1 SETUP RESPONSE message contains the Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the F1 SETUP RESPONSE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the BAP Address IE is included in the F1 SETUP REQUEST, the receiving gNB-CU shall, if supported, consider the information therein for discovering the collocation of an IAB-DU and an IAB-MT.

If the F1 SETUP REQUEST message is received from an IAB-donor-DU, the gNB-CU shall, if supported, include the BAP Address IE in the F1 SETUP RESPONSE message.

If the F1 SETUP RESPONSE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the Cell Information Notification List IE is present in the F1 SETUP RESPONSE message, the gNB-DU may store or update the information. If the NR Cell PRACH Configuration IE is present in the Cell Information Notification Item IE, the gNB-DU may store or update the information and use it to adjust the PRACH configuration of its serving cell.

8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

If the F1 SETUP FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the F1 setup towards the same gNB-CU.

8.2.3.4	Abnormal Conditions

Not applicable.

8.2.4	gNB-DU Configuration Update

8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If gNB-DU ID IE is contained in the GNB-DU CONFIGURATION UPDATE message for a newly established SCTP association, the gNB-CU will associate this association with the related gNB-DU.

If Served Cells To Add Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall add cell information according to the information in the Served Cell Information IE. For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE.

If Served Cells To Modify Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall modify information of cell indicated by Old NR CGI IE according to the information in the Served Cell Information IE and overwrite the served cell information for the affected served cell. Further, if the gNB-DU System Information IE is present the gNB-CU shall store and replace any previous information received.

If Served Cells To Delete Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall delete information of cell indicated by Old NR CGI IE.

If Cells Status Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall update the information about the cells, as described in TS 38.401 [4]. If if the Switching Off Ongoing IE is present in the Cells Status Item IE, contained in the GNB-DU CONFIGURATION UPDATE message, and the corresponding Service State IE is set to “Out-of-Service”, the gNB-CU shall ignore the Switching Off Ongoing IE.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If Cells to be Deactivated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall deactivate all the cells with NR CGI listed in the IE.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the GNB-DU CONFIGURATION UPDATE message includes gNB-DU TNL Association To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-DU TNL Association To Remove List IE, the gNB-CU shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-DU. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-DU TNL Association To Remove List IE in GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-DU.

If the Intended TDD DL-UL Configuration IE is present in the GNB-DU CONFIGURATION UPDATE message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration IE content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the GNB-DU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the Cell Information Notification List IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update the information. If the NR Cell PRACH Configuration IE is present in the Cell Information Notification Item IE, the gNB-DU may store or update the information and use it to adjust the PRACH configuration of its serving cell.

8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

8.2.4.4	Abnormal Conditions

Not applicable.

8.2.5	gNB-CU Configuration Update

8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

If in the gNB-CU CONFIGURATION UPDATE message the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows IAB-node access or not.

If the Cell Information Notification List IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update the information. If the NR Cell PRACH Configuration IE is present in the Cell Information Notification Item IE, the gNB-DU may store or update the information and use it to adjust the PRACH configuration of its serving cell.

8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

8.2.5.4	Abnormal Conditions

Not applicable.

////////////////////////////////////////////////////////////////////////skip unrealted text////////////////////////////////////////////////////////////////////////

9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information associated to an F1-C interface instance.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| gNB-CU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| Cell Information Notification List | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////

9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Cells to be Deactivated List |  | 0.. 1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| Cell Information Notification List | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////

9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  |  | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Global Identifier as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| Cell Information Notification List | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum nunmerbs of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unrealted text////////////////////////////////////////////////////////////////////////

9.3.1.x1	Cell Information Notification List

This IE used to deliver cell information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Cell Information Notification Item |  | 1 .. <maxCellingNBDU> |  |  |
| >CHOICE Cell Information Notification Item | M |  |  |  |
| >>NR |  |  |  |  |
| >>>NR Cell Information Notification Info |  | 1 |  |  |
| >>>>NR CGI | M |  | 9.3.1.12 |  |
| >>>>Served Cell Neighbouring List | O |  | 9.3.1.x2 | A list of cells served by the gNB-DU that neighbours this nofiticated cell |
| >>>>NR Mode Info Rel16 | O |  | 9.3.1.x3 |  |
| >>>>SSB Positions In Burst | O |  | 9.3.1.138 |  |
| >>>>NR Cell PRACH Configuration | O |  | 9.3.1.139 |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum nunmerbs of cells that can be served by a gNB-DU. Value is 512. |

9.3.1.x2	Served Cell Neighbouring List

This IE contains the CGIs of a list of cells served by the gNB-DU and neighbouring to the concerned cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Served Cell Neighbouring Item |  | 1 .. <maxnoofNeighbours> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 1024. |

9.3.1.x3	NR Mode Info Rel16

This IE contains the information of a NR cell which needs to be encoded differently for FDD and TDD.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NR-Mode-Info-Rel16 | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info-Rel16 |  | 1 |  |  |
| >>>UL FreqInfo | O |  | NR Frequency Info 9.3.1.17 |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 |  |
| >TDD |  |  |  |  |
| >>TDD Info-Rel16 |  | 1 |  |  |
| >>>NR FreqInfo | O |  | NR Frequency Info 9.3.1.17 |  |
| >>>TDD DL-UL Configuration Common NR | O |  | OCTET STRING | The tdd-UL-DL-ConfigurationCommon as defined in TS 38.331 [8] |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 |  |

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////