3GPP TSG-RAN WG3 Meeting #111-e	R3-210383

Electronic Meeting, Jan 25th – Feb 5th, 2021

Agenda item:		22.2.2

Source:	Intel Corporation

Title:	Session Management Discussion

Document for:		Discussion and Decision

# 1		Introduction

In RAN3#111e, the following were agreed:

Potential agreements or WA related to decision of SA2 for solution 2: 1/ WA: If solution 2 is agreed by SA2, have common procedure for Session start/Release for multicast and broadcast. Session Start message will contain at least the following information: TMGI, QoS Information, transport information, Broadcast area. 2/ If solution 2 is agreed by SA2, use an existing NG UE-associated procedure to setup the UE MBS context. Procedure is FFS. Potential agreements or WA related to decision of SA2 for solution 3: 1/ WA: If solution 3 is agreed by SA2, the UE MBS context is setup using PDU Session Modify procedure. 2/ If solution 3 is agreed by SA2, 5GC send to NG-RAN to setup MBS context at least the following information: MBS Session ID, MB-SMF ID, multicast QoS flow information. 3/ If solution 3 is agreed by SA2, agree that NG-RAN triggers a class 1 non-UE associated procedure to setup the user plane between NG-RAN and MB-UPF (procedure FFS). The uplink initiating message will include at least the following information: MB-SMF ID and DL Transport layer address. The downlink response message will include at least the following information: IP multicast address

Based on version 1.2.0 of [1], we provided discussion on the following:

- FFS on shared NG-u

- Potential agreement or WA for solution 3

# Discussion

## FFS on shared NG-u

During the discussion in RAN3#110, according the chair notes, there is an FFS on shared NG-U:

1/ for shared NG-U, it is FFS if gNB or 5GC makes the final decision for multicast transport or unicast transport

According to [1], from the viewpoint of 5G CN, there are two delivery methods:

- 5GC Individual MBS traffic delivery method: 5G CN receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions, hence for each such UE one PDU session is required to be associated with a multicast session

- 5GC Shared MBS traffic delivery method: 5G CN receives a single copy of MBS data packets and delivers a single copy of those MBS packets packet to a RAN node, which then delivers them to one or multiple UEs

According to [1], the primary aim of key issue #7 is to study the condition, location, and RAN consideration in switching between individual traffic delivery or MBS traffic delivery method. And the conclusion according to section 8.7 of [1] is shown below:

For delivery method switching not due to mobility, the following principle are agreed, - Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods. - If the NG-RAN node supports 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer

Observation 1: According to the conclusion of key issue #7, if the NG-RAN node supports 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

From the conclusion of [1] as shown above (highlighted), we can close the FFS by stating that it is the 5GC that makes the final decision on multicast or unicast.

Proposal 1: Close the FFS by agreeing with [1] that it is the 5GC that makes the final decision on multicast or unicast

## WA for solution 3

From [1] section 8.2, SA2 has made a conclusion on key issue #1 MSB Session Management. The conclusion is summarized in the call flow below copied from [1] section 8.2.3

Although, there is no explicit text in [1] that stated the conclusion of KI#1 is based on solution 3, but from section 8.22 and the call flow, it is obvious the conclusion is based on solution 3.

Observation 2: SA2 has concluded the study on key issue#1 based on solution 3.

Therefore, we proposed:

Proposal 2: RAN3 should agree on:

- The UE MBS context is setup using PDU Session Modify procedure.

- 5GC send to NG-RAN to setup MBS context at least the following information: MBS Session ID, MB-SMF ID, multicast QoS flow information

- NG-RAN triggers a class 1 non-UE associated procedure to setup the user plane between NG-RAN and MB-UPF (procedure FFS). The uplink initiating message will include at least the following information: MB-SMF ID and DL Transport layer address. The downlink response message will include at least the following information: IP multicast address

# Conclusion

In the present contribution we make the following observations:

Observation 1: According to the conclusion of key issue #7, if the NG-RAN node supports 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

Observation 2: SA2 has concluded the study on key issue#1 based on solution 3.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: Close the FFS by agreeing with [1] that it is the 5GC that makes the final decision on multicast or unicast

Proposal 2: RAN3 should agree on:

- The UE MBS context is setup using PDU Session Modify procedure.

- 5GC send to NG-RAN to setup MBS context at least the following information: MBS Session ID, MB-SMF ID, multicast QoS flow information

- NG-RAN triggers a class 1 non-UE associated procedure to setup the user plane between NG-RAN and MB-UPF (procedure FFS). The uplink initiating message will include at least the following information: MB-SMF ID and DL Transport layer address. The downlink response message will include at least the following information: IP multicast address

# Reference

[1] TR 23.757 “Study on architectural enhancement for 5G multicast-broadcast services”