3GPP TSG-RAN WG3 Meeting #111-e	R3-210395

E-meeting, 25 Jan – 5 Feb 2021

Title: 	(TP for SON BLCR for 38.423) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	other

1. Introduction

The MRO for SN Change Failure was discussed during RAN3#110-e. In R3-207029, the summary of the offline discussion was presented.

2. Background

In RAN3#110 e-meeting, there were some agreements and open issues on SN change failure. We’ll give our considerations based on the agreements and open issues.

Agreements:

“PSCell change” shall be mentioned in the definitions

WA: No need to transmit Time threshold (i.e. the Tstore_UE_cntxt) over network interface.

Open issues

- Whether “MN’s action” shall be mentioned in the definitions;

- Whether to enhance SCG Failure Information e.g. CGI of the Source PSCell, CGI of the Failed PSCell, timeSCGFailure;

- Which message is used from MN to the Source SN for SN triggered Pscell change;

3. Discussion

3.1 MRO definitions

In the TR, the MRO definitions were defined to include MN action, e.g. as follows:

-	Failures due to too late SN change triggering: an SCG failure occurs after the UE has stayed for a long period of time in the cell of the SN; the MN makes decisions for UE, making UE to establish the radio link connection in a different SN.

-	Failures due to too early SN change triggering: an SCG failure occurs shortly after a successful SN change from a source SN to a target SN or a SN change failure occurs during the SN change procedure; the MN makes decisions for UE, making UE to re-establish the radio link connection in the source SN.

-	Failures due to change to wrong SN triggering: an SCG failure occurs shortly after a successful SN change from a source SN to a target SN or a SN change failure occurs during the SN change procedure; the MN makes decisions for UE, making UE to establish the radio link connection in a SN other than the source SN or target SN.

When the UE detects SCG failure, it can send the SCG Failure Information to the MN. Currently there are potential measurements results in the SCG Failure Information reported by the UE. To quickly recover the SCG configuration and maintain the UE experience, it is highly anticipated that the MN will select a new target PScell for the UE upon reception of the SCGFailureInformation message for both MN and SN initiated SN mobility if the MN decides to continue DC for the UE. If we wait for the source SN to re-decide the new target PScell, it will bring additional latency due to the delivery of the SCGFailureInformation and the SN Modification Required messages. Besides, it is a bit questionable whether the source SN can know whether the new PSCell belonging to a new SN is a suitable candidate for SCG connection, e.g. whether there is an Xn connection between the MN and the new SN. It is also reasonable for MN to perform the new PScell decision.

Observation 1: For SCG failure, MN can makes the decision and initiates the SN addition procedure to reduce the latency for new PSCell addition.

Particularly, we should not mandate that the MN performs the addition of a new target PScell. For example, the MN may determine not to configure the UE with DC any more. In this case, the MN can also make a decision on which cell is a suitable PScell to further identify the failure type of the SN change.

Observation 2: For SCG failure, MN can decides the suitable target PScell for failure type analysis if the MN doesn’t configure DC to the UE.

In last meeting, it was agreed to mention the PSCell change in the definitions for the SN related mobility. One proposed solution is to instead the decisions of the measurements by referring to the best cell:

-	Too late PSCell change triggering: an SCG failure occurs after the UE has stayed for a long period of time in a PSCell of the SN; the measurements reported in the SCG failure report from the UE indicate another PSCell in the same or a different SN as the best candidate for SCG connection.

-	Too early PSCell change triggering: an SCG failure occurs shortly after a successful PSCell change or an SCG failure occurs during the PSCell change procedure (intra-SN or inter-SN); the measurements reported in the SCG failure report from the UE indicate the same PSCell that the UE used prior to the failure as the best candidate for SCG connection.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change or an SCG failure occurs during the PSCell change procedure (intra-SN or inter-SN); the measurements reported in the SCG failure report from the UE indicate a PSCell different than the source PSCell or the target PSCell, in the same or a different SN, as the best candidate for SCG connection.

In this solution we rely on the definition of "best candidate for SCG connection". The main problem with this definition is that this statement is a bit ambiguous. First question is who decides this. If it is the SN, there will be some problems as analysed in the above section, e.g., whether the target SN has Xn connection with the MN or undesirable latency if configuring new DC. The other problem is the definition of best, which just points to the best PSCell and not necessarily a suitable candidate. Hence this means that the definition can include cells that although not having good enough radio quality, still being the best of the ones detected is selected.

The solution we prefer is to allow the MN to indicate a suitable PSCell. In this way the MN can indicate a PSCell that has good enough radio quality and also being a suitable candidate in terms of e.g. Xn connectivity. Besides, for the Change to wrong PSCell case, it can also cover the SN addition case that the MN wrongly adds a new target PSCell for the UE.

-	Too late PSCell SN change triggering: an SCG failure occurs after the UE has stayed for a long period of time in a PSCell of the SN; the MN indicates another PSCell as a suitable candidate for SCG connection.

-	Too early PSCell change triggering: an SCG failure occurs shortly after a successful PSCell change or an SCG failure occurs during the PSCell change procedure (intra-SN or inter-SN); the MN indicates the same PSCell as a suitable candidate for SCG connection.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell addition/change or an SCG failure occurs during the PSCell addition/change procedure (intra-SN or inter-SN); the MN indicates a PSCell different than the source PSCell or the target PSCell as a suitable candidate for SCG connection.

Include that the MN indicates a suitable candidate for SCG connection in MRO definition

3.2 Enhancements to SCG failure information

As previously agreed, legacy UEs are supported by the MN storing information related to UEs to enable the agreed mechanism. For MN, it can store the source PSCell, failed PSCell related cell information. It can basically deduce the time since the PSCell change until the SCG failure based on the times when the MN added the SN and when the MN receives the SCG Failure Information from UE.

Hence, if we discuss additional information from the UE, we should only discuss adding information that enhances legacy behaviour.

The SCG Failure Information is not enhanced.

3.3 Signalling for MRO detection

As discussed in section 3.1, the MN can decide the new target PSCell in case of SCG failure. For MN, it can also receive the SCG Failure Information from the UE. The MN can perform the initial failure detection and analysis. The MN can finally decide the failure type based on the new PSCell decided by MN. The MN based detection solution can work well for both MN and SN initiated PSCell failure cases [1].

Therefore, if the SN change is triggered by MN or too late PScell change failure, the MN can decide the failure type and adjust the parameters setting.

If the SN change is triggered by the source SN, the MN performs the initial analysis, decides the failure type (too early PScell change, or change to wrong PScell) and informs the source SN, together with UE report (e.g., SCGFailureInformation) and the new target PScell information.  The resulting solution would require a single new “SCG report” message sent from MN to the source SN, which can be SCG CHANGE REPORT. The potential signalling procedures are provided for too early PScell change failure.

Besides, to retrieve more relevant information collected at the source SN side as part of the UE context, the MN provides UE ID for the source SN to identify the UE. But in MR-DC, the MN does not know the C-RNTI of UE in the source SN. Therefore it is preferred to use the SN UE X2AP ID as a reference to the UE context in the source SN.

Proposal 3:  If the PScell change is triggered by the source SN, the MN performs initial analysis and sends the SCG CHANGE REPORT message to the source SN;

Proposal 4: The SCG CHANGE REPORT message could include SN failure type, new target PScell information, the SCGFailureInformation container, and the SN UE X2AP ID for the source SN to identify the UE.

4. Conclusion

Based on the discussion in this paper, we propose the following:

- Include that the MN indicates a suitable candidate for SCG connection in MRO definition

The SCG Failure Information is not enhanced.

Proposal 3:  If the PScell change is triggered by the source SN, the MN performs initial analysis and sends the SCG CHANGE REPORT message to the source SN;

Proposal 4: The SCG CHANGE REPORT message could include SN failure type, new target PScell information, the SCGFailureInformation container, and the SN UE X2AP ID for the source SN to identify the UE.

5. Reference

- R3-206191, (TP for SON BLCR for 38.423) MRO for SN Change Failure, Huawei

Annex – TP on TS 38.423

Start of the first change

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SN change report | SN CHANGE REPORT |

Start of the next change

8.4.x	SN change Report

8.4.x.1	General

The purpose of the SN Change Report procedure is to transfer mobility related information from M-NG-RAN node to S-NG-RAN node.

The procedure uses non UE-associated signalling.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SN Change Report, successful operation

An M-NG-RAN node initiates the procedure by sending an SN CHANGE REPORT message to S-NG-RAN node. By sending the message M-NG-RAN node indicates to S-NG-RAN node that a SN mobility-related problem was detected.

If the SN change Report Type IE is set to "too early PScell change" or "change to wrong PScell", then the M-NG-RAN node indicates to S-NG-RAN node that, following a PScell change from a source PScell to a target PScell node, a SCG failure occurred and the UE established connection either at the original cell of PScell (too early PScell change), or at another cell (change to wrong PScell). If the SN change Report Type IE is set to "too late PScell change", then the M-NG-RAN node indicates to S-NG-RAN node that, following a before PScell change trigger from a source PScell to a target PScell, a SCG failure occurred.

8.4.x.3	Unsuccessful Operation

Not applicable.

8.4.x.4	Abnormal Conditions

Void.

Start of the next change

9.1.3.x	SN CHANGE REPORT

This message is sent by the M-NG-RAN node to report a PScell change failure event to S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| CHOICE SN change Report Type | M |  |  |  |  |  |
| > too early PScell change |  |  |  |  |  |  |
| >>Source PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PScell for SN change procedure | YES | ignore |
| >>Failure PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of target PScell for SN change procedure | YES | ignore |
| > too late PScell change |  |  |  |  |  |  |
| >>Source PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PScell for SN change procedure | YES | ignore |
| >>Target PScell CGI |  |  | NR CGI 9.2.2.7 | CGI of the new suitable PScell | YES | ignore |
| > change to wrong PScell |  |  |  |  |  |  |
| >>Source PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PScell for SN change procedure | YES | ignore |
| >>Failure PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of target PScell for SN change procedure | YES | ignore |
| >>Target PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of the new suitable PScell | YES | ignore |
| UE SCGfailureinformation Report Container | O |  | OCTET STRING | The received SCGFailureInformation or SCGFailureInformationEUTRA message defined in TS 38.331 [10] from UE. | YES | ignore |

End of the last change