3GPP TSG-RAN WG3 Meeting #111-e	R3-210400

E-meeting, 25 Jan – 5 Feb 2021

Title: 	(TP for SON BLCR for 38.413) Inter-System Load Balancing

Source: 	Huawei

Agenda item:	10.2.4

Document Type:	other

# 1. Introduction

The Inter-System Load Balancing was discussed during RAN3#110-e. In R3-207032, the Summary of the offline was presented.

# 2. Background

Introduce a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via modification of the Inter-System SON Information IE

Introduce a new mechanism for Inter System Status Request/Response/Update over S1: UL RAN CONFIGURATION TRANSFER and  S1: DL RAN CONFIGURATION TRANSFER, via reuse of the Inter-System SON Configuration Transfer IE

Support periodic inter system load reporting with periodicity not lower than 1000ms and threshold-based load reporting, subject to confirmation from CT

We do not support per slice load information for inter system load balancing in the current release

Support an explicitly signaled threshold configuration for inter system load information reporting; details are FFS

Agree to CAC encoding as defined in LTE, e.g. in TS36.413, as a starting point. Whether CAC is encoded according to the sender’s rules is FFS

Whether to support the Number of active UEs for inter system load balancing is FFS

It is FFS whether to support signaling of PRB utilization for inter system load balancing in the current release;

It is FFS whether to support signaling of the Number of RRC connections for inter system load balancing in the current release;

It is FFS whether to support signaling of the TNL Available Capacity for inter system load balancing in the current release;

To be continued...

# 3. Discussion

## 3.1 Signalling mechanism

If we compare the load reporting on Xn, the reporting across systems needs to be much simpler. One reason is that cost of signalling is higher. Another reason is that the delay for sending the command is bigger. This means that we should consider not having too short periodicity of measurements, but also that the general mechanism should be as simple as possible:

In Xn there can be multiple measurement tasks, all identified by measurements IDs. There is also a possibility to start/stop or modify these measurements. This complexity is not needed for inter system measurements. For inter system it is enough to have one reporting task per node pair (sending/receiving). There is also no need to explicitly modify measurements. In order to keep the signalling simple we would instead propose to simply overwrite the previous measurement configuration.

Since we assume there will be periodic and even triggered measurements, these can be defined with a choice structure.

For differentiating between starting (and modifying) and stopping measurement, we would propose to indicate that a measurement shall stop by using periodicity value = 0. This value can also be used for requesting single measurements.

Use as simple and clear message structure.

## 3.2 Event thresholds

The thresholds for event based signalling was discussed in LTE. The solution selected for LTE provided a simple way to send number of thresholds and was considered as an easy way for the requesting node to specify the granularity of the reporting. As described in TS 36.413:

## B.1.11	Event-Triggered Cell Load Reporting Request

This IE contains request information for inter-RAT cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number Of Measurement Reporting Levels | M |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale below the reporting node's threshold for overload. The reporting node sends a report each time the cell load changes from one reporting level to another, and when the cell load enters and exits overload state. If the reporting RAT is eHRPD, triggering is based on sector load. |

This is depicted in the figure below, where 3 reporting levels are assumed:

overload

100

Figure 1. LTE inter-system load reporting mechanism

We think this principle can be reused also for NG-RAN. One possible limitation of this scheme is however that we distribute the thresholds evenly. Considering that load balancing is typically more interesting for higher loads, one possible improvement would be to introduce a starting point for the distributions. This would make it possible to limit reporting to the overload states. This is depicted in the figure below.

CAC

overload

100

Starting

point

Figure 2. LTE inter-system load reporting with starting point mechanism

In this way, the node will only report when changing report levels for higher load.

For event reporting, use same principle as in LTE but add a starting point.

## 3.2 CAC encoding

In order to simplify the reporting and the handling of events, we propose to report the CAC as defined in 36.423. Apart from making the signalling and threshold handling simpler, it will also be easier for the eNB to interpret. Although this information is not the same as for Xn, we think this is good enough for measurement reporting across systems

Use CAC as defined in 36.423.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	Use as simple and clear message structure.

Proposal 2:	For event reporting, use same principle as in LTE but add a starting point.

Proposal 3:	Use CAC as defined in 36.423.

# Annex – TP

TS 38.413

<<<<<<<<<<<<<<<<<<<< First Changes Begin >>>>>>>>>>>>>>>>>>>>

### 8.8.2	Downlink RAN Configuration Transfer

#### 8.8.2.1	General

The purpose of the Downlink RAN Configuration Transfer procedure is to transfer RAN configuration information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Downlink RAN configuration transfer

The procedure is initiated with a DOWNLINK RAN CONFIGURATION TRANSFER message sent from the AMF to the NG-RAN node.

If the NG-RAN node receives, in the SON Configuration Transfer IE or in the EN-DC SON Configuration Transfer IE, the SON Information IE containing the SON Information Request IE, or in the Inter-system SON Configuration Transfer IE, the Inter-system SON information IE containing the Inter-system SON Information Request IE, it may transfer back the requested information either towards the NG-RAN node indicated in the Source RAN Node ID IE of the SON Configuration Transfer IE or towards an eNB indicated in the Source eNB-ID IE of the EN-DC SON Configuration Transfer IE by initiating the Uplink RAN Configuration Transfer procedure.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the Xn TNL Configuration Info IE containing the Xn Extended Transport Layer Addresses IE, it may use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the SON Information IE containing the SON Information Reply IE including the Xn TNL Configuration Info IE as an answer to a former request, it may use it to initiate the Xn TNL establishment.

In case the IP-Sec Transport Layer Address IE is present and the GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE.

In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the Xn GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE.

In case the Xn GTP Transport Layer Addresses IE is empty and the IP-Sec Transport Layer Address IE is present, SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE, within the Xn Extended Transport Layer Addresses IE.

In case the Xn SCTP Transport Layer Addresses IE is present and the IP-Sec Transport Layer Address IE is also present, the concerned SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in this IP-Sec Transport Layer Address IE, within the Xn Extended Transport Layer Addresses IE.

If the NG-RAN node receives the SON Information IE containing the SON Information Report IE it may use it as specified in TS 38.300 [8].

If the NG-RAN node receives the Inter-system SON Information IE containing the Inter-system SON Information Report IE it may use it as specified in TS 38.300 [8].

If the NG-RAN node receives the Inter-system SON Information IE containing the Inter-system SON Information Request IE, it may use it as specified in TS 38.300 [8].

If the NG-RAN receives the Inter-system SON Information IE containing the Inter-system SON Information Reply IE it may use it as specified in TS 38.300 [8].

If the NG-RAN node is configured to use one IPsec tunnel for all NG and Xn traffic (IPsec star topology) then the traffic to the peer NG-RAN node shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

#### 8.8.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< First Changes End >>>>>>>>>>>>>>>>>>>>

## 9.3	Information Element Definitions

#### 9.3.3.33	Inter-system SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, transmitted between an NG-RAN node and an eNB and additionally includes the node identifier of the destination of this configuration information and the node identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Transfer Type | M |  |  |  |
| >from E-UTRAN to NG-RAN |  |  |  |  |
| >>Source eNB-ID |  | 1 |  |  |
| >>>Global eNB ID | M |  | 9.3.1.165 |  |
| >>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| >>Target NG-RAN node ID |  | 1 |  |  |
| >>>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >from NG-RAN to E-UTRAN |  |  |  |  |
| >>Source NG-RAN Node ID |  | 1 |  |  |
| >>>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >>Target eNB-ID |  | 1 |  |  |
| >>>Global eNB ID | M |  | 9.3.1.165 |  |
| >>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| Inter-system SON Information | M |  | 9.3.3.34 |  |

#### 9.3.3.34	Inter-system SON Information

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information | M |  |  |  |
| >Inter-system SON Information Report |  |  |  |  |
| >>Inter-system SON Information Report | M |  | 9.3.3.36 |  |
| >Inter-system SON Information Request |  |  |  |  |
| >>Inter-system SON Information Request | M |  | 9.3.3.xx |  |
| >Inter-system SON Information Reply |  |  |  |  |
| >>Inter-system SON Information Reply | M |  | 9.3.3.yy |  |

#### 9.3.3.36	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >>Inter-system HO Report | M |  | 9.3.3.40 |  |
| >Failure Indication Information |  |  |  |  |
| >>Inter-system Failure Indication | M |  | 9.3.3.38 |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication | M |  | 9.3.3.aa |  |
| >Inter-system Load Reporting |  |  |  |  |
| >>Inter-system Load Report | M |  | 9.3.3.y2 |  |

#### 9.3.3.xx	Inter-system SON Information Request

This IE contains the request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Request | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Request | M |  | 9.3.3.zz |  |
| >Inter-system Load Reporting |  |  |  |  |
| >>Inter-system Load Reporting Request | M |  | 9.3.3.y1 |  |

#### 9.3.3.yy	Inter-system SON Information Reply

This IE contains the reply information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Response | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Reply | M |  | 9.3.3.bb |  |
| >Inter-system Load Reporting |  |  |  |  |
| >>Inter-system Load Reporting Reply | M |  | 9.3.3.y3 |  |

#### 9.3.3.Y1	Inter-system Load Reporting Request

This IE contains information on the requested Inter-system Load Reporting reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell List |  | 1 |  |  |
| >Cell Item |  | 1..<maxnoofCellList> |  |  |
| >>Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| CHOICE report type | M |  |  |  |
| >Event based reporting |  |  |  |  |
| >>Number Of Measurement Reporting Levels | M |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale between the starting point and the reporting node's threshold for overload. The reporting node sends a report each time the cell load changes from one reporting level to another, when the cell load enters and exits overload state. |
| >>Starting Point | M |  | INTEGER (0..100) | This indicates the starting point expressed in percentage of the reporting range of the measurement. |
| >Periodic Reporting |  |  |  |  |
| >>Reporting Periodicity | M |  | ENUMERATED(single, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting. If the value is “single” there is only one report. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of cells for inter system load reporting. Value is 16384. |

#### 9.3.3.Y2	Inter-system Load Report

This IE contains the Inter-system load report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Reply | M |  |  |  |
| >Inter-system load reporting |  |  |  |  |
| >>Cell List |  | 1 |  |  |
| >>>Cell Item |  | 1..<maxnoofCellList> |  |  |
| >>>>Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| >>>>Composite Available Capacity Group | M |  | OCTET STRING | Contains the Composite Available Capacity Group IE as defined in TS 36.423 [x]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of cells for inter system load reporting. Value is 16384. |

#### 9.3.3.y3	Inter-system Load Reporting Reply

This IE indicates for which cells the load reporting was successfully initiated.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell List |  | 1 |  |  |
| >Cell Item |  | 1..<maxnoofCellList> |  |  |
| >>Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of cells for inter system load reporting. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>