3GPP TSG-RAN3 #111-e	R3-210406

25th January – 5th February 2021

Online

Agenda Item:	09.3.8.1

Source:	Ericsson, Verizon Wireless

Title:	How to release NR-NR DC configurations between MN-CU and MN-DU

Document for:	Discussion, Decision

# Introduction

In this contribution we will look into the case of SCG removal and especially how an M-gNB-CU can communicate to an M-gNB-DU that the SCG configuration was released. In the following we will further elaborate on this issue, highlight the lack of signalling we identified and bring forward our proposals

# Discussion

The use case we would like to explore starts with a UE in NR-NR DC. At some point the UE moves from NR-NR-DC to single NR connectivity. The question is how the M-gNB-CU communicates to the M-gNB-DU the release of SCG configuration.

Currently in TS 38.473 there is no way that this can be notified. Correspondingly the M-gNB-DU is not aware of the SCG release and of the configuration changes that this entails. Since the UE is moved back to single NR connectivity, the M-gNB-DU should maximize its configuration. Lack of knowledge thereof would lead to significant performance loss.

Observation 1: the M-gNB-DU is not aware of the SCG release and of the configuration changes that this entails.

- A simple solution that would solve this problem would be to signal over F1 a new IE to indicate that an SCG is removed, which would allow the MN-gNB-DU to optimize its L1/L2 configuration and to maximize the performance in single connectivity.

- It should be highlighted that, if a new indication is added to flag to the gNB-DU that an SCG has been removed, it could be beneficial to also indicate that an SCG configuration has been added. The gNB-DU could deduce this information by extrapolation from the CG-Config IE, however, such process of extrapolation could be computationally expensive. A simpler procedure would be do receive an explicit indication, which would make the implementation easier.

In conclusion we believe the proposed solution is the most advantageous way forward.

- In light of the above the following is proposed:

Proposal 1: signal over F1 a new IE to indicate to the M-gNB-DU that an SCG is added or removed.

# Conclusion

### In this contribution the issue of SCG removal and corresponding communication between MN-gNB-CU and MN-gNB-DU has been discussed and the following proposal was made:

Proposal 1: signal over F1 a new IE to indicate to the M-gNB-DU that an SCG is added or removed.

- CRs reflecting the proposal above are available in R3-210407 and in R3-210408.

# References