3GPP TSG-RAN WG3 Meeting #111-e	R3-210446

E-meeting, 25 Jan – 5 Feb 2021

Title: 	(TP for MDT BL CR for TS 38.463): MDT coexistence with IDC

Source: 	Huawei

Agenda item:	10.3.2.1

Document Type:	other

# 1. Introduction

This contribution contains the TP for SON BLCR for TS 38.463 as discussed in [1].

# 2. Reference

[1] R3-210444	(TP for SON BLCR for TS 38.401): MDT coexistence with IDC

# 3. TP for SON BLCR for TS 38.463

First change

8.4.3	Cell Traffic Trace

8.4.3.1	General

The purpose of the Cell Traffic Trace procedure is to send the allocated Trace Recording Session Reference and the Trace Reference to the gNB-CU-CP. The procedure uses UE-associated signalling.

8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Cell Traffic Trace procedure. Successful operation.

The procedure is initiated with a CELL TRAFFIC TRACE message sent from the gNB-CU-UP to the gNB-CU-CP.

If the Privacy Indicator IE is included in the message, the gNB-CU-CP shall store the information so that it can be transferred towards the AMF.

If the IDC Information Request IE is included in the message, the gNB-CU-CP shall send the IDC information reported from the UE to the trace collection entity indicated by the Trace Collection Entity IP Address or Trace Collection Entity URI IE.

8.4.3.3	Abnormal Conditions

Void.

Next change

9.2.3.3	CELL TRAFFIC TRACE

This message is sent by the gNB-CU-UP to initiate a trace session for a UE.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | The Trace ID IE is composed of the following: Trace Reference defined in TS 32.422 [24] (leftmost 6 octets, with PLMN information coded as in 9.2.3.8), and Trace Recording Session Reference defined in TS 32.422 [24] (last 2 octets). | YES | ignore |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.2.2.1 | For File based Reporting. Defined in TS 32.422 [24]. Should be ignored if URI is present. | YES | ignore |
| Privacy Indicator | O |  | ENUMERATED (Immediate MDT, Logged MDT, ...) |  | YES | ignore |
| Trace Collection Entity URI | O |  | 9.3.2.8 | For Streaming based Reporting. Defined in TS 32.422 [24] Replaces Trace Collection Entity IP Address if present. | YES | ignore |
| IDC Information Request | O |  | ENUMERATED (True, ...) |  | YES | ignore |

End change