3GPP TSG-RAN WG3 Meeting #111-e	R3-210457

E-meeting, 25 Jan – 5 Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0549 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discussion on the size limitation of DTLS over SCTP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | DTLS over SCTP limits the maximum size of the messages to be 2^14 bit (about 16 Kbytes). However, in Release 16, the following inter-node RRC messages e.g. HandoverPreparationInformation, CG-ConfigInfo UERadioAccessCapabilityInformation etc. which will be transferred over the network interface may exceed the 16 K size. In case of DTLS over SCTP, the full set of inter-node RRC messages cannot be signalled over network interfaces. Therefore, it is necessary to introduce the segmentation feature to the message that contains the inter-node RRC messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Dedicated Message Segment Transfer Procedure. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it introduces a new procedure: Dedicated Message Segment Transfer Procedure. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interface messages that contains inter-node RRC messages could not be deliveried in case of DTLS over SCTP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.4.a, 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

<<<<<<<<<<<<<<<<<<<<<<<<<<< Unchanged part is omitted >>>>>>>>>>>>>>>>>>>>>>>>>>>

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Dedicated Message Segment Transfer | DEDICATED MESSAGE SEGMENT |

NEXT CHANGE

### 8.4.a	Dedicated Message Segment Transfer

#### 8.4.a.1	General

The purpose of the Dedicated Message Segment Transfer procedure is to transfer segments of a message between NG-RAN nodes. The procedure is initiated by an NG-RAN node whenever a message exceeds a limit.

The procedure uses UE-associated signalling.

#### 8.4.a.2	Successful Operation

The NG-RAN node1 initiates the procedure by sending the DEDICATED MESSAGE SEGMENT message to the peer NG-RAN node2.

Upon receipt of this message, the NG-RAN node2 shall consider the Segment Number IE in the DEDICATED MESSAGE SEGMENT message to assemble the segments in the correct order.

#### 8.4.a.3	Unsuccessful Operation

Not applicable.

NEXT CHANGE

#### 9.1.3.a	DEDICATED MESSAGE SEGMENT

This message is sent by a NG-RAN node1 to the peer NG-RAN node2 to transfer segments of a message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Segment Number | M |  | INTEGER (1.. maxnoofSegments, ...) | Indicates the sequence number of a segment of a message. | YES | reject |
| Message Segment Container | M |  | OCTET STRING |  | YES | reject |
| Message Segment Type | M |  | ENUMERATED (notLastSegment, lastSegment, …) | Indicates whether the included message segment is the last segment of the message or not. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSegments | Maximum no. of segments. Value is 256. |

NEXT CHANGE

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

MobilityChangeFailure,

AccessAndMobilityIndication,

DedicatedMessageSegmentTransfer

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-dedicatedMessageSegmentTransfer

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

dedicatedMessageSegmentTransfer			,

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY 				ignore

}

dedicatedMessageSegmentTransfer XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		DedicatedMessageSegmentTransfer

PROCEDURE CODE			id-dedicatedMessageSegmentTransfer

CRITICALITY 				ignore

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SegmentNumber,

MessageSegmentContainer,

MessageSegmentType

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SegmentNumber,

id-MessageSegmentContainer,

id-MessageSegmentType,

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- ACCESS AND MOBILITY INDICATION

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ AccessAndMobilityIndication-IEs}},

...

}

AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RACHReportInformation			CRITICALITY ignore		TYPE RACHReportInformation			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DEDICATED MESSAGE SEGMENT TRANSFER

--

-- **************************************************************

DedicatedMessageSegmentTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ DedicatedMessageSegmentTransfer-IEs}},

...

}

DedicatedMessageSegmentTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-SegmentNumber						CRITICALITY reject		TYPE SegmentNumber									PRESENCE mandatory }|

{ ID id-MessageSegmentContainer	CRITICALITY reject		TYPE MessageSegmentContainer	PRESENCE mandatory }|

{ ID id-MessageSegmentType					CRITICALITY reject		TYPE MessageSegmentType							PRESENCE mandatory },

...

}

END

-- ASN1STOP

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofSegments

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### -- M

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MessageSegmentContainer ::= OCTET STRING

MessageSegmentType ::= ENUMERATED {notLastSegment, lastSegment, ...}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### -- S

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

SecurityResult ::= SEQUENCE {

integrityProtectionResult				ENUMERATED {performed, not-performed, ...},

confidentialityProtectionResult			ENUMERATED {performed, not-performed, ...},

iE-Extensions							ProtocolExtensionContainer { {SecurityResult-ExtIEs} } OPTIONAL,

...

}

SecurityResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SegmentNumber::= INTEGER (1..maxnoofSegments)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-dedicatedMessageSegmentTransfer								ProcedureCode ::= a1

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofSegments										INTEGER ::= 256

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<     Unchanged part omitted     >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SegmentNumber																					ProtocolIE-ID ::= a2

id-MessageSegmentContainer																			ProtocolIE-ID ::= a3

id-MessageSegmentType																				ProtocolIE-ID ::= a4