3GPP TSG-RAN WG3 #111 electronic	R3-210463

Online, 25 January – 4 February 2021

Agenda Item:	22.2.4

Source:	CATT

Title:	TP for 38.463 on MBS Context Management at E1 interface

Document for:	Discussion, Decision

1	Introduction

In last RAN3 meeting, an agreement of MBS session resource over F1/E1 has been made:

-	F1/E1 MBS Bearer management procedure can be discussed, but details on e.g. information to signal are pending RAN2/SA2 progress

In this contribution, we provide some proposals on MBS context management procedure over E1 interface and the corresponding TPs is annexed below.

2	Discussion

According to the conclusions of the last RAN3 meeting, a new non-UE associated MBS bearer management procedure needs to be introduced into E1 interface. For example, at least an MBS bearer context setup procedure (class 1) needs to be specified in E1AP.

Observation 1: An MBS bearer context setup procedure (class 1) needs to be specified in E1AP.

Similar to the existing bearer management procedure over E1 interface, in addition to defining a MBS bearer context setup procedure, the modification and deletion procedure of MBS bearer should also be considered, specifically, the gNB-CU-CP initiated MBS bearer context modification procedure is needed to allow the gNB-CU-CP to modify a MBS bearer context in the gNB-CU-UP. In addition, there is a need to introduce the gNB-CU-UP-initiated MBS bearer context modification procedure to allow the gNB-CU-UP to modify a MBS bearer context (e.g., due to local problems) and inform the gNB-CU-CP.

Proposal 1: An gNB-CU-CP initiated MBS bearer context modification procedure (class 1) should be specified in E1AP.

Proposal 2: An gNB-CU-UP initiated MBS bearer context modification procedure (class 1) should be specified in E1AP.

Finally, both gNB-CU-CP and gNB-CU-UP initiated MBS bearer context release procedures need to be introduced into the E1 interface. Similar to the existing bearer context release procedures, an gNB-CU-CP initiated MBS bearer context release procedure (class 1) should be specified in E1AP, and an gNB-CU-UP initiated MBS bearer context release request procedure (class 2) should be specified in E1AP.

Proposal 3：An gNB-CU-CP initiated MBS bearer context release procedure (class 1) should be specified in E1AP.

Proposal 4：An gNB-CU-UP initiated MBS bearer context release request procedure (class 2) should be specified in E1AP.

3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: An MBS bearer context setup procedure (class 1) needs to be specified in E1AP.

Proposal 1: An gNB-CU-CP initiated MBS bearer context modification procedure (class 1) should be specified in E1AP.

Proposal 2: An gNB-CU-UP initiated MBS bearer context modification procedure (class 1) should be specified in E1AP.

Proposal 3：An gNB-CU-CP initiated MBS bearer context release procedure (class 1) should be specified in E1AP.

Proposal 4：An gNB-CU-UP initiated MBS bearer context release request procedure (class 2) should be specified in E1AP.

4	Reference

- RP-201038    “New Work Item on NR Multicast and Broadcast Services”

- R3-207056	(TP to TS 38.401 BL CR) Bearer management over F1 and E1, Huawei

5	Text Proposals for stage 3

5.1	TP for TS 38.463

--------------------------------Start of the First Change-----------------------------

8	E1AP procedures

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| MBS Bearer Context Setup |  | MBS BEARER CONTEXT SETUP REQUEST |  | MBS BEARER CONTEXT MODIFICATION RESPONSE |  | MBS BEARER CONTEXT MODIFICATION FAILURE |  |
| MBS Bearer Context Modification (gNB-CU-CP initiated) |  | MBS BEARER CONTEXT MODIFICATION REQUEST |  | MBS BEARER CONTEXT SETUP RESPONSE |  | MBS BEARER CONTEXT SETUP FAILURE |  |
| MBS Bearer Context Modification Required (gNB-CU-UP initiated) |  | MBS BEARER CONTEXT MODIFICATION REQUIRED |  | MBS BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| MBS Bearer Context Release (gNB-CU-CP initiated) |  | MBS BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |
| MBS Bearer Context Release Request (gNB-CU-UP initiated) |  |  | MBS BEARER CONTEXT RELEASE REQUEST |  |  |

------------------------------------------Next Change---------------------------------------

8.3.X	MBS Bearer Context Setup

8.3.X.1	General

The purpose of the MBS Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a MBS bearer context in the gNB-CU-UP.

8.3.X.2	Successful Operation

TBD

8.3.X.3	Unsuccessful Operation

TBD

8.3.X.4	Abnormal Conditions

TBD

8.3.Y	MBS Bearer Context Modification (gNB-CU-CP initiated)

8.3.Y.1	General

The purpose of the MBS Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a MBS bearer context in the gNB-CU-UP.

8.3.Y.2	Successful Operation

TBD

8.3.Y.3	Unsuccessful Operation

TBD

8.3.Y.4	Abnormal Conditions

TBD

8.3.Z	MBS Bearer Context Modification Required (gNB-CU-UP initiated)

8.3.Z.1	General

The purpose of the MBS Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a MBS bearer context (e.g., due to local problems) and inform the gNB-CU-CP.

8.3.Z.2	Successful Operation

TBD

8.3.Z.3	Abnormal Conditions

TBD

8.3.XX	MBS Bearer Context Release (gNB-CU-CP initiated)

8.3.XX.1	General

The purpose of the MBS Bearer Context Release procedure is to allow the gNB-CU-CP to command the release of all MBS bearer context in the gNB-CU-UP.

8.3.XX.2	Successful Operation

TBD

8.3.XX.3	Abnormal Conditions

TBD

8.3.YY	MBS Bearer Context Release Request (gNB-CU-UP initiated)

8.3.YY.1	General

The purpose of the MBS Bearer Context Release Request procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to release all MBS bearer context.

8.3.YY.2	Successful Operation

TBD

8.3.YY.3	Abnormal Conditions

TBD

------------------------------------------Next Change---------------------------------------

9.2.X.1	MBS BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| >MRB To Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| >>QoS Flows Information To Be Setup | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.2	MBS BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested MBS bearer context.

Direction:gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB Setup List |  | 1 |  |  | - | - |
| >MRB Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| TBD |  |  |  |  |  |  |
| MRB Failed List |  | 0.. 1 |  |  | - | - |
| >MRB Failed Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.3	MBS BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the MBS bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD. |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.4	MBS BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB To Setup List |  | 1 |  |  | - | - |
| >MRB To Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| >>QoS Flows Information To Be Setup | M |  | FFS |  | - | - |
| MRB To Modify List |  | 1 |  |  | - | - |
| >MRB To Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB To Remove List |  | 1 |  |  | - | - |
| >MRB To Remove Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.X.5	MBS BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB Setup List |  | 1 |  |  | - | - |
| >MRB Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Failed List |  | 1 |  |  | - | - |
| >MRB Failed Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Modified List |  | 1 |  |  | - | - |
| >MRB Modified Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Failed to Modify List |  | 1 |  |  | - | - |
| >MRB Failed to Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.X.6	MBS BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the MBS bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | Cause |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Criticality Diagnostics |

9.2.X.7	MBS BEARER CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to inform the gNB-CU-CP that a modification of a MBS bearer context is required (e.g., due to local problems at the gNB-CU-UP).

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB To Modify List |  | 1 |  |  | - | - |
| >MRB To Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB To Remove List |  | 1 |  |  | - | - |
| >MRB To Remove Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.8	MBS BEARER CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU-CP to confirm the modification of the requested MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB Modified List |  | 1 |  |  | - | - |
| >MRB Modified Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.9	MBS BEARER CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release all MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.X.10	MBS BEARER CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of all MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.11	MBS BEARER CONTEXT RELEASE REQUEST

This message is sent by the gNB-CU-UP to request the release of all MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB Status List |  | 0.. 1 |  |  | YES | ignore |
| >MRB Status Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD |  |  |  |  |  |  |
| TBD |  |  |  |  |  |  |

- -------------------------------------End of the Changes----------------------------------