3GPP TSG-RAN3 Meeting #111-e	R3-210539

E-meeting, 25 Jan – 5 Feb 2021

Title: 	Evaluation of slicing solutions

Source: 	Huawei

Agenda item:	17.2

Document Type:	Discussion and Decision

1. Introduction

In this contribution, the evaluation of the solutions is given considering the criteria listed in TR 38.832[1].

2. Discussion

2.1 Re-mapping Policy in target NG-RAN node

This solution is about the slice re-mapping policy configuration in the target NG-RAN. It can be differentiated based on the configuration source and granularity.

- Configuration source: preconfigured by OAM or signalled from 5GC.

- Configuration granularity: UE level if configured by OAM, UE/PDU session-level if configured by the 5GC.

Table 2.1-1 illustrates the 5GS system impacts of the proposed solution above.

Table 2.1-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| Configuration in target NG-RAN node | RAN makes the re-mapping decision. | May be notified in case of any remapping e.g., for charging purpose | Signal slice re-mapping policy to the NG-RAN | No impact | Can be used for slice-resource shortage scenarios. FFS can be applied to non-supported slice scenarios |
| Signalled from 5GC | RAN makes the re-mapping decision. | Signal the re-mapping policy to the NG-RAN | No impact | FFS, depending on SA2 feedback | FFS, depending on SA2 feedback |

2.2 Configuration Based Solution

This solution is about the implementation of the slice re-mapping based on the shared/prioritized/dedicated resource modeling in TS 28.541 [2]. Further evaluation is given in Table 2.2.1.

Table 2.2-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| Configuration based Solution | RAN makes the re-mapping decision. | May be notified in case of any remapping e.g., for charging purpose | Signal slice re-mapping policy to the NG-RAN | No impact | The slice service continuity can be guaranteed for the resource shortage scenario. whether it can be applied to non-supported slice scenarios, depending on SA2 feedback |

2.3 Candidate solutions with/without CN involvement

This solution is for the non-supported slice scenario. The anchor gNB remains unchanged after the HO procedure, and the UL/DL traffics are relayed between the S-gNB and the T-gNB via the Xn tunnel. Table 2.3-1 illustrates the 5GS system impacts of the proposed solution.

Table 2.3-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| Solution with CN involvement | RAN makes the re-mapping decision. | Signal the re-mapping policy to the NG-RAN | No impact | FFS, depending on SA2 feedback | Whether it can be applied to scenario 2 is dependent on SA2 feedback |
| Solution without CN involvement | RAN makes the RAN part of slice re-mapping decision. | No impact | Make and transfer the slice re-mapping policy to the NG-RAN | No impact | Can be applied for scenario 2 |

2.4 Slice resource re-partitioning

This solution is based on the soft partition of RAN resource for the slice, where the 5GS may allow a slice to use another slice’s resources on a temporary basis. Table 2.4-1 illustrates the 5GS system impacts of the solution.

Table 2.4-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| Slice resource re-partitioning | RAN makes the re-mapping decision. | May be notified in case of any remapping e.g., for charging purpose | Signal slice re-mapping policy to the NG-RAN | No impact | Can be used for slice-resource shortage scenarios. |

2.5 Multi-carrier radio resource sharing

This solution focuses on radio resource sharing between slices using DC/CA. Table 2.5-1 illustrates the 5GS system impacts of the solution.

Table 2.5-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| Multi-carrier radio resource sharing | RAN makes the re-mapping decision. CA is applicable, whether DC needs enhancement is FFS | whether DC needs enhancement is FFS | No impact | No impact | Can be used for slice-resource shortage scenarios. It is constrained by the slice deployment and UE capability |

2.6 5GC Solution based on SSC-mode 3

This is a CN-centric solution, where a new PDU session of the re-mapped slice is established using SSC-mode 3. Table 2.6-1 illustrates the 5GS system impacts of the solution.

Table 2.6-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| 5GC Solution based on SSC-mode 3 | RAN makes the re-mapping decision. | FFS, depending on SA2 feedback | Signal the re-mapping policy to the NG-RAN | No impact | FFS, depending on SA2 feedback |

2.7 Slice Remapping decision in 5GC

This solution is about the 5GC making the re-mapping decision. For Xn-based HO, if the original slice is not supported by the T-gNB, NG-based HO will be triggered instead. Table 2.7-1 illustrates the 5GS system impacts of the solution.

Table 2.7-1: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| Slice Remapping decision in 5GC | RAN initiates the NG-based HO when needed. | FFS, depending on SA2 feedback | No impact | No impact | FFS, depending on SA2 feedback |

- Proposal 1: Capture the evaluation of slice re-mapping solutions in TR 38.832.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- Proposal 1: Capture the evaluation of slice re-mapping solutions in TR 38.832.

4. Reference

- 3GPP TR 38.832 V0.4.0, Study on enhancement of Radio Access Network (RAN) slicing.

- 3GPP TS 28.541 V17.0.0, 5G Network Resource Model (NRM).

5. TP for TR 38.832

6.3 Solution evaluation

The evaluation criteria are as follows:

- RAN impact

The point here is to analyze RAN impact of the solution (standardization and node behaviour), for example what signalling procedures may be affected and at what extent.

- Core impact

The point here is to analyze Core impact of the solution (standardization and node behaviour), for example what signalling procedures may be affected and at what extent. Such analysis needs to be carried out together with SA2 and CT groups.

- OAM impact

The point here is to analyze operator and maintenance effort, for example how many network elements (e,g. gNB, NF) should be configured and managed by OAM. Such analysis may need to involve SA5.

- UE Impact

This is to analyse the impact at NAS and AS level on the UE. Such analysis needs to be carried out together with RAN2, SA2 and CT groups.

- Effectiveness of solution

The point here is to analyse the effectiveness after applying the solution, for example the UE’s service experience after applying the solution.

Editor note: A better definition is needed.

Table 6.3-1: Key impacts of the solutions

| Criteria Solution |  |  | RAN impact | Core impact | OAM impact | UE impact | Effectiveness |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solution 6.2.1: Re-mapping Policy in target NG-RAN node | Configuration in target NG-RAN node |  | RAN makes the re-mapping decision. | May be notified in case of any remapping e.g., for charging purpose | Signal slice re-mapping policy to the NG-RAN | No impact | Can be used for slice-resource shortage scenarios. FFS can be applied to non-supported slice scenarios |
|  | Signalled from 5GC |  | RAN makes the re-mapping decision. | Signal the re-mapping policy to the NG-RAN | No impact | FFS, depending on SA2 feedback | FFS, depending on SA2 feedback |
| Solution 6.2.2: Configuration based Solution |  |  | RAN makes the re-mapping decision. | May be notified in case of any remapping e.g., for charging purpose | Signal slice re-mapping policy to the NG-RAN | No impact | The slice service continuity can be guaranteed for the resource shortage scenario. whether it can be applied to non-supported slice scenarios, depending on SA2 feedback |
| Solution 6.2.3: Candidate solutions with/without CN involvement |  | Solution with CN involvement | RAN makes the re-mapping decision. | Signal the re-mapping policy to the NG-RAN | No impact | FFS, depending on SA2 feedback | Whether it can be applied to scenario 2 is dependent on SA2 feedback |
|  |  | Solution without CN involvement | RAN makes the RAN part of slice re-mapping decision. | No impact | Make and transfer the slice re-mapping policy to the NG-RAN | No impact | Can be applied for scenario 2 |
| Solution 6.2.4: Slice resource re-partitioning |  |  | RAN makes the re-mapping decision. | May be notified in case of any remapping e.g., for charging purpose | Signal slice re-mapping policy to the NG-RAN | No impact | Can be used for slice-resource shortage scenarios. |
| Solution 6.2.5: Multi-carrier radio resource sharing |  |  | RAN makes the re-mapping decision. CA is applicable, whether DC needs enhancement is FFS | whether DC needs enhancement is FFS | No impact | No impact | Can be used for slice-resource shortage scenarios. It is constrained by the slice deployment and UE capability |
| Solution 6.2.6: 5GC Solution based on SSC-mode 3 |  |  | RAN makes the re-mapping decision. | FFS, depending on SA2 feedback | Signal the re-mapping policy to the NG-RAN | No impact | FFS, depending on SA2 feedback |
| Solution 6.2.7: Slice Remapping decision in 5GC |  |  | RAN initiates the NG-based HO when needed. | FFS, depending on SA2 feedback | No impact | No impact | FFS, depending on SA2 feedback |